3GPP TSG-RAN WG4 Meeting #85	R4-1712428

Reno, Nevada, USA, 27 November – 1 December 2017

Source:	Ericsson

Title:	TP to TR 36.715-03-01: CA_3DL_7A-8A-38A_1UL_BCS0

Agenda item:	8.3.2

Document for:	Approval

# 1. Introduction

This contribution provides a TP for TR 36.715-03-01 to include the requirements for the 3DL/1UL CA combination CA_3DL_7A-8A-38A_1UL_BCS0 as defined in RP-171749 [1].

# 2. Text Proposal for TR 36.715-03-01

##### ---Start of changes---

## 5.x	CA_3DL_7A-8A-38A_1UL_BCS0

### 5.x.1	Operating bands for CA

Table 5.x.1-1: Inter-band CA operating bands

| E-UTRA CA Band | E-UTRA Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_7-8-38 | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | 38 | 2570 MHz | – | 2620 MHz | 2570 MHz | – | 2620 MHz | TDD |

### 5.x.2	Channel bandwidths per operating band for CA

Table 5.x.2-1: Supported E-UTRA bandwidths per CA configuration for inter-band CA

| CA operating / Channel bandwidth |  |  |  |  |  |  |  |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth Combination Set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA Configuration | E-UTRA Bands | 1.4 MHz | 3 MHz |  |  | 5 MHz | 10 MHz | 15 MHz | 20 MHz |  |  |
| CA_7A-8A-38A1 | 7 |  |  |  |  |  | Yes | Yes | Yes | 50 | 0 |
|  | 8 |  |  |  |  | Yes | Yes |  |  |  |  |
|  | 38 |  |  |  |  | Yes | Yes | Yes | Yes |  |  |
| NOTE 1: UL carrier shall be supported in Band 8 only. Power imbalance between downlink carriers on Band 7 and Band 38 is assumed to be within [6dB]. |  |  |  |  |  |  |  |  |  |  |  |

### 5.x.3	Co-existence studies

Table 5.x.3-3 summarizes frequency ranges where harmonics occur due to Band 7, Band 8 and Band 38 CA with 1 UL.

Table 5.x.3-3: Impact of 1 UL Harmonic Interference

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| 7 | 2500 | 2570 | 2620 | 2690 | 5000 | 5140 | 7500 | 7710 |
| 8 | 880 | 915 | 925 | 960 | 1760 | 1830 | 2640 | 2745 |
| 38 | 2570 | 2620 | 2570 | 2620 | 5140 | 5240 | 7710 | 7860 |

It can be seen from Table 5.x.3-3 for CA_7A-8A-38A that there is no harmonic interference from uplink bands towards downlink bands.

### 5.x.4	ΔTIB,c and ΔRIB,c values

ΔTIB,c values are defined in table 5.x.4-1.

Table 5.x.4-1: ΔTIB,c for 3DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_7A-8A-38A | 7 | 0.5 |
|  | 8 | 0.5 |
|  | 38 | 0.5 |

ΔRIB,c values are defined in table 5.x.4-2.

Table 5.x.4-2: ΔRIB,c for 3DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_7A-8A-38A | 7 | 0 |
|  | 8 | 0 |
|  | 38 | 0.2 |

### 5.x.5	 MSD

Updates needed to Table 7.3.1A-0bE in 36.101 is specified in table 5.x.5-1.

# Table 5.x.5-1: Reference sensitivity for carrier aggregation QPSK PREFSENS, CA (exceptions due to cross band isolation issues of TDD and FDD bands)

| EUTRA CA Configuration | EUTRA band | Channel bandwidth |  |  |  |  |  | Duplex mode | Applicable active UL band |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) |  |  |
| CA_7A-8A-38A | 719 |  |  |  | [-91.2] | [-89.7] | [-88.6] | FDD | 8 |
|  | 8 |  |  | -97 | -94 |  |  |  |  |
|  | 38 |  |  | [-93.8] | [-91.2] | [-89.7] | [-88.6] | TDD |  |

Updates needed to Table 7.3.1A-0bF in 36.101 is specified in table 5.x.5-2.

# Table 5.x.5-2: Uplink configuration for reference sensitivity (exceptions due to cross band isolation issues of TDD and FDD bands)

| E-UTRA Band / Channel bandwidth of the affected DL band / NRB / Duplex mode |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | E-UTRA Band | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Duplex Mode |
| CA_7A-8A-38A | 8 |  |  | 25 | 50 |  |  | FDD |

---End of changes---

# Reference

[1]	RP-171749, “Revised WID: LTE Advanced inter-band CA Rel-15 for 3DL/1UL”, RAN#77, Huawei, HiSilicon