3GPP TSG-RAN WG4 Meeting #85	R4-1712445

Reno, US, 27th Nov – 1st Dec 2017

Source: 	Nokia, Nokia Shanghai Bell, Skyworks Solutions, Inc.

Title: 					NR A-MPR for UTRA

Agenda Item: 			9.4.3.3.1

Document for:	Approval

# 1	Introduction

This contribution is presents simulation results for a simulation excercise where it was studied if A-MPR on top of tentative MPR is needed.

# 2	Discussion

In previous meeting tentative MPR value were agreed [1] and are presented in Table 1. Highlighted 256-QAM MPR numbers are not taken from [1] as those were based on results from only one comppany. In this meeting we have our 256-QAM results [2] and in Table 1 the 256-QAM MPR is average of [1] and [2].

Table 1: MPR

| Modulation | MPR (dB) |  |
| --- | --- | --- |
|  | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM PI/2 BPSK | ≤ 0 | ≤ 0 |
| DFT-s-OFDM QPSK | ≤ 1 | ≤ 0 |
| DFT-s-OFDM 16 QAM | ≤ 1.5 | ≤ 0.5 |
| DFT-s-OFDM 64 QAM | ≤ 2 |  |
| DFT-s-OFDM 256 QAM | ≤ 4 |  |
| CP-OFDM QPSK | ≤ 3 | ≤ 0.5 |
| CP-OFDM 16 QAM | ≤ 3 | ≤ 1.5 |
| CP-OFDM 64 QAM | ≤ 3 |  |
| CP-OFDM 256 QAM | ≤ 6 |  |

Simulation assumptions were as follows

- PA calibration point: QPSK 100 RB DFT-s-OFDM signal with 0 dB MPR

- LO leakage and IQ-Image = 28 dBc (256-QAM used same parameters as was used for LTE 256-QAM)

- NR ALCR = 30 dBc

- NR general SEM

- UTRAACLR1  = 33 dBc

- UTRAACLR2  = 36 dBc

Results for channel banwidths of 5 – 40 MHz can be found in Annex. It is assumed that bands where UTRA is deployd it is not possible to use CH bandwidths larger than 40 MHz. Results format is such that required backof is compared against MPR in Table 1 and if MPR is enough then negative value is reported with green color. In case MPR is not sufficient then positive value is reported with red color. An example is presented in Figure 1.

Figure 1: 20 MHz, 15 kHz, QPSK, CP-OFDM and DFT-s-OFDM

In all cases MPR presented in Table 1 is sufficient. With one exception which was 40 MHz BW, 60 kHz scs, pi/2 BPSK, DFT-S,OFDMA were one data point showed 0.2 dB A-MPR. We consider that as an outlier and do not take that into account.

Proposal: If tentative MPR [1] is confirmed no A-MPR is needed for UTRA protection.

Observation: Even if 256-QAM MPR is tightened 1 dB compared to [1] no A-MPR is needed for UTRA protection.

# 3	Conclusion

In this contribution we present results for a simulation excercise where it was studied if A-MPR on top of tentative MPR is needed. Based on our reusults we proposed following.

Proposal: If tentative MPR [1] is confirmed no A-MPR is needed for UTRA protection.

Observation: Even if 256-QAM MPR is tightened 1 dB compared to [1] no A-MPR is needed for UTRA protection.

# 4	References

[1] R4-1711559, WF on MPR for Sub-6 NR, Qualcomm Incorporated, Nokia, [Skyworks Solutions], RAN4#84bis

[2] R4-1712446, NR 256-QAM MPR,  Nokia, Nokia Shanghai Bell, Skyworks Solutions, Inc., RAN4#85

# Appendix: Results