3GPP TSG-RAN WG4 Meeting #85	R4-1712451

Reno, US, 27th Nov – 1st Dec 2017

Source: 	Nokia

Title: 					n71 A-MPR study

Agenda Item: 			9.4.3.3.1

Document for:	Approval

# 1	Introduction

This contribution presents simulation results for band n71 study where it eas checked if current tentative NR MPR [1] is sufficient to meet the FCC emision mask for 600 MHz frequency range.

# 2	Discussion

Simulation assumptions were as follows

- PA calibration point: QPSK 100 RB DFT-s-OFDM signal with 0 dB MPR

- LO leakage and IQ-Image = 28 dBc

- NR ALCR = 30 dBc

- NR SEM as in Table 1

Table 1: SEM

| ΔfOOB (MHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Measurement bandwidth (unless otherwise stated) |
| --- | --- | --- | --- | --- | --- |
|  0-0.1 | -15 | -18 | -20 | -21 | 30 kHz |
|  0.1-6 | -13 | -13 | -13 | -13 | 100 kHz |
|  6-10 | -251 | -13 | -13 | -13 | 100 kHz |
|  10-15 |  | -251 | -13 | -13 | 100 kHz |
|  15-20 |  |  | -251 | -13 | 100 kHz |
|  20-25 |  |  |  | -25 | 1 MHz |
| Note 1: The measurement bandwidth shall be 1 MHz |  |  |  |  |  |

Tentative MPR from [1] is presented in Table 2. 256-QAM was not simulated.

Table 2: Tentative MPR

| Modulation | MPR (dB) |  |
| --- | --- | --- |
|  | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM PI/2 BPSK | ≤ 0 | ≤ 0 |
| DFT-s-OFDM QPSK | ≤ 1 | ≤ 0 |
| DFT-s-OFDM 16 QAM | ≤ 1.5 | ≤ 0.5 |
| DFT-s-OFDM 64 QAM | ≤ 2 |  |
| DFT-s-OFDM 256 QAM |  |  |
| CP-OFDM QPSK | ≤ 3 | ≤ 0.5 |
| CP-OFDM 16 QAM | ≤ 3 | ≤ 1.5 |
| CP-OFDM 64 QAM | ≤ 3 |  |
| CP-OFDM 256 QAM |  |  |

Results are presented in detain in Annex and results format is such that required backof is compared against MPR in Table 2 and if MPR is enough then negative value is reported with green color. In case MPR is not sufficient then positive value is reported with red color. Based on our results no A-MPR is needed for n71 to meet the FCC SEM. Based on our studies this conclusion holds also to 256-QAM altough we did not present results for 256-QAM as the tentative agreement for 256-QAM was only based on results from one comppany.

# 3	Conclusion

In this contribution we have presented band n71 A-MPR simulation results to meet the FCC mask for 600 MHz range. Based on our results we propose.

Proposal: No A-MPR is defined for n71 due to FCC SEM

# 4	References

[1] R4-1711559, WF on MPR for Sub-6 NR, Qualcomm Incorporated, Nokia, [Skyworks Solutions], RAN4#84bis

# Annex

NS_35 SEM, CHBW 5MHz, PAcal 100RB QPSK edge MPR=0dB

NS_35 SEM, CHBW 10MHz, PAcal 100RB QPSK edge MPR=0dB

NS_35 SEM, CHBW 15MHz, PAcal 100RB QPSK edge MPR=0dB

NS_35 SEM, CHBW 20MHz, PAcal 100RB QPSK edge MPR=0dB