3GPP TSG-RAN WG4 Meeting  #85                                                                               R4-1712584

Reno,USA, 27 Oct – 1 Nov, 2017

Source:	CATT

Title:	Draft CR to TS 37.105 for OTA Transmit ON/OFF power in section 9.5

Agenda Item:	8.27.2

Document for:	  Approval

Introduction

In RAN4#83 meeting, the WF on TS 37.105 spec drafting was approved [1]. In RAN4#84, the agreed WF[2] pointed out the Transmit ON/OFF power can be co-location requirement as one option.

In this contribution, draft specification text for TS 37.105 sub-clause 9.5 on OTA Transmit ON/OFF power is created for approval based on agreed WF on Transmit ON/OFF power and transient period in RAN4#84bis [3].

Discussion

The technical text for OTA transmit ON/OFF power is modified from the conducted Transmit ON/OFF power in Rel-13 TS 37.105 section 6.4 based on agreed WF[3,4] , the changes are listed below:

1)  TAB connector is replaced with co-location reference antenna conducted output, which could more clearly express the number of output. For example, for a dual polarized co-location antenna with two connectors, there are two co-location reference antenna conducted outputs, the total off power from all co-location reference antenna conducted outputs should meet the off power requirement.

2)  The rquirement is a co-location requirement and specified at the co-location reference antenna conducted output.

3)  There is no OTA transmitter OFF power requirement and OTA Transmitter transient period requirement for UTRA operation based on the agreement “Radiated requirements applicable for eAAS not support UTRA TDD” in WF[4].

4)  For E-UTRA, the total off power level from all co-location reference antenna conducted outputs shall be less than [-106] dBm/MHz, where fixed scalling factor is considered (9dB)

5) The conductive on/off transient period for MSR and E-UTRA could be reused for OTA requirement.

6) The figure for on/off transient period is modeifed slightly, where it’s label of vertical axis is changed to “The power level at co-location reference antenna conducted output” from “Transmitter Output Power”, because the ON power level and OFF power level should have the same reference point.

Conclusion

In this contribution, the draft specification for TS 37.105 sub-clause 9.5 on OTA Transmit ON/OFF power is created in the annex for approval.

Reference

- R4-1706126, WF on spec drafting, Huawei, RAN4#83, 15-19 May, 2017.

- R4-1708892, WF on Tx ON/OFF, Huawei, RAN4#84, 21-25 Aug, 2017.

- R4-1711799, WF on Transmit ON/OFF power and transient period, CATT, Huawei, RAN4#84bis, 9th – 13st Oct, 2017.

- R4-1711786, Way-forward on eAAS complexity reduction, Ericsson, RAN4#84bis, 9th-13th October 2017.

Annex

<Text Proposal>

<Unchanged texts omitted>

- 9.5	OTA Transmit ON/OFF power

- 9.5.1	General

OTA transmitter ON/OFF power requirements apply only to TDD operation of E-UTRA.

The OTA Transmit ON/OFF power requirements are a co-location requirement and specified at the co-location reference antenna conducted output(s).

- 9.5.2	OTA Transmitter OFF power

- 9.5.2.1	General

OTA transmitter OFF power is defined as the mean power measured over 70 s filtered with a square filter of bandwidth equal to the Base Station RF Bandwidth (s) centred on the central frequency of the Base Station RF Bandwidth (s) during the transmitter OFF period.

For multi-band co-location reference antenna conducted output(s), the requirement is only applicable during the transmitter OFF period in all supported operating bands.

For single band co-location reference antenna conducted output(s) supporting transmission in multiple operating bands, the requirement is applicable per supported operating band.

- 9.5.2.2	Minimum requirement for MSR operation

There is no OTA transmitter OFF power requirement for UTRA operation.

The minimum requirement for E-UTRA operation is defined in subclause 9.5.2.4.

- 9.5.2.3	Minimum requirement for single RAT UTRA operation

There is no OTA transmitter OFF power requirement for UTRA operation.

- 9.5.2.4	Minimum requirement for single RAT E-UTRA operation

The total power from all co-location reference antenna conducted output(s) shall be less than [-106] dBm/MHz.

- 9.5.3	OTA Transmitter transient period

- 9.5.3.1	General

The OTA transmitter transient period is the time period during which the transmitter unit is changing from the OFF period to the ON period or vice versa. The OTA transmitter transient period is illustrated in figure 9.5.3.1-1.

Figure 9.5.3.1-1: Illustration of the relations of transmitter ON period,
transmitter OFF period and transmitter transient period

This requirement applies at co-location reference antenna conducted output(s)  supporting transmission in the operating band.

- 9.5.3.2	Minimum requirement for MSR operation

For MSR AAS BS the OTA transmitter transient period shall be shorter than the values in Table 9.5.3.2-1.

# Table 9.5.3.2-1: Minimum requirements for the transmitter transient period

| Transition | Transient period length [s] |
| --- | --- |
| OFF to ON | 6.25 |
| ON to OFF | 17 |

- 9.5.3.3	Minimum requirement for single RAT UTRA operation

There is no OTA Transmitter transient period requirement for UTRA operation.

- 9.5.3.4	Minimum requirement for single RAT E-UTRA operation

For single RAT AAS BS the OTA transmitter transient period shall be shorter than the values in Table 9.5.3.4-1.

# Table 9.5.3.4-1 Minimum requirements for the transmitter transient period

| Transition | Transient period length [us] |
| --- | --- |
| OFF to ON | 17 |
| ON to OFF | 17 |

<Unchanged texts omitted>

<End of Text Proposal>