3GPP TSG-RAN WG4 Meeting #85	R4-1712697

Reno, USA, 27 Nov - 1 Dec, 2017

Agenda item:	8.2.2

Source:	Dish Network

Title:	TP for TR36.715-02-01: Requirements for B70+B71 CA

Document for:	Approval

# 1	Introduction

∆TIB and ∆RIB and MSD for B70+B71 was approved in RAN4#84bis [1]. This TP captures the requirements for CA_70A-71A.

# 2	Discussion

Agreements from [1] are shown below. The only remaining thing was UL allocation for the low band due to harmonic issue.

Table 1: ΔTIB,c

| Inter-band CA Combination | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| B70+B71 | 70 | 0.3 |
|  | 71 | 0.6 |

Table 2: ΔRIB,c

| Inter-band CA Combination | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| B70+B71 | 70 | 0 |
|  | 71 | 0 |

Table 3: REFSENS exceptions

| Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| CA_70A-71A5,6 | 70 |  |  | -90 | -89.5 | -89.2 |  | FDD |
|  | 71 |  |  | -97.2 | -94.2 | -92 | -87.5 |  |

In [1], the UL allocations to verify harmonic MSD were proposed as follows with an action to consider if these are feasible because allocation for 15MHz BW is wider than those for single carrier B71.

Table 4: Uplink configuration for the low band (exceptions due to harmonic issue)

| E-UTRA Band / Channel bandwidth of the high band / NRB / Duplex mode |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | UL band | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Duplex mode |
| CA_70A-71A | 71 |  |  | 8 | 16 | 25 |  | FDD |
| NOTE 1: refers to the UL resource blocks, which shall be centred within the transmission bandwidth configuration for the channel bandwidth. NOTE 2: the UL configuration applies regardless of the channel bandwidth of the low band unless the UL resource blocks exceed that specified in Table 7.3.1-2 for the uplink bandwidth in which case the allocation according to Table 7.3.1-2 applies. NOTE 3: 3 refers to the UL resource blocks shall be located between 2373-2400MHz. NOTE 4: These configurations apply when there is at least one individual RE within the uplink transmission bandwidth of a low band for which the 3rd transmitter harmonic is within the downlink transmission bandwidth of a high band. NOTE 5: These configurations apply when there is at least one individual RE within the uplink transmission bandwidth of a low band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of a high band. |  |  |  |  |  |  |  |  |

Please note that UL allocations for B71 are as follows.

| 71 |  |  | 25 | 251 | 201 | 201 | FDD |
| --- | --- | --- | --- | --- | --- | --- | --- |

After additional considerations, we suggest using the same approach as used in RAN4 for other harmonic combinations in case the low band allocation is not full in single band configuration. That is to use the same UL allocation for the low band in harmonic configuration that is used for the low band in single band configuration.

The remaining part of the document is TP.

# 3	TP for TR36.715-01-01

## 5.X	CA_70A-71A_BCS0

### 5.X.1	Channel bandwidths per operating band for CA

Table 5.X.1-1: Inter-band CA operating bands

| E-UTRA CA Band | E-UTRA Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_70-71 | 70 | 1695 MHz | – | 1710 MHz | 1995 MHz | – | 2020 MHz | FDD |
|  | 71 | 663 MHz | – | 698 MHz | 617 MHz | – | 652 MHz |  |

Table 5.X.1-2: E-UTRA CA configurations and bandwidth combination sets defined for CA_70A-71A

| E-UTRA CA Configuration | E-UTRA Bands | 1.4 | 3 | 5 | 10 | 15 | 20 | Maximum aggregated bandwidth | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | MHz | MHz | MHz | MHz | MHz | MHz | [MHz] |  |
| CA_70A-71A | 70 |  |  | Yes | Yes | Yes |  | 35 | 0 |
|  | 71 |  |  | Yes | Yes | Yes | Yes |  |  |

### 5.X.2	UE co-existence studies

UE harmonics are shown in table below.

Table 5.X.2-1 UE harmonics

| UE UL frequencies | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| Frequency [MHz] | 1695 | 1710 | 663 | 698 |
| 2nd harmonic [MHz] | 3390 | 3420 | 1326 | 1396 |
| 3rd harmonic [MHz] | 5085 | 5130 | 1989 | 2094 |
| 4th harmonic [MHz] | 6780 | 6840 | 2652 | 2792 |
| 5th harmonic [MHz] | 8475 | 8550 | 3315 | 3490 |

Third harmonic from B71 hits on top of B70 DL. This is captured in MSD section.

### 5.X.3	ΔTIB,c and ΔRIB,c values

For the UE which supports CA_70A-71A, the ΔTIB,c and ΔRIB,c  are defined for applicable bands in Table 5.X.3-2 and 5.X.3-3 respectively.

Table 5.X.3-2: ΔTIB,c

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_70A-71A | 70 | 0.3 |
|  | 71 | 0.6 |

Table 5.X.3-3: ΔRIB,c

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_70A-71A | 70 | 0 |
|  | 71 | 0 |

### 5.X.4	REFSENS requirements

Table 5.X.4-1:  Reference sensitivity for carrier aggregation QPSK PREFSENS, CA (exceptions due to

harmonic issue)

| Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| CA_70A-71A5,6 | 70 |  |  | -90 | -89.5 | -89.2 |  | FDD |
|  | 71 |  |  | -97.2 | -94.2 | -92 | -87.5 |  |
| NOTE 1: The transmitter shall be set to PUMAX as defined in subclause 6.2.5A. NOTE 2: Reference measurement channel is A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1 NOTE 3: The signal power is specified per port NOTE 4: No requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the low band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of the high band. The reference sensitivity is only verified when this is not the case (the requirements specified in clause 7.3.1 apply). NOTE 5: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of a low band for which the 3rd transmitter harmonic is within the downlink transmission bandwidth of a high band. NOTE 6: The requirements should be verified for UL EARFCN of a low band (superscript LB) such that in MHz and with the carrier frequency of a high band in MHz and the channel bandwidth configured in the low band. |  |  |  |  |  |  |  |  |

The UL allocations to verify harmonic MSD are as follows:

Table 5.X.4-2: Uplink configuration for the low band (exceptions due to harmonic issue)

| E-UTRA Band / Channel bandwidth of the high band / NRB / Duplex mode |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | UL band | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Duplex mode |
| CA_70A-71A | 71 |  |  | 8 | 16 | 20 |  | FDD |
| NOTE 1: refers to the UL resource blocks, which shall be centred within the transmission bandwidth configuration for the channel bandwidth. NOTE 2: the UL configuration applies regardless of the channel bandwidth of the low band unless the UL resource blocks exceed that specified in Table 7.3.1-2 for the uplink bandwidth in which case the allocation according to Table 7.3.1-2 applies. NOTE 3: 3 refers to the UL resource blocks shall be located between 2373-2400MHz. NOTE 4: These configurations apply when there is at least one individual RE within the uplink transmission bandwidth of a low band for which the 3rd transmitter harmonic is within the downlink transmission bandwidth of a high band. NOTE 5: These configurations apply when there is at least one individual RE within the uplink transmission bandwidth of a low band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of a high band. |  |  |  |  |  |  |  |  |

# 4	References

[1] R4-1711064, “Requirements for B70+B71 CA”, Dish network, RAN4#84Bis, Dubrovnik, Croatia, October 2017