3GPP TSG-RAN WG4 Meeting #85	R4-1712710

Reno, USA, 27 Nov - 1 Dec, 2017

Agenda item:	9.4.4.1.1

Source:	Dish Network

Title:	Discussion on FR1 REFSENS and ∆RIB

Document for:	Approval

# 1	Introduction

This contribution provides further consideration and proposals on Sub-6GHz REFSENS and ∆RIB.

# 2	Discussion

Sub-6Ghz was discussed in RAN4#NR-AH3. The spread in REFSENS proposals was quite large, several dB’s so hence the outcome was pretty much expected; no agreements.

In this contribution, we concentrate on two points:

-Make a proposal on how to treat ∆RIB.

-Make a proposal on REFSENS for new NR bands

In LTE REFSENS, the baseline NF was 9dB and IM was 2.5dB. These were used for the first LTE band, B1. There is no officially agreed procedure for bands that have different characteristics in LTE. Most used practice has been to first search a band that is roughly similar and then compare RF FE loss differences to define REFSENS for a new band. There is some history behind the first bands that use 12dB NF as baseline (700MHz is US) that relates to, at that time, limited performance of duplexers.

The current REFSENS definition would need a lot of improvements. As a minimum, NF and IM would need to be clearly defined. As an example, some low bands just assume 12dB NF without any specific definition. Baseline for these 700M bands was chosen years ago when the duplexer isolation was modest. The increased NF is almost entirely due to increased impact of TX. Our view is it would be much more straightforward to just assume a combined NF from LNA input “RFIC NF” and FE loss. Then the real impact of TX would come visible in calculations with assumed duplexer performance.

REFSENS is the building block for receiver requirements and that it should not be specified too stringently. On the contrary, we question re-using some LTE REFSENS values for NR for some bands which duplexer isolation performance has improved 15-20dB over the years.

There needs to be some margin in REFSENS; there is no doubt. We suggest the margin would be hidden mostly in the “RFIC NF”. Basically, the achievable NF’s are well known within the industry. Hence, it would be quite easy to agree what NF including some margin is used for each frequency range/RF bandwidth. RF FE loss for B1 includes roughly 2dB filter loss and 2dB other losses consisting of trace, antenna switch, coupler, and so on. For bands which have different filter loss and/or difference in other FE losses the 4dB assumption should be modified accordingly.

Observation 1: Parameters for REFSENS should be defined clearly to achieve consistency amongst different bands.

Then ∆RIB is considered. The rationale for this parameter is well known as in many cases CA requires additional components/more complicated routing increasing FE loss. However, the treatment in RAN4 has not been consistent over all CA combos meaning some combos have say 0.5dB ∆RIB while another similar type of combination has 0dB ∆RIB.

Observation 2: ∆RIB is not consistent over different band combinations in RAN4

We suggest that RAN4 could get rid of this parameter. The impact of this could be accounted in REFSENS re-definition process. The benefits include greatly simplified specification and easier standardization of new combinations.

Based on these considerations and observations, we have the following proposals:

Proposal1: Define NF and IM parameters clearly for REFSENS definition

- Split NF into NF at LNA input “RFIC Part” and NF for RF FE loss “RF FE part”

- Most of the required margin can be hidden in the “RFIC part” of NF

Proposal2: ∆RIB shall not be used in NR specifications

# 3	Conclusions

Sub-6GHz REFSENS and ∆RIB was considered with the following observations and proposals.

Observation 1: Parameters for REFSENS should be defined clearly to achieve consistency amongst different bands.

Observation 2: ∆RIB is not consistent over different band combinations in RAN4

Proposal1: Define NF and IM parameters clearly for REFSENS definition

- Split NF into NF at LNA input “RFIC Part” and NF for RF FE loss “RF FE part”

- Most of the required margin can be hidden in the “RFIC part” of NF

Proposal2: ∆RIB shall not be used in NR specifications

# 4	References