3GPP TSG-RAN WG4 RAN4-85	R4-1712726

Reno, United States, 27 November - 1 December, 2017

Source:		Ericsson

Title:			TP to Section 4.4 in TS 38.113 (NR) Receiver exclusion bands (radiated immunity)

Agenda item:		9.6.2

Document for:	Approval

# Introduction

A text proposal to specify the transmitter and receiver exclusion bands for the EMC test conditions (Section 4.4)  for TS 38.113 version  0.2.0 [1] is presented in this document.

# Discussion

During the meeting in Dubrovnik, an initial text of TS 38.113 was approved. Considering this initial agreement and the skeleton approved for TS 38.113, we propose in this contribution a TP for TS 38.113 on Exclusion bands (Section 4.4).

Proposal 1: Agree on the TP to Section 4.4 in TS 38.113 (NR) transmitter and receiver exclusion bands (radiated immunity) attached to this contribution.

# Conclusion

Based on the above discussion, it is proposed to agree on TP to Section 4.4 in TS 38.113 (NR) attached to this contribution.

# References

Draft TS 38.113 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; NR; Base Station (BS)  ElectroMagnetic Compatibility (EMC), v. 0.2.0

# TP to the draft TS 38.113

In this section, TP to Section 4.4 in TS 38.113 [1] is proposed for approval.

- ------------------------------ Proposed Text ------------------------------

## 4.4	Exclusion bands

4.4.1 Transmitter exclusion band

For testing of radiated immunity there shall be no transmitter exclusion band.

4.4.2 Receiver exclusion band

The receiver exclusion band for Base Stations is the band of frequencies over which no tests of radiated immunity of a receiver are made.

For 1-C type NR BS in an operating band less than 200 MHz wide or 1-H type NR BS in an operating band less than 100 MHz wide, the exclusion band extends from the lower frequency of the Base Station receive band minus 20 MHz to the upper frequency of the Base Station receive band plus 20 MHz.

For 1-C type NR BS in an operating band less than 200 MHz wide or 1-H type NR BS in an operating band less than 100 MHz wide, the exclusion band extends from the lower frequency of the Base Station receive band minus 60 MHz to the upper frequency of the Base Station receive band plus 60 MHz.

The specific frequency range(s) for NR excluded from assessment shall be detailed in the technical documentation, according to Table  [X]

<table will be added>

<For NR type 1-O and 2-O, receiver exclusion band requirement TBD>

------------------------------ End of proposed text ------------------------------