3GPP TSG-RAN WG4 Meeting #85	R4-1712912

Reno, USA, 27th November – 1st December 2017

Title:                        NR PC2 and PC3 MPR evaluation for sub 6GHz

Source: 	CMCC

Agenda Item:	9.4.3.3.1

Document for:	Approval

1 Introduction

The WF on MPR for Sub-6 NR was approved in RAN4#84bis meeting [1]. Simulations for MPR have been provided in the documents in the references based on assumptions from WF in [2].

In this contribution, we further discuss MPR evaluation and update the simulation results for sub 6GHz in [3].

2 Discussion

The PA calibration point and simulation assumptions set for sub 6GHz MPR as shown below:

- PA calibration point: PA calibration point is based on No MPR allowed for 100 RB QPSK DFT-s-OFDM (15 KHz SCS) signal.

- ACLR: UTRA_ACLR1 = 33 dBc, UTRA_ACLR2=36dBc, E-UTRA_ACLR=30dBc

- Tx Modulator impairments : IQ suppression 28 dBc, Carrier leakage 28 dBc, Single carrier C_IM3 = 60dB

- PA model uses the polynomial model in [4]

- SEM, EVM and other assumptions refer to WF [2]

The simulation results are conduct by using Keysight SystemVue, and the simulation link as shown in Figure 1;

Figure 1 NR Uplink transmitter link for MPR simulation

Figure 2 shows the PA calibration for 100RB QPSK DFT-s-OFDM (15KHz SCS) signal, the MPR value for QPSK is around 0dB for 100RB (DFT-s-OFDM).

Figure 2 the MPR value for QPSK is around 0dB for 100RB (DFT-s-OFDM)

We summarize the following tables show the sub 6GHz MPR simulation results for PC 3.

|  |  | Outer | Inner |
| --- | --- | --- | --- |
| WF type | Modulation | MPR for all BW and SCS |  |
| DFT-S-OFDM | Pi/2-BPSK | N/A | N/A |
|  | QPSK | 0 | 0 |
|  | 16QAM | 0.45 | 0 |
|  | 64QAM | 0.56 |  |
|  | 256QAM | TBD |  |

Table 1 PC3 MPR simulation results for DFT-S-OFDM

|  |  | Outer | Inner |
| --- | --- | --- | --- |
| WF type | Modulation | MPR for all BW and SCS |  |
| CP-OFDM | QPSK | 1.65 | 0 |
|  | 16QAM | 1.70 | 0 |
|  | 64QAM | 1.70 |  |
|  | 256QAM | TBD |  |

Table 2 PC3 MPR simulation results for CP-OFDM

The following tables show the sub 6GHz MPR simulation results for PC 2.

|  |  | Outer | Inner |
| --- | --- | --- | --- |
| WF type | Modulation | MPR for all BW and SCS |  |
| DFT-S-OFDM | Pi/2-BPSK | N/A | N/A |
|  | QPSK | 0 | 0 |
|  | 16QAM | 0.45 | 0 |
|  | 64QAM | 0.56 |  |
|  | 256QAM | TBD |  |

Table 3 PC2 MPR simulation results for DFT-S-OFDM

|  |  | Outer | Inner |
| --- | --- | --- | --- |
| WF type | Modulation | MPR for all BW and SCS |  |
| CP-OFDM | QPSK | 1.65 | 0 |
|  | 16QAM | 1.70 | 0 |
|  | 64QAM | 1.70 |  |
|  | 256QAM | TBD |  |

Table 4 PC2 MPR simulation results for CP-OFDM

Based on the above simulation results, we found that the simulation results of Power Class 2 and Power Class 3 is the same.  In addition, the MRP table of LTE is applicable to PC1, PC2 and PC3 in TS 36.101. So we suggest that NR MPR for Power Class 2 and Power Class 3 should share the same MPR values table.

Proposal 1: NR MPR for Power Class 2 and Power Class 3 should share the same MPR values table.

3 Conclusion

In this contribution, we provide our proposal on MPR evaluation and update the simulation results for sub 6GHz

Proposal 1: NR MPR for Power Class 2 and Power Class 3 should share the same MPR values table.

- Reference

[1] R4-1711559, “WF on MPR for Sub-6 NR”, Qualcomm Incorporated, Nokia, Skyworks Solutions

[2] R4-1709944, “WF on MPR table and allocation definition for sub-6GHz”, Skyworks Solutions

[3] R4-1710147, “MPR evaluation for sub 6GHz,” CMCC

[4] R4-164542, “Response LS on realistic power amplifier model for NR waveform evaluation”, Nokia