3GPP TSG-RAN WG4 #85	R4-1713492

# Reno, USA, Nov. 27 – Dec. 1 2017

Agenda Item:	8.19.4

Source:	Ericsson

Title:	Further discussion on transmit diversity for PC5

Document for:	Discussion

# Introduction

In RAN1#90 meeting, RAN1 had send one LS about transmit diversity for PC5 in [1]. In the last time, the group had been discussed the MPR issue, RSRP impact and MMSE-IRC performance impact. The group get some consensus on the MPR issue. However, for RSRP impact and MMSE-IRC performance impact, the group had no conclusion yet. In this paper, we provide further analysis for these issues.

# Impact on PSSCH-RSRP measurement accuracy of Rel-14 UEs

In this section, simulation results are provided for PSSCH-RSRP measurement accuracy of Rel-14 UE. In the simulation, the simulation parameters are shown in Table 1.

Table 1: Simulation parameters

| Parameters | Value |
| --- | --- |
| Side condition | 0 dB |
| DMRS assumption | Same as uplink MIMO |
| Measured PRBs | 3 PRBs |
| Channel model | EPA5 |

In Figure 2, the PSSCH-RSRP cumulative distribution is given. In the figure, the definition of PSSCH-RSRP accuracy is defined as:

ΔRSRP= abs( ideal RSRP– estimated RSRP)

where both ideal RSRP and estimated RSRP are for one antenna port. Based on the figure, we have the following observations:

Almost no RSRP bias will be observed comparing to the RSRP from one antenna port

The variance of RSRP has some degradation and the degradation is less than 0.6 dB in most range

Figure 2: Cumulative distribution of difference between estimated RSRP and ideal RSRP

# Conclusion

In this paper, we provide further analysis for TxD impact on the PSSCH-RSRP measurement accuracy. Based on the current simulation results, we have the following observations:

Observation 1	Almost no RSRP bias will be observed comparing to the RSRP from one antenna port

Observation 2	The variance of RSRP has some degradation and the degradation is less than 0.6 dB in most range

# References

R1-1715308, LS on transmit diversity for PC5, Aug.21~Aug.25, 2017