3GPP TSG-RAN WG4 #85	R4-1714109

Reno, USA, November 27th ‒ December 1st, 2017

Agenda item:	9.3.1

Source: 	Qualcomm Incorporated

Title: 	TP for TS 38.101-2: Channel Bandwidth

Document for:	Approval

# Introduction

In this paper we present the text proposal for the channel bandwidth definition for FR2.

# Reference

<<< Start of TP >>>

## 5.3	UE Channel Bandwidth

## 5.3.1	General

## The BS channel bandwidth supports a single NR RF carrier in the uplink or downlink at the Base Station. Different UE channel bandwidths may be supported within the same spectrum for transmitting to and receiving from UEs connected to the BS. The placement of the UE channel bandwidth is flexible, but can only be completely within the BS channel bandwidth.

### 5.3.2	Maximum transmission bandwidth configuration

The maximum transmission bandwidth configuration NRB for each channel bandwidth and subcarrier spacing is specified in Table 5.3.3.-1

Table 5.3.2-1: Maximum transmission bandwidth configuration NRB

| SCS [kHz] | 50MHz | 100MHz | 200MHz | 400 MHz |
| --- | --- | --- | --- | --- |
|  | NRB | NRB | NRB | NRB |
| 60 | 66 | 132 | 264 | N.A |
| 120 | 32 | 66 | 132 | 264 |

### 5.3.3	Minimum guardband and transmission bandwidth configuration

The minimum guardband for each UE channel bandwidth and SCS is specified in Table 5.3.3-1 The relationship between the channel bandwidth, the guardband and the transmission bandwidth configuration is shown in Figure 5.3.3-1.

Table 5.3.3-1: Minimum Guradband for each UE channel bandwidth and SCS (kHz)

| SCS [kHz] | 50MHz | 100MHz | 200MHz | 400 MHz |
| --- | --- | --- | --- | --- |
| 60 | 1210 | 2450 | 4930 | N.A |
| 120 | 1900 | 2420 | 4900 | 9860 |

Figure 5.3.3-1 Definition of channel bandwidth and transmission bandwidth configuration for one NR channel

The number of RBs configured in any channel bandwidth shall ensure that the minimum guardband specified in this clause is met. [In the case that multiple numerologies are multiplexed in the same symbol, the minimum guardband is dictated by the subcarrier spacing of the respective edge RB.]

### Editor’s note: FFS on how to capture the SU/Minimum guardband for the additional BS channel BW.

### 5.3.4	RB alignment with Different Numerologies

Editor’s note: Will be described with reference to the RAN1 definition.

### 5.3.5	Channel bandwidth per Operating Band

The requirements in this specification apply to the combination of channel bandwidths, SCS and operating bands shown in Table 5.3.5-1. The transmission bandwidth configuration in Table 5.3.3.1-1 shall be supported for each of the specified channel bandwidths. The channel bandwidths are specified for both the Tx and Rx path.

Table 5.3.5.-1 Channel Bandwidths for Each NR band

| NR band / SCS / Channel bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| NR Band | SCS kHz | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| nX | 60 |  |  |  |  |
|  | 120 |  |  |  |  |

<<< End of TP >>>