3GPP TSG-RAN WG4 #85	R4-1714114

November 27th ‒ December 1st, 2017

Reno, NV, USA

Agenda item:	9.3.2

Source: 	Qualcomm Incorporated

Title: 	TP for TS 38.101-1: Channel arrangement

Document for:	Approval

# Introduction

In this paper we present the text proposal for the channel arrangement for FR1.

# Reference

<<< Start of TP >>>

## 5.4	Channel Arrangement

### 5.4.1	Channel Spacing

#### 5.4.1.1 	Channel spacing for adjacent NR carriers

The spacing between carriers will depend on the deployment scenario, the size of the frequency block available and the channel bandwidths. The nominal channel spacing between two adjacent NR carriers is defined as following:

Nominal Channel spacing = (BWChannel(1) + BWChannel(2))/2 ±[5kHz]

where BWChannel(1) and BWChannel(2) are the channel bandwidths of the two respective NR carriers. The channel spacing can be adjusted depending on the channel raster to optimize performance in a particular deployment scenario.

#### 5.4.1.2 	Channel Spacing for CA

Editor’s note: Detailed structure of the subclause is TBD.

### 5.4.2	Channel Raster

5.4.2.1	Channel Raster and Numbering

The channel raster entry defines a set of RF reference frequencies that are used to identify the RF channel position. The RF reference frequency for an RF channel maps to a resource element on the carrier.

A global frequency raster is defined for all frequencies from 0 to 100 GHz and is used to define the set of allowed RF reference frequencies. The granularity of the global frequency raster is ΔFGlobal. For each operating band, a subset of frequencies from the global frequency raster are applicable for that band and forms a channel raster for that band with a granularity ΔFRaster, which may be equal to are larger than ΔFGlobal.

FREF = FREF-Offs + ΔFraster (NREF – NREF-Offs)

Table 5.4.2-1: NR-ARFCN parameters for the global frequency raster

| Frequency range | ΔFGlobal | FREF-Offs [MHz] | NREF-Offs | Range of NREF |
| --- | --- | --- | --- | --- |
| 0 – 3000 MHz | 5 kHz | 0 MHz | 0 | 0 – 599999 |
| 3000 – 24250 MHz | 15 kHz | 3000 MHz | 600000 | 600000 – 2016666 |
| 24250 – 100000 MHz | 60 kHz | 24250 MHz | 2016667 | 2016667 – 3279167 |

For SUL bands defined in Table 5.2.1-1.

FREF_SUL = FREF+ΔSUL, ΔSUL=0kHz or 7.5kHz.

ΔSUL is signalled by the network.

The mapping between the channel raster and corresponding resource element is given in Section 5.4.2.2. The applicable entries for each operating band are defined in Section 5.4.2.3

#### 5.4.2.2	Channel Raster to Resource Element Mapping

The mapping between the channel raster and the corresponding resource element is given in Table 5.4.2.2-1. The mapping depends on the total number of RBs that are allocated in the channel and applies to both UL and DL.

Table 5.4.2.2-1: Channel Raster to Resource Element Mapping

| Resource element index | 0 | 6 |
| --- | --- | --- |
| Physical resource block number |  |  |

,  ,  are as defined in TS 38.211[?].

#### 5.4.2.2	Channel raster entries for each operating band

The NR-ARFCN applicable in each NR operating band are given in Table 5.4.2.2-1.

-	For NR operating bands with 100 kHz channel raster, ΔFRaster = 20 × ΔFGloba. In this case every 20th NR-ARFCN within the operating band are applicable for the channel raster within the operating band and the step size for the channel raster in Table 5.4.2.2-1 is given as <20>.

-	For NR operating bands with 5 kHz below 3GHz, ΔFRaster = 3 × ΔFGlobal. In this case all NR-ARFCN within the operating band are applicable for the channel raster within the operating band and the step size for the channel raster in Table 5.4.2.2-1 is given as <3>.

-	For NR operating bands with 15 kHz and 60 kHz channel raster above 3GHz, ΔFRaster = ΔFGlobal. In this case all NR-ARFCN within the operating band are applicable for the channel raster within the operating band and the step size for the channel raster in Table 5.4.2,.-2 is given as <1>.

Table 5.4.2.2-1: Applicable NR-ARFCN per operating band

| NR Operating Band | ΔFRaster [kHz] | Downlink Range of NREF (First – <Step size> – Last) | Uplink Range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| 1 | 100 kHz | 422000 – <20> – 434000 | 384000 – <20> – 396000 |
| … |  |  |  |
| 77 | 15 kHz | 819800 – <1> – 879800 | 819800 – <1> – 879800 |
| … |  |  |  |

### 5.4.3	Synchronization Raster

5.4.3.1	Synchronization Raster and Numbering

The synchronization raster indicates the frequency positions of the synchronization block that can be used by the UE for system acquisition when explicit signalling of the synchronization block position is not present.

A global synchronization raster is defined for all frequencies. The frequency position of the SS block is defined as SSref with corresponding number GSCN. The GSCN and are numbered in increasing frequency order. The parameters defining the SSref and GSCN for all the frequency ranges are in Table 5.4.3.1-1.

The mapping between the SSref and the corresponding SS block resource element is given in subclause 5.4.3.2. The synchronization raster and the subcarrier spacing of the synchronization block is defined separately for each band in subclause

Table 5.4.3.1-1: GSCN parameters for the global frequency raster

| Frequency range | SS_Block frequency position | Range of SSREF |
| --- | --- | --- |
| 0 – 2650 MHz | N*900kHz+ M*5kHz, N=1:[2944], M=-1:1 | 1 – [8832] |
| 2400-24250 MHz | 2400MHz+N*1.44MHz,N= 0:[15173] | [8833-24056] |
| 24250 – 100000 MHz | [24250.08] MHz+N*[17.28]MHz, N= 0:[4383] | [24057 – 28441] |

#### 5.4.3.2	Synchronization Raster to Synchronization Block Resource Element Mapping

The mapping between the synchronization raster and the corresponding resource element of the SS block is given in Table 5.4.2-1. The mapping depends on the total number of RBs that are allocated in the channel and applies to both UL and DL.

Table 5.4.3.2-1: Synchroniztion Raster to SS block Resource Element Mapping

| Resource element index | 0 |
| --- | --- |
| Physical resource block number of the SS block |  |

,  ,are as defined in TS 38.211[?].

#### 5.4.3.3.	Synchronization Raster entries for each operating band

The synchronization raster for each band is give in Table 5.4.3.3-1. The distance between applicable GSCN entries is given by the <Step size> indicated in Table 5.4.3.3-1.

Table 5.4.3.3-1: Applicable SS raster entries per operating band

| NR Operating Band | Range of SSREF (First – <Step size> – Last) |
| --- | --- |
| 1 |  |
| … |  |
| 77 |  |
| … |  |

<<< End of TP >>>