3GPP TSG-RAN WG4 Meeting #85	R4-1714153

Reno, Nevada, USA, 27 November – 1 December 2017

Agenda Item:	9.5.3.6.1

Source: 	Nokia, Nokia Shanghai Bell

Title: 	TP to TR 38.817-02: NR BS conducted receiver intermodulation requirements in FR1 (7.7)

Document for:	Approval

# 1.	Introduction

Below 6GHz NR BS receiver intermodulation requirements have been discussed in RAN4, and it is for further study if it is possible to reuse the E-UTRA BS receiver intermodulation requirements for the conducted requirements at antenna connector of NR BS below 6GHz [1].

This contribution provides proposals to specify the below 6GHz NR BS receiver intermodulation conducted requirements in the RAN4 specifications, and a text proposal to the NR BS TR 38.817-02 [4].

# 2.	Discussion

To investigate the received blocking signal power for below 6GHz NR BS from an adjacent NR system, UL simulation runs have also been performed for 100MHz NR victim and 100MHz NR aggressor in urban macro scenarios (which is the most demanding case for the BS in-band blocking requirements as the UE transmit power will be the highest to compensate for the highest path loss) with 750m ISD (i.e. 500m cell range) at 3.5 GHz carrier frequency, and the simulation results for the received blocking signal power at the victim BS antenna connector (i.e. with antenna element and array gain) are provided in Figures 1 and 2 below. The simulation assumptions and parameters used here are the same as those RAN4 used for NR coexistence simulation for WP5D reply, as recorded in TR 38.803 [1], except the following:

-	Besides coordinated site deployment (0% grid shift), un-coordinated site deployment (100% grid shift) between victim and interfering systems (which RAN4 has long been using in below 6GHz coexistence simulation) is also used.

-	NR UE ACLR = 30dB and BS ACS = 46dB, ACLR and ACS models as explained in TR 36.942 [4].

-	4x8 antenna array (with 3dB polarization gain) is used in the NR BS considering the lower frequency.

-	Omni-directional antenna is used in NR UE.

-	LOS path loss formula is slightly modified per the updated ones in TR 38.901 [5].

-	NR interfering UE with uplink power control parameters of gamma = 1 and CLxile = 88+10*log10(200/100) = 91.

Figure 1: CDF of received blocking signal power with antenna element and array gain of victim BS (UMa 0% grid shift)

Figure 2: CDF of received blocking signal power with antenna element and array gain of victim BS (UMa 100% grid shift)

For comparison, UL simulation runs have also been performed for 100MHz NR victim and 10MHz E-UTRA aggressor in urban macro scenarios with 750m ISD at 3.5 GHz carrier frequency, and the simulation results are shown in Figures 3 and 4 below. The simulation assumptions and parameters are further updated as follows:

-	ACLR and ACS models for asymmetric victim and aggressor bandwidths, as explained in TR 36.942 [4].

-	4x8 antenna array (with 3dB polarization gain) is only used in the NR BS, while passive directional antenna is used in the E-UTRA BS.

-	Omni-directional antenna is used in both NR and E-UTRA UE.

-	E-UTRA interfering UE with uplink power control parameters of gamma=1 and CLxile=112.

Figure 3: CDF of received blocking signal power with antenna element and array gain of victim BS (UMa 0% grid shift)

Figure 4: CDF of received blocking signal power with antenna element and array gain of victim BS (UMa 100% grid shift)

Comparing the simulation results in Figures 1 and 2 to those in Figures 3 and 4, it can be seen the received blocking signal power level at the NR BS antenna connector would be higher with the NR interfering system under the same deployment scenario, because of the lower CLxile and thus the higher transmit power of the NR UE comparing the E-UTRA UE at the same location. Hence coexistence with NR interfering system in the same geographical area would be the more demanding factor for the NR BS receiver in-band blocking conducted requirements comparing to the coexistence with E-UTRA system in the same geographical area.

However, it can be seen from Figures 1 to 4 that adopting the interfering signal power level for wide area E-UTRA BS class (-52dBm) to that for the wide area NR BS class for the receiver intermodulation conducted requirements would ensure sufficient protection (> 98 %-tile) of the NR BS receiver against E-UTRA or NR in-band uplink signal in all simulated scenarios. Therefore, it is proposed to specify the below 6GHz NR BS receiver intermodulation conducted requirements for each NR BS class with an interfering signal power equal to the corresponding E-UTRA BS class.

For the wanted signal power level, a 6dB desensitization (compared to the BS reference sensitivity) is allowed for both UTRA and E-UTRA BS receiver intermodulation requirements. It is proposed to adopt the 6dB desensitization (compared to the BS reference sensitivity) also for below 6GHz NR BS receiver intermodulation conducted requirements. The SNR for the BS reference sensitivity can be obtained at 95% relative throughput from link level simulations.

Moreover, the interfering signal should be defined as the same type as the interfering signal for the ACS requirement, with offset between the interfering signal centre frequency and the nominal band edge of the wanted carrier specified to ensure the intermodulation products fall (almost) on the edge resource blocks of an operating channel bandwidth and avoid orthogonality between sub-carriers of wanted and interfering signal.

# 3.	Conclusion and proposals

This contribution has provided simulation results and proposals to specify the below 6GHz NR BS receiver intermodulation conducted requirements in the RAN4 specifications.

Proposals:

1)	To specify the below 6GHz NR BS receiver intermodulation conducted requirements for each NR BS class with an interfering signal power equal to that for the corresponding E-UTRA BS class, and the wanted signal level calculated as the BS reference sensitivity plus 6dB.

2)	The SNR for the BS reference sensitivity can be obtained at 95% relative throughput from link level simulations.

3)	The interfering signal should be defined as the same type as the interfering signal for the ACS requirement, with offset between the interfering signal centre frequency and the nominal band edge of the wanted carrier specified to ensure the intermodulation products fall (almost) on the edge resource blocks of an operating channel bandwidth and avoid orthogonality between sub-carriers of wanted and interfering signal.

# 4.	Text Proposal

## 7.7	Receiver intermodulation

The following have been agreed for conducted receiver intermodulation requirements in FR1:

-	To specify the below 6GHz NR BS receiver intermodulation conducted requirements for each NR BS class with an interfering signal power equal to that for the corresponding E-UTRA BS class, and the wanted signal level calculated as the BS reference sensitivity plus 6dB.

-	The SNR for the BS reference sensitivity can be obtained at 95% relative throughput from link level simulations.

-	The bandwidth of the modulated interfering signal should be defined as the same as that of the interfering signal for the ACS requirement, with SCS is the same as that of the wanted signal to ensure that any peaks of the IM product align with the SCS of the wanted signal.

-	The offsets between the CW and modulated interfering signal centre frequency and the nominal band edge of the wanted carrier specified to ensure the intermodulation products fall (almost) on the edge resource blocks of an operating channel bandwidth and avoid orthogonality between sub-carriers of wanted and interfering signal.

-	Requirements are set for all possible SCS of the wanted signal, but only the lowest SCS supported by the BS needs to be tested.

Based on above agreements, the receiver intermodulation requirement is specified in Table 6.6.3-2.

# Table 7.7-1: Intermodulation performance requirement in FR1

| BS type | Wanted signal mean power [dBm] | Interfering signal mean power [dBm] | Type of interfering signal |
| --- | --- | --- | --- |
| Wide Area BS | PREFSENS + 6dB* | -52 | See Table 7.7-2 |
| Medium Range BS | PREFSENS + 6dB** | -47 |  |
| Local Area BS | PREFSENS + 6dB*** | -44 |  |
| Note*: PREFSENS depends on the channel bandwidth as specified in Table 7.2-1. Note**: PREFSENS depends on the channel bandwidth as specified in Table 7.2-2. Note*** PREFSENS depends on the channel bandwidth as specified in Table 7.2-3. |  |  |  |

Table 7.7-2: Interfering signal for intermodulation performance requirement in FR1

| E-UTRA channel bandwidth of the lowest/highest carrier received [MHz] | Interfering signal centre frequency offset from the lower/upper Base Station RF Bandwidth edge [MHz] | Type of interfering signal |
| --- | --- | --- |
| 5 | ±7.5 | CW |
|  | ±17.5 | 5MHz NR signal |
| 10 | ±7.45 | CW |
|  | ±17.5 | 5MHz NR signal |
| 15 | ±7.43 | CW |
|  | ±17.5 | 5MHz NR signal |
| 20 | ±7.38 | CW |
|  | ±17.5 | 5MHz NR signal |
| 25 | ±7.45 | CW |
|  | ±25 | 20MHz NR signal |
| 40 | ±7 | CW |
|  | ±25 | 20MHz NR signal |
| 50 | ±7.35 | CW |
|  | ±25 | 20MHz NR signal |
| 60 | ±7.49 | CW |
|  | ±25 | 20MHz NR signal |
| 80 | ±7.44 | CW |
|  | ±25 | 20MHz NR signal |
| 100 | ±7.45 | CW |
|  | ±25 | 20MHz NR signal |

# Table 7.7-3: Narrowband intermodulation performance requirement in FR1

| BS type | Wanted signal mean power [dBm] | Interfering signal mean power [dBm] | Type of interfering signal |
| --- | --- | --- | --- |
| Wide Area BS | PREFSENS + 6dB* | -52 | See Table 7.7-4 |
| Medium Range BS | PREFSENS + 6dB** | -47 |  |
| Local Area BS | PREFSENS + 6dB*** | -44 |  |
| Note*: PREFSENS depends on the channel bandwidth as specified in Table 7.2-1. Note**: PREFSENS depends on the channel bandwidth as specified in Table 7.2-2. Note*** PREFSENS depends on the channel bandwidth as specified in Table 7.2-3. |  |  |  |

# Table 7.7-4: Interfering signals for narrowband intermodulation requirement in FR1

| E-UTRA channel bandwidth of the lowest/highest carrier received [MHz] | Interfering RB centre frequency offset from the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap [kHz] | Type of interfering signal |
| --- | --- | --- |
| 5 | ±360 | CW |
|  | ±1420 | 5MHz NR signal, 1 RB* |
| 10 | ±325 | CW |
|  | ±1780 | 5MHz NR signal, 1 RB* |
| 15** | ±380 | CW |
|  | ±1600 | 5MHz NR signal, 1 RB* |
| 20** | ±345 | CW |
|  | ±1780 | 5MHz NR signal, 1 RB* |
| 25** | ±325 | CW |
|  | ±1990 | 20MHz NR signal, 1 RB* |
| 40** | ±310 | CW |
|  | ±2710 | 20MHz NR signal, 1 RB* |
| 50** | ±330 | CW |
|  | ±3250 | 20MHz NR signal, 1 RB* |
| 60** | ±350 | CW |
|  | ±3790 | 20MHz NR signal, 1 RB* |
| 80** | ±390 | CW |
|  | ±4870 | 20MHz NR signal, 1 RB* |
| 100** | ±340 | CW |
|  | ±5770 | 20MHz NR signal, 1 RB* |
| Note*: Interfering signal consisting of one resource block positioned at the stated offset, the channel bandwidth of the interfering signal is located adjacently to the lower/upper Base Station RF Bandwidth edge. Note**: This requirement shall apply only for a G-FRC mapped to the frequency range at the channel edge adjacent to the interfering signals. |  |  |

# References

[1]	3GPP TR 38.803 v14.0.0, “Study on New Radio Access Technology; RF and co-existence aspects”.

[2]	3GPP TR 36.942 v13.0.0, “Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Frequency (RF) system scenarios”.

[3]	3GPP TR 38.901 v14.0.0, “Study on channel model for frequencies from 0.5 to 100 GHz”.

[4]	R4-1712124, “TR 38.817-02 v0.4.0: General aspects for BS RF for NR”, Ericsson.