3GPP TSG-RAN WG4 Meeting #85	R4-1714330

Reno, Nevada, USA, 25thNovember – 1st December 2017

Source: 	Ericsson

Title:  		TP to TS 38.101-2 v0.1.1: ON/OFF time masks (Section 6.3.3)

Agenda Item:	9.4.3.5

Document for:	Approval

# Background

During RAN4#NR AH3, a way forward has been agreed on UE ON/OFF mask design in [2]. Based on this way forward, we provided a contribution where we proposed our understanding related to UE ON/OFF mask in [3].

# Proposal

We propose to adopt the text in this contribution to TR 38.101-2.

# References

R4-1710085, TR 38.817-02 v0.2.0: General aspects for BS RF for NR

R4-1709946, WF on ON/OFF mask use cases for NR UE transmissions, Ericsson, Qualcomm

R4-1713527, Further discussions on ON/OFF mask design for NR UE transmissions, Ericsson

# Text proposal for TS 38.101-2

# <Start of Text proposal>

## 7.6	Transmit intermodulation

Detailed structure of the subclause is TBD.

## 7.7 ON/OFF time mask for FR2

### 7.7.1 UE transient time for FR2

### 7.7.2 General ON/OFF time mask

The General ON/OFF time mask defines the observation period between Transmit OFF and ON power and between Transmit ON and OFF power. ON/OFF scenarios include; the beginning or end of DTX, measurement gap, contiguous, and non contiguous transmission, etc

The OFF power measurement period is defined in a duration of at least one slot excluding any transient periods. The ON power is defined as the mean power over one slot excluding any transient period.

Figure 7.7.1-1: General ON/OFF time mask for NR UL transmission in FR2

### 7.7.3 PRACH time mask

The PRACH ON power is specified as the mean power over the PRACH measurement period excluding any transient periods as shown in Figure 7.7.3-1. The measurement period for different PRACH preamble format is specified in Table 7.7.3-1.

Table 7.7.3-1: PRACH ON power measurement period

| PRACH preamble format | Measurement period (ms) |
| --- | --- |
| TBD | TBD |
| TBD | TBD |
| TBD | TBD |
| TBD | TBD |
| TBD | TBD |

Figure 5.7.3-1: PRACH ON/OFF time mask

### 7.7.4 SRS time mask

In the case a single SRS transmission, the ON power is defined as the mean power over the symbol duration excluding any transient period; Figure 7.7.4-1.

Figure 7.7.4-1: Single SRS time mask for NR UL transmission

In the case multiple consecutive SRS transmission, the ON power is defined as the mean power for each symbol duration excluding any transient period. See Figure 7.7.4-2

Figure 7.7.4-2: Consecutive SRS time mask for the case when no power change is required

<Editor’s note: Consecutive SRS time mask with power change to be defined later>

### 7.7.5 PUCCH / PUSCH / SRS time mask

<Editor’s note: PUCCH/PUSCH/SRS  time mask with power change to be defined later>

# 8	UE Receiver characteristics (frequency range 2)

## 8.1	General

Detailed structure of the subclause is TBD.

# <End of Text proposal>