3GPP TSG-RAN WG4 #84	R4-1714338

November 27st – December 1st, 2017

Reno, Neveda, USA

Agenda item:	9.4.4.4.2

Source: 	Qualcomm Incorporated

Title: 	TP to TS 38.101-2 IBB requirement for mmW (section 7.6.1)

Document for:	Approval

# Introduction

In-band blocking (IBB) requirements for range 2 NR were already agreed in case of single carrier transmission [1][2].

In this contribution, we propose IBB requirements for carrier aggregation (CA) and we make a text proposal for section 7.6.1 of 38.101-2.

# Discussion

In the following, we will make specific proposal to finalize IBB requirements including tests specifications for the CA cases. The following sections are organized as follows:

- Summary of single carrier requirements

- Proposals for CA requirements

- Text Proposals for 38.801-2 specifications

## IBB: single carrier requirement

The IBB requirement was studied in [3] and agreed based on proposals in [1]. Like for the ACS test case, it was decided to have same unwanted and wanted signals bandwidth in IBB test. It was also agreed to have a variable location of IBB compared to the wanted signal, in particular center frequency of IBB signals can go from FDL_low - 1.5 BW to FDL_low + 1.5 BW, as shown in Figure 1.

Regarding the wanted and IBB signals, it was agreed to define the test as follows [2]:

- Wanted signal is at REFSENS + 14dB

- REFSENS + 42dB for bands n257, n258, n259

- REFSENS + 41dB for band n260

Figure 1. Pictorial representation of wanted, ACS and IBB test signals for the single carrier case.

## Proposals for CA requirements

Three different cases need to be taken into account:

- Intra-band contiguous CA

- Intra-band non-contiguous CA

- Inter band CA

### Intra-band contiguous CA

For intra-band contiguous CA, we follow the approach adopted for ACS. i.e. the IBB test signal bandwidth is equal to the aggregated wanted signal bandwidth.

Proposal 1: for the intra-band contiguous case, the ACS signal bandwidth is equal to the aggregated wanted signal bandwidth.

Regarding, the signal levels, the following proposal is made:

Proposal 2: for the intra-band contiguous CA case, the IBB test should be specified as follows:

- Power in transmission bandwidth configuration per CC = REFSENS + 14dB

- Pinterfer = aggregated power + 42 dB for band n257, n258

- Pinterfer = aggregated power + 41 dB for band n260

Figure 2. Pictorial representation of wanted and IBB test signals for the intra-band contiguous CA case.

### Intra-band non-contiguous CA

Even for the IBB test case, we have the two possibilities to define the non-contiguous CA test based on the component carriers BW or based on the end-to-end bandwidth. For the similar reason discussed above, our preference is to define the test based on the component carriers BW and thus defining the test provided that a minimum gap is required. As it can be seen in Figure 3, if we use the same approach adopted in ACS test and we focus on the wider of the component carriers, a minimum frequency separation ∆fIBB ≥ 1.5 (BW1+ BW2) + 3 max(BW1, BW2) is needed to define the in-gap requirement. Regarding the level, we acknowledge some concerns raised due to very large receiver BW, so we propose to use an IBB level equal to the ACS level.

Proposal 3: for the intra-band non-contiguous CA case, the IBB test should be specified as follows

- Within the gap the test is defined only if ∆fIBB ≥ 0.5(BW1 + BW2) + 3 max(BW1, BW2) and the tests parameters for the single carrier applies considering the largest channel bandwidth between BW1 and BW2

- For both in-gap and out-of-gap IBB signal level is equal to the ACS signal level for the corresponding configuration.

Figure 3. Pictorial representation of wanted, ACS and IBB test signals for the intra-band non-contiguous CA.

### Inter-band CA

For the inter-band CA case, we propose to use the same test as the single carrier case apply to each component carrier.

Proposal 4: for the intra-band non-contiguous CA IBB test case, the single carrier test applies to each component carrier.

# Text Proposal for TS 38.801-2

In this section we provide text proposal to 38.801-2.

Proposal 5: to approve the following Text Proposal to TS 38.801-2.

<<<<<<<<  Begin Text Proposal to TS 38.101-2   >>>>>>>>

### 7.6.1	In-band blocking

In-band blocking is defined for an unwanted interfering signal falling into the UE receive band or into the spectrum equivalent to twice the channel bandwidth below or above the UE receive band at which the relative throughput shall meet or exceed the minimum requirement for the specified measurement channels.

#### 7.6.1.1	Minimum requirements

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.X.X (with one sided dynamic OCNG Pattern for the DL-signal as described in Annex A.X.X.X) with parameters specified in Tables 7.6.1.1-1.

Table 7.6.1.1-1: In band blocking minimum requirements

| Rx parameter | Units | Channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| Power in Transmission Bandwidth Configuration | dBm | REFSENS + 14dB |  |  |  |
| BWInterferer | MHz | 50 | 100 | 200 | 400 |
| PInterferer for bands n257, n258 | dBm | REFSENS + 42 | REFSENS + 42 | REFSENS + 42 | REFSENS + 42 |
| PInterferer for bands n260 | dBm | REFSENS + 41 | REFSENS + 41 | REFSENS + 41 | REFSENS + 41 |
| FIoffset | MHz | 100 / -100 NOTE 4 | 200 / -200 NOTE 4 | 400 / -400 NOTE 4 | 800 / -800 NOTE 4 |
| FInterferer | MHz | FDL_low – 75 to FDL_low + 75 | FDL_low – 150 to FDL_low + 150 | FDL_low – 300 to FDL_low + 300 | FDL_low – 600 to FDL_low + 600 |
| NOTE 1: The interferer consists of the Reference measurement channel specified in Annex A.X.X with one sided dynamic OCNG Pattern as described in Annex A.X.X.X and set-up according to Annex C.X.X. NOTE 2: The REFSENS power level is specified in Table 7.X.X.X. NOTE 3: The wanted signal consists of the reference measurement channel specified in Annex A.X.X (QPSK, R=X/X) with one sided dynamic OCNG pattern as described in Annex A.X.X.X and set-up according to Annex C.X.X NOTE 4: The absolute value of the interferer offset FInterferer (offset) shall be further adjusted to FInterferer/SCS+0.5SCS MHz with SCS the sub-carrier spacing of the wanted signal in MHz. Wanted and interferer signal have same SCS. |  |  |  |  |  |

For the UE which supports inter band CA configuration in Table 7.3.1-1A, PInterferer power defined in Table 7.6.1.1-2 is increased by the amount given by ΔRIB,c in Table 7.3.1-1A.

#### 7.6.1.1A	Minimum requirements for CA

For intra-band contiguous carrier aggregation with two component carriers and aggregated bandwidth BWChannel_CA up to 400MHz, the SCC(s) shall be configured at nominal channel spacing to the PCC. The UE shall fulfil the minimum requirement specified in Table 7.6.1.1A-1 for an adjacent channel interferer on either side of the aggregated downlink signal at a specified frequency offset and for an interferer power up to -25 dBm. The throughput of each carrier shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.X.X  (with one sided dynamic OCNG Pattern for the DL-signal as

described in Annex A.X.X.X).

For intra-band contiguous carrier aggregation with more than two component carriers or aggregated bandwidth BWChannel_CA larger than 400MHz the requirement is FFS.

Table 7.6.1.1A-1: In band blocking minimum requirements for intra-band contiguous CA

| Rx Parameter | Units | CA Bandwidth Class |
| --- | --- | --- |
|  |  | All CA bandwidth Classes |
| Pw in Transmission Bandwidth Configuration, per CC |  | REFSENS + 14 dB |
| Pinterferer for band n257, n258 | dBm | Aggregated power + 28 |
| Pinterferer for band n260 | dBm | Aggregated power + 27 |
| BWInterferer | MHz | BWChannel_CA |
| FIoffset | MHz | + BWChannel_CA / BWChannel_CA NOTE 4 |
| FInterferer | MHz | FDL_low – 1.5*BWChannel_CA To FDL_high + 1.5*BWChannel_CA |
| NOTE 1: The interferer consists of the Reference measurement channel specified in Annex X.X with one sided dynamic OCNG Pattern as described in Annex A.X.X.X and set-up according to Annex C.X.X. NOTE 2: The REFSENS power level is specified in Table 7.X.X.X. NOTE 3: The Finterferer (offset) is the frequency separation between the center of the aggregated CA bandwidth and the center frequency of the Interferer signal. NOTE 4: The absolute value of the interferer offset FInterferer (offset) shall be further adjusted to FInterferer/SCS+0.5SCS MHz with SCS the sub-carrier spacing of the wanted signal in MHz. Wanted and interferer signal have same SCS. |  |  |

For intra-band non-contiguous carrier aggregation with two component carriers and aggregated bandwidth BWChannel_CA up to 400MHz, the requirement applies to out-of-gap and in-gap. For out-of-gap, the UE shall meet the requirements for each component carrier with parameters as specified in 7.6.1.1A-2. The requirement associated to the maximum channel between across the component carriers is selected. For in-gap, the requirement shall apply if the following minimum gap condition is met: ∆fIBB ≥ 0.5(BW1 + BW2) + 3 max(BW1, BW2), where ∆fIBB is the frequency separation between the center frequencies of the component carriers and BWk/2 are the channel bandwidths of carrier k, k = 1,2. If the minimum gap condition is met, the UE shall meet the requirement specified in Table 7.6.1.1A-2 considering parameters associated to the largest channel bandwidth between the two component carriers, i.e. max(BW1, BW2). For both out-of-gap and in-gap, the requirements shall apply while all downlink carriers are active.

For intra-band non-contiguous carrier aggregation with more than two component carriers or aggregated bandwidth BWChannel_CA larger than 400MHz the requirement is FFS.

Table 7.6.1.1A-2: In band blocking minimum requirements for intra-band non-contiguous CA

| Rx parameter | Units | Channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| Power in Transmission Bandwidth Configuration | dBm | REFSENS + 14dB |  |  |  |
| BWInterferer | MHz | 50 | 100 | 200 | 400 |
| PInterferer for bands n257, n258 | dBm | REFSENS + [35.5] | REFSENS + [35.5] | REFSENS + [35.5] | REFSENS + [35.5] |
| PInterferer for bands n260 | dBm | REFSENS + [34.5] | REFSENS + [34.5] | REFSENS + [34.5] | REFSENS + [34.5] |
| FIoffset | MHz | 100 / -100 NOTE 4 | 200 / -200 NOTE 4 | 400 / -400 NOTE 4 | 800 / -800 NOTE 4 |
| FInterferer | MHz | FDL_low – 75 to FDL_low + 75 | FDL_low – 150 to FDL_low + 150 | FDL_low – 300 to FDL_low + 300 | FDL_low – 600 to FDL_low + 600 |
| NOTE 1: The interferer consists of the Reference measurement channel specified in Annex A.X.X with one sided dynamic OCNG Pattern as described in Annex A.X.X.X and set-up according to Annex C.X.X. NOTE 2: The REFSENS power level is specified in Table 7.X.X.X. NOTE 3: The wanted signal consists of the reference measurement channel specified in Annex A.X.X (QPSK, R=X/X) with one sided dynamic OCNG pattern as described in Annex A.X.X.X and set-up according to Annex C.X.X NOTE 4: The absolute value of the interferer offset FInterferer (offset) shall be further adjusted to FInterferer/SCS+0.5SCS MHz with SCS the sub-carrier spacing of the wanted signal in MHz. Wanted and interferer signal have same SCS. |  |  |  |  |  |

<<<<<<<<  End Text Proposal to TS 38.101-2   >>>>>>>>

# Reference

- R4-1711399, “Finalization of UE IBB requirement for mmW”, Qualcomm Incorporated.

- RAN4#84Bis Meeting report.

- R4-1708449, “On UE IBB requirement for mmW”, Qualcomm Incorporated.

- R4-1711979, “3GPP TS 38.101-2 v0.1.0 (2017-10)”.

- R4-1705478, “UE ACLR, ACS and IBB requirements for mmW”, Qualcomm Incorporated.