3GPP TSG-WG RAN4 Meeting #86	R4-1801464

# Athens, Greece 26th Feb – 2nd March, 2018

Source: 	Nokia, Nokia Shanghai Bell, AT&T

Title: 					TP to TR 36.715-04-01: CA_4DL_2A-29A-30A-66A_1UL_BCS0

Agenda Item: 			6.4.2

Document for:	Approval

# Introduction

This TP introduces 4DL/1UL CA configuration CA_2A-29A-30A-66A in to the TR 36.715-04-01.

**************************** Start of changes ********************************************

## 5.X	CA_2A-29A-30A-66A_BCS0

### 5.X.1	Channel bandwidths per operating band for CA

Table 5.X.1-1: Supported E-UTRA bandwidths per CA configuration for 4DL inter-band CA

| CA operating / Channel bandwidth |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA Configuration | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth Combination Set |
| CA_2A-29A-30A-66A | 2 |  |  | Yes | Yes | Yes | Yes | 60 | 0 |
|  | 29 |  |  | Yes | Yes |  |  |  |  |
|  | 30 |  |  | Yes | Yes |  |  |  |  |
|  | 66 |  |  | Yes | Yes | Yes | Yes |  |  |

### 5.X.2	Co-existence studies

Table 5.X.2-1 summarizes frequency ranges where harmonics occur due to Band 2 and Band 46 CA with 1 UL.

Table 5.X.2-1: Impact of 1 UL Harmonic Interference

|  |  |  | 2nd Harmonic |  | 3rd Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| 2 | 1850 | 1910 | 3700 | 3820 | 5550 | 5730 |
| 30 | 2305 | 2315 | 4610 | 4630 | 6915 | 6945 |
| 66 | 1710 | 1780 | 3420 | 3560 | 5130 | 5340 |

It can be seen from Table 5.X.2-1 that there is no harmonic relation for this band combination.

### 5.X.3	∆TIB,c and ∆RIB,c values

For 4DL/1UL CA_2A-29A-30A-66A, the TIB,c and RIB,c values are shown in Table 5.X.3-1 and Table 5.X.3-2, respectively, as derived from CA_2A-30A-66A. The low SDL band (band 29) is simply diplexed.

Table 5.X.3-1: ΔTIB,c for 4DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_2A-29A-30A-66A | 2 | 0.5 |
|  | 30 | 0.3 |
|  | 66 | 0.5 |

Table 5.X.3-2: ΔRIB,c for 4DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_2A-29A-30A-66A | 2 | 0.4 |
|  | 30 | 0.5 |
|  | 66 | 0.4 |

### 5.X.4	REFSENS requirements

Reference sensitivity for CA_2A-29A-30A-66A are specified in Table 5.X.4-1 for the uplink configurations specified in Table 7.3.1-2 of TS36.101.

Table 5.X.4-1: Reference sensitivity QPSK PREFSENS (CA with a SDL band)

| Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| CA_2A-29A-30A-66A | 2 |  |  | -97.6 | -94.6 | -92.8 | -91.6 | FDD |
|  | 29 |  |  | -97 | -94 |  |  |  |
|  | 30 |  |  | -98.5 | -95.5 |  |  |  |
|  | 66 |  |  | -99.1 | -96.1 | -94.3 | -93.1 |  |

**************************** End of changes ********************************************