3GPP TSG-RAN WG4 Meeting #86	R4-1802886

Athens, GR, 26th Feb. – 2nd March, 2018

Source:	NTT DOCOMO, INC.

Title:	On uncertainty values for EIRP measurement for anechoic chamber test method

Agenda item:	6.27.3.1.4

Document for:	Approval

1	Introduction

In the last RAN4 meeting #84bis, it was agreed on the procedure for the discussion on TRP measurement for Rel-15 eAAS conformance test specification [1]. Following agreement, EIRP uncertainty for each TRP requirement will be discussed in the following meetings. Moreover, it was also agreed that TRP requirements can be classified into 3 categories. Cat.1 is output power performance on the wanted signal. Cat. 2 includes out of band unwanted emissions such as OBUE and ACLR. Cat. 3 is for spurious emission. These categorized criteria are based on output power and frequency difference. In this contribution, we discuss on factors for evaluating uncertainty values for EIRP measurement for indoor anechoic chamber test method.

2	Discussion

2.1 TRP definition

In TR.37.843 [2], there is a description on TRP requirement as follows

- TRP requirements

  - TRP is defined as:

,where EIRP is the total EIRP of two orthogonal polarizations.

Correspondingly, and in order that the test requirement part of conformance test specification is measurable the total power the above equation is approximated as the sum of the EIRP at a number of discrete directions around the sphere as follows:

,where EIRP is sampled at N locations along the -axis and M locations along the φ-axis.

2.2 Agreed procedure on TRP measurement

In the last RAN4 meeting, we agreed the procedure on TRP measurement for Rel-15 eAAS conformance test specification as followings,

Observation1: As TRP measurement is defined as the sum of EIRP measurement results, EIRP measurement methods and measurement uncertainty have to be confirmed for each TRP requirement.

Proposal 1:  RAN4 should discuss EIRP measurement methods and measurement uncertainty on each TRP requirement except for output power firstly.

Observation2: TRP requirements can be classified into 3 categories. Cat.1 is output power performance on the wanted signal. Cat. 2 includes out of band unwanted emissions such as OBUE and ACLR. Cat. 3 is spurious emission.

Proposal 2:  RAN4 should consider EIRP measurement methods and measurement uncertainty by categories which are classified from output power and frequency.

Proposal 3:  After discussion on EIRP measurements for TRP requirements, TRP measurement method and measurement uncertainty should be discussed on TRP requirements for each category.

2.3 On EIRP uncertainty values of each TRP requirement for indoor anechoic chamber test method

We need to consider EIRP uncertainty values of each TRP requirement for indoor anechoic chamber test method except for output power which has already been discussed. Indoor anechoic chamber test method is based on the measurement in the far field. The system setup of EIRP measurement for each TRP requirement is fundamentally the same as that of output power requirement as shown in Fig. 10.3.1.1.2.101 in TR 37.842. Therefore, test procedure and calibration on EIRP measurement for TRP requirements are also the same as the descriptions in TR 37.842.

It is apparent that uncertainty value on EIRP indoor anechoic chamber measurement for Cat.1, which is for output power, is the same as Table 10.3.1.1.2.5-1.1 described in TR 37.842. Here, the following uncertainty distribution and standard uncertainty (σ) values proposed by test vendors were adopted for the RF power measurement equipment, RF signal generator, and network analyzer to calculate the uncertainty budget.

According to Table E-1 in TR 37.842, standard uncertainty of RF power measurement equipment on total amplitude accuracy is not indicated when the input levels down to under -70 dBm. When we consider the uncertainty value on EIRP measurement for Cat.2, which is for out of band emission, the measurement data of below -70 dBm in the OTA region is possible because of pass loss between DUT and receiving antenna. On the other hand, it is difficult to find the standard uncertainty of RF measurement equipment under -70 dBm from test venders’ sites. Therefore, to maintain the integrity of the specifications, the input power to RF power measurement system should be larger than -70 dBm to utilize the same test equipment uncertainty values as Table E-1 in TR 37.842.

# Table 1: Test equipment uncertainty values

| Instrument | Use case | Measurement Uncertainty type | Standard uncertainty σ (dB) |  | Probability distribution |
| --- | --- | --- | --- | --- | --- |
|  |  |  | f ≦ 3 GHz | 3 GHz < f ≦ 4.2 GHz |  |
| RF power measurement equipment (e.g. spectrum analyzer, power meter) | EIRP measurement stage | Total amplitude accuracy (with input levels down to -70 dBm) | 0.14 | 0.26 | Gaussian |
| RF signal generator | EIS measurement stage | Level error | 0.46 | 0.46 | Gaussian |
| Network analyzer | Calibration stage | Accuracy of transmission measurements | 0.13 | 0.20 | Gaussian |
| NOTE: Standard uncertainty values were derived from datasheets of mid-tier to high-end RF signal generators, spectrum analyzers, and VNAs. Standard uncertainty values of power measurement equipment were derived from datasheet of spectrum analyzers. |  |  |  |  |  |

Observation 1: According to Table E-1 in TR 37.842, standard uncertainty of RF power measurement equipment on total amplitude accuracy is not indicated when the input levels down to under -70 dBm.

Observation 2: When we consider the uncertainty value on EIRP measurement for Cat.2, which is for out of band emission, the measurement data of below -70 dBm in the OTA region is possible because of pass loss between DUT and receiving antenna.

Proposal 1: The input power to RF power measurement system should be larger than -70 dBm to utilize the same test equipment uncertainty values as Table E-1 in TR 37.842 for anechoic chamber test method.

The uncertainty distribution and standard uncertainty (σ) value for the reference antenna derived as the maximum of companies' proposals were also adopted to calculate the uncertainty budget when we discussed Rel-13 specification described in Table E-2 in TR 37.842.

According to Tables E-1 and E-2 in TR 37.842, standard uncertainties of test equipment and reference antenna are not indicated in the whole frequency range of spurious emission which is defined as the frequency range from 30 MHz to 12.75 GHz. Therefore, RAN4 should agree the uncertainty distribution and standard uncertainty values for test equipment and reference antenna and update Tables E-1 and E-2 for Cat.3 to Table 2 and Table 3.

# Table 2: Test equipment uncertainty value

| Instrument | Use case | Measurement Uncertainty type | Standard uncertainty σ (dB) |  | Probability distribution |
| --- | --- | --- | --- | --- | --- |
|  |  |  | f ≦ 3 GHz | 3 GHz < f ≦ 12.75 GHz |  |
| RF power measurement equipment (e.g. spectrum analyzer, power meter) | EIRP measurement stage | Total amplitude accuracy (with input levels down to -70 dBm) | 0.14 | 0.26 | Gaussian |
| Network analyzer | Calibration stage | Accuracy of transmission measurements | 0.13 | 0.20 | Gaussian |
| NOTE: Standard uncertainty values were derived from datasheets of mid-tier to high-end RF signal generators, spectrum analyzers, and VNAs. Standard uncertainty values of power measurement equipment were derived from datasheet of spectrum analyzers. |  |  |  |  |  |

# Table 3: Reference antenna uncertainty value

| Instrument | Use case | Standard uncertainty σ (dB) |  | Probability distribution |
| --- | --- | --- | --- | --- |
|  |  | f ≦ 3GHz | 3GHz < f ≦ 12.75 GHz |  |
| Reference antenna | Calibration stage | 0.29 | 0.25 | Rectangular |

Observation 3: According to Table E-1 in TR 37.842, standard uncertainty of test equipment is not indicated in the whole frequency range of spurious emission which is from 30 MHz to 12.75 GHz.

Proposal2: For anechoic chamber method, test equipment uncertainty value between 4.2GHz and 12.75 GHz should be 0.25, which is the same as that between 3GHz and 4.2 GHz if other values are not proposed for test equipment uncertainty value between 4.2GHz and 12.75 GHz in the next meeting.

Observation 4: According to Table E-2 in TR 37.842, standard uncertainty of reference antenna is not indicated in the whole frequency range of spurious emission which is from 30 MHz to 12.75 GHz.

Proposal 3: For anechoic chamber method, the reference antenna uncertainty value between 4.2GHz and 12.75 GHz should be 0.25, which is the same as that between 3GHz and 4.2 GHz if other values are not proposed for the reference antenna uncertainty value between 4.2GHz and 12.75 GHz in the next meeting.

3	Summary

We discuss on measurement uncertainty and propose some requirements for evaluating values. After discussion above, we made the following proposal based on the following observations.

Observation 1: According to Table E-1 in TR 37.842, standard uncertainty of RF power measurement equipment on total amplitude accuracy is not indicated when the input levels down to under -70 dBm.

Observation 2: When we consider the uncertainty value on EIRP measurement for Cat.2, which is for out of band emission, the measurement data of below -70 dBm in the OTA region is possible because of pass loss between DUT and receiving antenna.

Proposal 1: The input power to RF power measurement system should be larger than -70 dBm to utilize the same test equipment uncertainty values as Table E-1 in TR 37.842 for anechoic chamber test method.

Observation 3: According to Table E-1 in TR 37.842, standard uncertainty of test equipment is not indicated in the whole frequency range of spurious emission which is from 30 MHz to 12.75 GHz.

Proposal2: For anechoic chamber method, test equipment uncertainty value between 4.2GHz and 12.75 GHz should be 0.25, which is the same as that between 3GHz and 4.2 GHz if other values are not proposed for test equipment uncertainty value between 4.2GHz and 12.75 GHz in the next meeting.

Observation 4: According to Table E-2 in TR 37.842, standard uncertainty of reference antenna is not indicated in the whole frequency range of spurious emission which is from 30 MHz to 12.75 GHz.

Proposal 3: For anechoic chamber method, the reference antenna uncertainty value between 4.2GHz and 12.75 GHz should be 0.25, which is the same as that between 3GHz and 4.2 GHz if other values are not proposed for the reference antenna uncertainty value between 4.2GHz and 12.75 GHz in the next meeting.

References

- R4-1710996, “How to proceed to discussion on TRP measurement methods for conformance test requirements,” NTT DOCOMO, RAN4#84bis, October, 2017

- 3GPP TR 37.843 V0.3.0.