3GPP TSG RAN WG4 Meeting #86	R4-1803453

# Athens, Greece, 26 February – 2 March 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-2 | CR |  | rev | - | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of requirements for Intra-band CA combinations NR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-03-02 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Introduction of requirements for Intra-band CA combinations NR |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduction of requirements for the following Intra-band CA combination NR: CA_n257B CA_n257D CA_n257E CA_n257F CA_n257G CA_n257H CA_n257I CA_n257J CA_n257K CA_n257L CA_n257M |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Requirements for Intra-band CA combinations NR are not specified |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5, 6, 7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  | x |  | Test specifications |  |  | TS/TR ... CR ... 38.521-3 |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<Start of changes>

### 5.2A.1	Intra-band CA

NR intra-band contiguous carrier aggregation is designed to operate in the operating bands defined in Table 5.2A.1-1, where all operating bands are within FR2.

Table 5.2A.1-1: Intra-band contiguous CA operating bands in FR2

| NR CA Band | NR Band (Table 5.2-1) |
| --- | --- |
| CA_n257B | n257 |
| CA_n257D | n257 |
| CA_n257E | n257 |
| CA_n257F | n257 |
| CA_n257G | n257 |
| CA_n257H | n257 |
| CA_n257I | n257 |
| CA_n257J | n257 |
| CA_n257K | n257 |
| CA_n257L | n257 |
| CA_n257M | n257 |

<Unchanged sections omitted>

### 5.3.5A Channel bandwidth per operating band for CA

Table 5.3.5A-1: CA bandwidth classes

| NR CA bandwidth class | Aggregated channel bandwidth | number of contiguous CC | Fallback group |
| --- | --- | --- | --- |
| A | CBW ≤ 400 MHz | 1 |  |
| B | 400 MHz < CBW ≤ 800 MHz | 2 | 1 |
| C | 800 MHz < CBW ≤ 1200 MHz | 3 |  |
| D | 200 MHz < CBW ≤ 400 MHz | 2 | 2 |
| E | 400 MHz < CBW ≤ 600 MHz | 3 |  |
| F | 600 MHz < CBW ≤ 800 MHz | 4 |  |
| G | 100 MHz < CBW ≤ 200 MHz | 2 | 3 |
| H | 200 MHz < CBW ≤ 300 MHz | 3 |  |
| I | 300 MHz < CBW ≤ 400 MHz | 4 |  |
| J | 400 MHz < CBW ≤ 500 MHz | 5 |  |
| K | 500 MHz < CBW ≤ 600 MHz | 6 |  |
| L | 600 MHz < CBW ≤ 700 MHz | 7 |  |
| M | 700 MHz < CBW ≤ 800 MHz | 8 |  |
| O | 100 MHz ≤ CBW ≤ 400 MHz | 2 | 4 |
| P | 150 MHz ≤ CBW ≤ 400 MHz | 3 |  |
| Q | 200 MHz ≤ CBW ≤ 400 MHz | 4 |  |

Table 5.3.5A1-2: NR CA configurations and bandwidth combination sets defined for intra-band contiguous CA

|  |  | E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configurations | Component carriers in order of increasing carrier frequency |  |  |  |  |  |  |  | Maximum aggregated BW [MHz] | BCS | Fallback group |
|  |  | CBW [MHz] | CBW [MHz] | CBW [MHz] | CBW [MHz] | CBW [MHz] | CBW [MHz] | CBW [MHz] | CBW [MHz] |  |  |  |
| CA_257B |  | 50, 100 , 200 ,400 | 400 |  |  |  |  |  |  | 800 | 0 | 1 |
| CA_257D |  | 50, 100, 200 | 200 |  |  |  |  |  |  | 400 | 0 | 2 |
|  |  | 100 | 100 |  |  |  |  |  |  |  |  |  |
| CA_257E |  | 50, 100, 200 | 200 | 200 |  |  |  |  |  | 600 | 0 | 2 |
| CA_257F |  | 50, 200 | 200 | 200 | 200 |  |  |  |  | 800 | 0 | 2 |
| CA_257G |  | 50 | 100 |  |  |  |  |  |  | 150 | 0 | 3 |
| CA_257H |  | 100 | 100 | 100 |  |  |  |  |  | 300 | 0 | 3 |
| CA_257I |  | 100 | 100 | 100 | 100 |  |  |  |  | 400 | 0 | 3 |
| CA_257J |  | 50, 100 | 100 | 100 | 100 | 100 |  |  |  | 500 | 0 | 3 |
| CA_257K |  | 100 | 100 | 100 | 100 | 100 | 100 |  |  | 600 | 0 | 3 |
| CA_257L |  | 50, 100 | 100 | 100 | 100 | 100 | 100 | 100 |  | 700 | 0 | 3 |
| CA_257M |  | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 800 | 0 | 3 |

<Unchanged sections omitted>

6.4A	Transmit signal quality for CA

6.4A.2.2	Carrier leakage for CA

For intra-band contiguous CA with UL and DL configured for same component carriers, carrier leakage is an additive sinusoid waveform that is confined within the aggregated transmission bandwidth configuration. The carrier leakage requirement is defined for each component carrier and is measured on the component carrier with PRBs allocated. The measurement interval is one slot in the time domain.

For intra-band contiguous CA with different component carriers configured for UL than for DL, carrier leakage is an additive sinusoid waveform that is confined within aggregated receiver bandwidth configuration.  The carrier leakage requirement is defined for each component carrier and is measured on the component carrier with PRBs allocated. The measurement interval is one slot in the time domain.

For intra-band non-contiguous CA, carrier leakage is an additive sinusoid waveform that is confined within outer edges of the aggregated receiver bandwidth configuration. The carrier leakage requirement is defined for each component carrier and is measured on the component carrier with PRBs allocated. The measurement interval is one slot in the time domain.

Note: For intra-band non-contiguous CA, carrier leakage may land outside component carrier bandwidth.

The relative carrier leakage power is a power ratio of the additive sinusoid waveform and the modulated waveform. The relative carrier leakage power shall not exceed the values specified in Table 6.4.2.2A-1.

Table 6.4.2.2A-1: Minimum requirements for Relative Carrier Leakage Power

| Parameters | Relative Limit (dBc) |
| --- | --- |
| Output power >TBD dBm | [-25] |
| -TBD dBm ≤ Output power ≤ TBD dBm | [-20] |
| -TDB dBm ≤ Output power < -TBD dBm | [-10] |

6.4A.2.3	IQ Image for CA

IQ Image for DL CA is specified in relation to DL carrier frequency.

Table 6.4.2.3-1: Requirements for IQ Image

| Parameter description | Unit | Limit (NOTE 1) |  | Applicable Frequencies |
| --- | --- | --- | --- | --- |
| IQ Image | dB | [-25] | Output power > 10 dBm | Image frequencies |
|  |  | [-20] | Output power ≤ 10 dBm |  |
| NOTE 1: The measurement bandwidth is 1 RB and the limit is expressed as a ratio of measured power to the measured average power per allocated RB, where the averaging is done across all allocated RBs. For pulse-shaped pi/2 BPSK, the limit is expressed as a ratio of measured power in one non-allocated RB to the measured power in the allocated RB with highest PSD NOTE 2: The measurement bandwidth is 1 RB and the limit is expressed as a ratio of measured power in one non-allocated RB to the measured total power in all allocated RBs. NOTE 3: All powers are EIRP in beam peak direction.. |  |  |  |  |

<Unchanged sections omitted>

7.3	Reference sensitivity power level

For intra-band contiguous carrier aggregation the throughput of each component carrier shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in [Annex A] (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal) with peak reference sensitivity specified in Table 7.3-1.

<End of changes>