3GPP TSG-RAN WG4 Meeting #86-bis	R4-1803668

Melbourne, Australia, 16 – 20 Apr, 2018

Source:	Mediatek Inc

Title:	TP for TR36.715-04-01: the support of CA_2C-66A-66A_1UL_BCS0

Agenda item:	6.4.3

Document for:	Approval

1.  Introduction

CA_2C-66A-66A_1UL_BCS0 was approved in 4DL/1UL basket WID in the RAN-P [1]. This paper is to provide TP for the completion of this combination.

2.  Reference

[1]      RP-180572 LTE Advanced inter-band CA Rel-15 for 4DL/1UL, RAN-P #79.

Annex: TP for TR36.715-04-01 (va.b.c)

[Unchanged portions skipped]

5.x	CA_2C-66A-66A _BCS0

5.x.1	Channel bandwidths per operating band for CA

Table 5.x.1-1: Inter-band CA

| E-UTRA CA Band | E-UTRA operating Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | UE transmit / BS receive |  |  | UE receive / BS transmit |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_2-66 | 2 | 1850 MHz | – | 1980 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | 66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz |  |

Table 5.x.1-2: Supported E-UTRA bandwidths per CA configuration for inter-band CA

| CA operating / channel bandwidth |  |  |  |  |  |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth Combination Set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |  |  |
| CA_2C-66A-66A | 2 | See CA_2C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  | 80 | 0 |
|  | 66 | See CA_66A-66A Bandwidth Combination Set 0 in Table 5.6A.1-3 in 36.101 |  |  |  |  |  |  |  |

5.x.2	UE Co-existence studies

The analysis has been done in its fallback modes. The results can be reused.

5.x.3	∆TIB and ∆RIB values

The TIB,c values are derived from CA_2C-66A-66A and are shown in table 5.x.3-1.

# Table 5.x.3-1: ΔTIB,c for 3DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_2C-66A-66A | 2 | 0.5 |
|  | 66 | 0.5 |

The RIB,c values are derived from CA_2C-66A-66A  and are shown in table 5.x.3-2.

# Table 5.x.3-2: ΔRIB,c for 3DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_2C-66A-66A | 2 | 0.3 |
|  | 66 | 0.3 |

5.x.4	REFSENS requirements

No new REFSENS requirements need to be specified for CA_2C-66A-66A.

[Unchanged portions skipped]

form change history:

2013-12-06 v1.14.1 modified §11 to read: <FamilyName>, <GivenName>, (If the person is new to 3GPP work, give full contact coordinates, in particular, email address.)

2013-10-03 v1.14.0 removal of embedded help text

v1.13.2: adds tdoc header

v1.13.1: minor changes resulting from discussions at CT#41 & SA#41

v1.13.0: mods to enforce linkage amongst stages 1, 2, 3

draft mods Scarrone-Meredith 2008-07 ff

v1.12.1: removes revision marks following approval at SP-29
v1.12.0: includes provision for Study Items (SP-29)

v1.11.0: includes those changes from v1.8.0 agreed at SP-25.

v1.10.0: full circle

v1.9.0: a clean sheet

v1.8.0: includes comments from SA#24

v1.7.0: includes comments from RAN, CN and T #24; also includes “early implementation” data

v1.6.0: includes comments made during review period prior to TSGs#24

v1.5.0: includes comments made at TSGs#23 (Phoenix)

v1.4.0: offered to SA#23 for approval

v1.3.0: offered to CN#23, RAN#23 and T#23 for comments

DRAFT4 v1.3.0: 2004-03-09: Incorporation of comments from Leaders list

DRAFT3 v1.3.0: 2004-02-19: Incorporation of comments from MCC members

DRAFT2 v1.3.0: 2004-01-29: Complete redraft:

v1.2.0: 2002-07-04: "USIM" box changed to "UICC apps"

2003-05-28: spelling of “rapporteur” corrected

2002-07-04: "USIM" box changed to "UICC apps"