3GPP TSG-RAN WG4 Meeting #86bis	R4-1804670

Melbourne, Australia, 16 - 20 Apr, 2018

Agenda item:	7.4.4

Source:	Dish Network

Title:	Pi/2 BPSK and spectrum shaping

Document for:	Discussion

# 1	Introduction

This contribution discusses Pi/2 BPSK with spectrum shaping.

# 2	Discussion

Discussion around Pi/2 BPSK and spectrum shaping has been really heated during the past RAN4 meetings as well as recent RAN plenary.

In our understanding, the controversial topics are:

- Whether Pi/2 BPSK is always using spectrum shaping

- Whether the feature is mandatory or optional for FR1 and FR2

The related features are named “pi/2-BPSK for PUSCH” and “pi/2-BPSK for PUCCH format 3/4" in NR UE feature list.

RAN4 recommendation from RAN4#86 is these features are optional for FR1 and TBD for FR2. RAN4 has agreed it will define the same minimum requirements for pulse-shaped pi/2 BPSK and non pulse-shaped pi/2 BPSK for FR2.

In RAN#79 there was again a lot of discussion around this topic, changing the name of the features to associate spectrum shaping with Pi/2 BPSK was discussed as well as making these features mandatory for both FR1 and FR2.

The following is an excerpt from RAN#79 summary [1]:

- For ‘Pi/2 BPSK’: the RAN4 decision of having this functionality optional for FR1 was not changed by RAN#79

•Pi/2 BPSK for FR2 is still open, and is expected to be further discussed in RAN4

The conclusion from RAN#79 was no changes were done to RAN4 recommendations.

The intrinsic feature of spectrum shaping is that one can get more power out of PA. This would be achieved using less MPR than in DFT-S-OFDM QPSK or even negative MPR. The benefits are obvious for FR2 where the coverage is often limited by TX power. For FR1 the benefits must be thought thoroughly, as today the 23dBm power class is easily met by QPSK without spectrum shaping which would require relaxations for FR1 spectrum flatness requirements.

FR1:

For FR1 most likely new power classes would need to be defined for all the bands because the 23dBm power class would likely be exceeded with spectrum shaped Pi/2 BPSK given that the PA is sized to meet 23dBm with DFT-S-OFDM QPSK. Alternatively, the PA could be beaten a bit less when pulse shaped Pi/2 BPSK is used still meeting 23dBm. Currently >23dBm power class has only been defined for a certain set of bands. Hence the use cases for Pi/2 BPSK with spectrum shaping being mandatory for FR1 would need to be thoroughly reasoned for all the bands including LTE re-farming bands, not only for one band or for a set of bands. Actually, we find it a bit weird that spectrum shaping would be a UE specific feature for FR1 and FR2. Especially for FR1 it would be easier to have that as band specific feature assuming it were needed for some FR1 bands. Please note that even in case spectrum shaping would be optional for FR1 and UE would support it, the feature would be there for all FR1 bands that UE supports even it could not use it for clear majority of the current bands. “Pi/2-BPSK for PUSCH” and “pi/2-BPSK for PUCCH format 3/4" features have been agreed to be UE specific with FR1/FR2 differentiation. It has not been agreed how or if spectrum shaping is going to be associated with Pi/2 BPSK. Our current understanding is that it is an UE implementation option whether to support spectrum shaping or not as long as the UE meets the minimum requirements.

The essential question is why to specify optional/mandatory spectrum shaping with Pi/2 BPSK for FR1 in UE specific manner if it cannot be used for most of the bands?

FR2:

WF for FR2 MPR looks as follows [2]

Note:

- UE requirements for the waveform defined by BW = 100MHz, SCS=60KHz, DFT-S-OFDM Pi/2 BPSK, 128RB0 shall be set to [-2 to 0]dB MPR

- Above WF MPR table is assumed that UE is provided with PA calibration gap when UE needs

It has been agreed that the waveform defined by BW = 100MHz, SCS=60KHz, DFT-S-OFDM QPSK, 128RB0 is the reference waveform with 0dB MPR and is used for the power class definition. WF suggest that the MPR for Pi/2 BPSK is 0…2 dB lower than that for QPSK DFT-S-OFDM. Please note that the WF does not distinguish between shaped or non-shaped Pi/2 BPSK as spectrum shaping has been considered as UE implementation option for pi/2 BPSK modulation.

Our view is that in order to make spectrum shaping beneficial, MPR gains should be visible in RAN4 requirements. We stress that we are totally fine with this feature for FR2, but there should be MPR defined which clearly is beneficial in terms of UE coverage.

It has been agreed in RAN4 that same requirements are defined for spectrum shaped and non spectrum-shaped Pi/2 BPSK. Based on how 3GPP usually works the MPR results will likely be defined according to the case which results in more generous MPR. If this is true, then the gains of spectrum shaping wouldn’t be visible.

How to achieve visible gains likely cannot be solved without reverting some agreements. Maybe the most straightforward option would be to define different requirement for Pi/2 BPSK using spectrum shaping and for Pi/2 BPSK not using spectrum shaping. Alternatively, UE vendors could provide MPR for Pi/2 BPSK always with spectrum shaping so the requirements could then be kind of agreed always assuming spectrum shaping for FR2. In first option there would be separate features for FR2 Pi/2 BPSK with and without spectrum shaping while in latter option it would be assumed spectrum shaping is used with Pi/2 BPSK.

# 3	Conclusions

Pi/2 BPSK with spectrum shaping was considered from practical operation perspective.

For FR1 the usage and benefits of spectrum shaping are still unclear as most of the bands, especially LTE re-farming bands, do not have >23dB power class defined. The essential question is why to specify optional/mandatory spectrum shaping with Pi/2 BPSK for FR1 in UE specific manner if it cannot be used for most of the bands? The current agreement on per UE is for pi/2 BPSK feature and it does not include the aspect of spectrum shaping or no spectrum shaping.

For FR2 spectrum shaping would be beneficial but there should be clearly visible gains in MPR specifications. We find it unlikely to see gains unless some agreements are reverted.

# 4	References

[1] RAN#79 Summary, RAN Chairman

[2] R4-1803263, “WF on MPR for FR2”, Intel, Qualcomm, RAN4#86