3GPP TSG-WG RAN4 Meeting #86bis	R4-1804941

# Melbourne, Australia 16th – 20th April, 2018

Source: 	Nokia, Nokia Shanghai Bell, [T-Mobile USA]

Title: 					TP to TR 37.865-01-01: BCS and Fallback groups for CA_n257

Agenda Item: 			7.1.3.7.3

Document for:	Approval

# 1 Introduction

This TP introduces fallbackgroups of intraband contiguous CA of band n257.

As we agreed to use band combination approrach for CA and DC [1], we propose to combine all fallback groups and their BCS included into clause 6.4, CA_257.

# 2 Reference

[1] R4-1802520 Introduction of CA/DC band combination-based basket approach, NTT DOCOMO.

**************************** Start of changes ********************************************

## 6.4	CA_n257

### 6.4.1	Operating bands for CA

## Table 6.4.1-1: Intra-band CA

| NR CA Band | NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n257 | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

### 6.4.2		Channel bandwidths per operating band for CA

### Table 6.4.2-1: Supported NR bandwidths per CA configuration for intra-band CA for 2CC

|  |  | E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configurations | Component carriers in order of increasing carrier frequency |  |  |  |  |  |  |  | Maximum aggregated BW (MHz) | BCS | Fallback group |  |
|  |  | CBW (MHz) | CBW (MHz) | CBW (MHz) | CBW (MHz) | CBW (MHz) | CBW (MHz) | CBW (MHz) | CBW (MHz) |  |  |  |
| CA_257B |  | 50, 100 , 200 ,400 | 400 |  |  |  |  |  |  | 800 | 0 | 1 |
| CA_257D |  | 50, 100, 200 | 200 |  |  |  |  |  |  | 400 | 0 | 2 |
| CA_257E |  | 50, 100, 200 | 200 | 200 |  |  |  |  |  | 600 | 0 | 2 |
| CA_257F |  | 50, 200 | 200 | 200 | 200 |  |  |  |  | 800 | 0 | 2 |
| CA_257G |  | 100 | 100 |  |  |  |  |  |  | 200 | 0 | 3 |
| CA_257H |  | 100 | 100 | 100 |  |  |  |  |  | 300 | 0 | 3 |
| CA_257I |  | 100 | 100 | 100 | 100 |  |  |  |  | 400 | 0 | 3 |
| CA_257J |  | 100 | 100 | 100 | 100 | 100 |  |  |  | 500 | 0 | 3 |
| CA_257K |  | 100 | 100 | 100 | 100 | 100 | 100 |  |  | 600 | 0 | 3 |
| CA_257L |  | 100 | 100 | 100 | 100 | 100 | 100 | 100 |  | 700 | 0 | 3 |
| CA_257M |  | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 800 | 0 | 3 |

### 6.4.3	Co-existence studies

### Table 6.4.3-1: Impact of UL and DL Harmonic Interference

|  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  | 6th Harmonic |  | 7th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL DL Low Band Edge | UL DL High Band Edge | UL DL Low Band Edge | UL DL High Band Edge | UL DL Low Band Edge | UL DL High Band Edge | UL DL Low Band Edge | UL DL High Band Edge | UL DL Low Band Edge | UL DL High Band Edge | UL DL Low Band Edge | UL DL High Band Edge | UL DL Low Band Edge | UL DL High Band Edge |
| n257 | 26500 | 29500 | 53000 | 59000 | 79500 | 88500 | 106000 | 118000 | 132500 | 147500 | 159000 | 177000 | 185500 | 206500 |

**************************** End of changes ********************************************