3GPP TSG-RAN WG4 Meeting #86bis	R4-1805345

# Melbourne, AU, 16th – 20th April, 2018

Source:	NTT DOCOMO, INC.

Title:	Indoor anechoic chamber test method procedure and MU for ACLR OTA

Agenda item:	6.26.3.1.4

Document for:	Approval

- Introduction

This contribution presents ACLR OTA test procedure and MU for an indoor anechoic chamber test method for conformance testing.

- ACLR conformance requirement

The ACLR requirement is the power ratio of the desired signal channel and the neighbouring channel bandwidth total powers that defined as the sum of the EIRP at a number of discrete directions around the sphere and the ACLR is defined as follows:

, where EIRPd is the filtered mean power within the desired signal channel bandwidth, and p1 and p2 denotes two orthogonal polarizations.

, where EIRPe is the filtered mean emission power in the neighbouring channel bandwidth, and p1 and p2 denotes two orthogonal polarizations.

To capture properly EIRP, EIRPd, p1 and EIRPe, p2 is associated to two orthogonal polarizations.

The number of discrete points required for conformance of the ACLR OTA being defined as the TRP requirement is subject to the TRP sampling grid discussion.

- Indoor anechoic chamber Test Method for ACLR OTA

The method measures the ACLR OTA in an anechoic chamber with the separation between the manufacturer declared coordinate system reference point of the AAS BS and the phase centre of the receiving antenna of no less than 2D2/λ, where D is the largest dimension of the antenna of AAS BS and λ is the wavelength. The measurement system setup is as depicted in figure 10.3.2.1-1.

The maximum size of the DUT is a chamber restriction that would affect the quality of the quiet zone. For larger DUT sizes larger size chambers should be considered such that the uncertainty of the quiet zone is taken into account.

The test consists of two stages: the calibration and the measurement. The test procedure is as follows.

# Figure 10.3.1.1.1-1: Indoor Anechoic Chamber measurement system setup for ACLR OTA

# Figure 10.3.1.1.1-2: Indoor Anechoic Chamber calibration system setup for ACLR OTA

# Stage 1 - Calibration: [Note: This stage may be omitted provided calibration stage has been performed already during output power measurement]

1)	Connect the reference antenna and the receiving antenna to the measurement RF out port and RF in port of the network analyzer, respectively, as shown in figure 10.3.1.1.1-2.

2)	Install the reference antenna with its beam peak direction and the height of its phase centre aligned with the receiving antenna.

3)	Set the centre frequency of the network analyzer to the carrier centre frequency of the tested signal for ACLR measurement of AAS BS and measure LFEIRP, E→D , which is equivalent to 20log|S21| (dB) obtained by the network analyzer:

-	LFEIRP, E→D: Pathloss between E and D in figure 10.3.1.1.1-2.

4)	Measure the cable loss, LFEIRP, E→F between the reference antenna connector and the network analyzer connector:

-	LFEIRP, E→F: Cable loss between E and F in figure 10.3.1.1.1-2.

5)	Calculate the calibration value between A and D with the following formula:

-	LEIRP_cal, A→D = LFEIRP, E→D  + GREF_ANT_EIRP, A→F -LFEIRP, E→F.

-	LEIRP_cal, A→D:  Calibration value between  A and D in figure 10.3.1.1.1-2.

-	GREF_ANT_EIRP, A→F: Antenna gain of the reference antenna.

# Stage 2 - Measurement:

6)	Uninstall the reference antenna and install the AAS BS with the manufacturer declared coordinate system reference point in the same place as the phase centre of the reference antenna. The manufacturer declared coordinate system orientation of the AAS BS is set to be aligned with the testing system.

7)	Set the AAS BS to generate the tested beam with the beam peak direction intended to be the same as the testing direction.

8)	Rotate the AAS BS to make the testing direction aligned with the direction of the receiving antenna.

9)	Set the AAS BS to transmit the test signal at the maximum power according to E-TM1.1.

10)	Measure the mean power within the desired signal channel bandwidth and the mean emission power in the neighbouring channel bandwidth for each carrier arriving at the measurement equipment connector, denoted by PR_AAS_desired, D, PR_AAS_emissiont, D

-	PR_AAS_desired, D: Measured mean power within the desired signal channel bandwidth for each carrier at the measurement equipment connector at D in figure 10.3.1.1.1-1.

-	PR_AAS_emission, D: Measured mean emission power in the neighbouring channel bandwidth for each carrier at the measurement equipment connector at D in figure 10.3.1.1.1-1.

11)	Calculate the EIRP with the following formula:

# EIRPd = PR_AAS_desired, D+ LEIRP_cal, A→D

# EIRPe = PR_AAS_emission, D+ LEIRP_cal, A→D

12)	Repeat the above steps 10)~11) at a number of discrete directions around the sphere.

13)	Calculate TRPs from EIRPd and EIRPe following equations

, where EIRPd is the filtered mean power within the desired signal channel bandwidth, and p1 and p2 denotes two orthogonal polarizations.

emission power in the neighbouring channel bandwidth, and p1 and p2 denotes two orthogonal polarizations.

14) Calculate ACLR.

- Indoor anechoic chamber Uncertainty Assessment for ACLR OTA

TRP requirement is defined as the sum of measured EIRP values around sphere. Therefore, the EIRP measurement uncertainties of the desired signal channel and the neighbouring channel bandwidth for an indoor anechoic chamber are the same as EIRP measurement for radiated transmit power in Rel-13 AAS specification.

# Table 1: Indoor Anechoic Chamber uncertainty assessment for EIRP measurement of OTA ACLR

| UID | Uncertainty source | Uncertainty value f ≦ 3GHz | Uncertainty value 3GHz < f ≦ 4.2 GHz | Distribution of the probability | Divisor based on distribution shape | ci | Standard uncertainty ui [dB] f ≦ 3GHz | Standard uncertainty ui [dB] 3GHz < f ≦ 4.2 GHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Stage 2: DUT measurement |  |  |  |  |  |  |  |  |
| 1 | Positioning misalignment between the AAS BS and the reference antenna | 0.03 | 0.03 | Rectangular | √3 | 1 | 0.02 | 0.02 |
| 2 | Pointing misalignment between the AAS BS and the receiving antenna | 0.3 | 0.3 | Rectangular | √3 | 1 | 0.17 | 0.17 |
| 3 | Quality of quiet zone | 0.10 | 0.10 | Gaussian | 1 | 1 | 0.10 | 0.10 |
| 4 | Polarization mismatch between the AAS BS and the receiving antenna | 0.01 | 0.01 | Rectangular | √3 | 1 | 0.01 | 0.01 |
| 5 | Mutual coupling between the AAS BS and the receiving antenna | 0.00 | 0.00 | Rectangular | √3 | 1 | 0.00 | 0.00 |
| 6 | Phase curvature | 0.05 | 0.05 | Gaussian | 1 |  | 0.05 | 0.05 |
| 7 | Uncertainty of the RF Power Measurement Equipment | 0.14 | 0.26 | Gaussian | 1 | 1 | 0.14 | 0.26 |
| 8 | Impedance mismatch in the receiving chain | 0.14 | 0.33 | U-shaped | √2 | 1 | 0.10 | 0.23 |
| 9 | Random uncertainty | 0.1 | 0.1 | Rectangular | √3 | 1 | 0.06 | 0.06 |
| Stage 1: Calibration measurement |  |  |  |  |  |  |  |  |
| 10 | Impedance mismatch between the receiving antenna and the network analyzer | 0.05 | 0.05 | U-shaped | √2 | 1 | 0.04 | 0.04 |
| 11 | Positioning and pointing misalignment between the reference antenna and the receiving antenna | 0.01 | 0.01 | Rectangular | √3 | 1 | 0.01 | 0.01 |
| 12 | Impedance mismatch between the reference antenna and the network analyzer. | 0.05 | 0.05 | U-shaped | √2 | 1 | 0.04 | 0.04 |
| 13 | Quality of quiet zone | 0.10 | 0.10 | Gaussian | 1 | 1 | 0.10 | 0.10 |
| 14 | Polarization mismatch for reference antenna | 0.01 | 0.01 | Rectangular | √3 | 1 | 0.01 | 0.01 |
| 15 | Mutual coupling between the reference antenna and the receiving antenna | 0.00 | 0.00 | Rectangular | √3 | 1 | 0.00 | 0.00 |
| 16 | Phase curvature | 0.05 | 0.05 | Gaussian | 1 | 1 | 0.05 | 0.05 |
| 17 | Uncertainty of the network analyzer | 0.13 | 0.20 | Gaussian | 1 | 1 | 0.13 | 0.20 |
| 18 | Influence of the reference antenna feed cable | 0.05 | 0.05 | Rectangular | √3 | 1 | 0.03 | 0.03 |
| 19 | Reference antenna feed cable loss measurement uncertainty | 0.06 | 0.06 | Gaussian | 1 | 1 | 0.06 | 0.06 |
| 20 | Influence of the receiving antenna feed cable | 0.05 | 0.05 | Rectangular | √3 | 1 | 0.03 | 0.03 |
| 21 | Uncertainty of the absolute gain of the reference antenna | 0.5 | 0.43 | Rectangular | √3 | 1 | 0.29 | 0.25 |
| 22 | Uncertainty of the absolute gain of the receiving antenna | 0.00 | 0.00 | Rectangular | √3 | 1 | 0.00 | 0.00 |
| Combined standard uncertainty (1σ) [dB] |  |  |  |  |  |  | 0.44 | 0.54 |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) [dB] |  |  |  |  |  |  | 0.87 | 1.06 |

- Conclusion

This contribution presents:

- OTA ACLR measurement description and MU values

- Further investigation of the dynamic range of ACLR measurement equipment may be necessary.