3GPP TSG-RAN WG4 #87	R4-1806157

Busan, Korea, 21 May-26 May 2018

Agenda Item:	7.12.2.4

Source:	Ericsson

Title:	Discussion on PRACH performance requirements

Document for:	Discussion

# Introduction

In RAN4#86bis, in [1], one high level WF is approved for BS demodulation. In this paper, we provide our view on some details setup for PRACH.

# Background

In RAN1, Four PRACH formats with long sequence are defined and nine PRACH formats with short sequence are defined. In Table 1, the PRACH formats with long sequence are given and the PRACH formats with short sequence are given in Table 2.

Table 1: PRACH preamble formats for  and .

| Format |  |  |  |  | Support for restricted sets |
| --- | --- | --- | --- | --- | --- |
| 0 | 839 | 1.25 kHz |  |  | Type A, Type B |
| 1 | 839 | 1.25 kHz |  |  | Type A, Type B |
| 2 | 839 | 1.25 kHz |  |  | Type A, Type B |
| 3 | 839 | 5 kHz |  |  | Type A, Type B |

Table 2: Preamble formats for  and  where .

| Format |  |  |  |  | Support for restricted sets |
| --- | --- | --- | --- | --- | --- |
| A1 | 139 |  |  |  | - |
| A2 | 139 |  |  |  | - |
| A3 | 139 |  |  |  | - |
| B1 | 139 |  |  |  | - |
| B2 | 139 |  |  |  | - |
| B3 | 139 |  |  |  | - |
| B4 | 139 |  |  |  | - |
| C0 | 139 |  |  |  | - |
| C2 | 139 |  |  |  |  |

For PRACH formats with short sequence, each format can have potential 4 different subcarrier spacing (SCS).

# Test cases down selections

As discussed in section 2, we have at least about 4+9*4=40 formats considering all possible combination of SCS and PRACH formats. If they are combined with the bandwidth, different channel model and different frequency offset and doppler, tremendous test cases will be introduced for PRACH formats. It is quite costly and not feasible to define test cases for all PRACH configuration. The group should discuss how to make down selection to reduce the test cases number.

RAN4 only define performance requirements for selected PRACH formats and SCS combinations

In RAN4, as one of the purposes of RAN4 performance requirements, we define performance requirements to verify BS can properly handle the corresponding physical channel. For PRACH, two types of PRACH structure are introduced, one is for PRACHs with long sequence, and one is for PRACHs with short sequence. For PRACH with long sequence, one PRACH symbol is across multiple OFDM data symbols. For PRACH with short sequence, it consists multiple short OFDM symbols. The structure is different from PRACH with long sequence and short sequence. It will lead to different hardware implementation. For the same PRACH structure, the only difference is the parameters configuration. It has not too much difference for the receiver itself. Therefore, as the first principle, one structure of PRACH is with one test or limited test cases and different structure of PRACH can have different test cases. Thus, for PRACH with long sequence, we just need to define one test case.

At most one test case is defined for PRACH with long sequence

In principle, for PRACH with short sequence, one test case for one selected PRACH format is enough as well. However, considering FR1 and FR2, different SCS may be used for PRACH transmission, thus, at least multiple SCSs shall be tested. To have more coverage with limited test cases, we can make some combination of SCS and PRACH formats. More specifically, we can introduce multiple test cases for PRACH with short sequence, but for a given SCS, only one PRACH format is selected for the test.

For PRACH with short sequences, multiple test cases can be introduced, but at most one test case is introduced for a given SCS

As a second principle, in Rel-15, we can just define performance requirements for the typical operation and down prioritize the configuration for the performance enhancement and optimization. For the PRACH with long sequence, we have four formats. For format 1, it is used for large cell. For Format 2, it is used for coverage enhancement and for format 3, it is used for high speed case. The usage scenarios are shown in Table 2.

Table 2: Usage scenarios for PRACH with long sequence

| Format |  |  |  |  | Usage scenarios |
| --- | --- | --- | --- | --- | --- |
| 0 | 839 | 1.25 kHz |  |  | LTE refarming |
| 1 | 839 | 1.25 kHz |  |  | Large cell, up to 100 km |
| 2 | 839 | 1.25 kHz |  |  | Coverage enhancement |
| 3 | 839 | 5 kHz |  |  | High speed case |

Obviously, Format 0 is much more essential for NR operation. For format 1, 2 and 3, it is purely used only for selected scenarios. Thus, if one is selected for the PRACH with long sequence, Format 0 is preferable.

As another example for PRACH with short sequence, in 38.211, we define format B2, B3, but B2 and B3 cannot be used independently, it can only be used as a package with A2 and B3. As shown in the Table 6.3.3.2-2 of 38.211 and recapped in Table 3, we only define index to A2/B2 and A3/B3, and there is no index to indicate B2 and B3 only. The package of A2/B2 and A3/B3 is only used to maximize the possible number of preambles within one slot. It is only used for the optimization scenarios. Thus, B2 and B3 or A2/B2 and A3/B3 shall not be selected as the test PRACH format. For Format C2, one issue is the overhead. The overhead of C2 is 38%. Thus, it is also used for some special scenarios.

Table 3: Partial random-access configurations for a package A2/B2 and A3/B3

| … | … | … | … | … | … | … | … | … |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 137 | A2/B2 | 2 | 1 | 2,6,9 | 0 | 1 | 3 | 4 |
| 138 | A2/B2 | 2 | 0 | 4 | 0 | 2 | 3 | 4 |
| 139 | A2/B2 | 1 | 0 | 4 | 0 | 1 | 3 | 4 |
| 140 | A2/B2 | 1 | 0 | 1,6 | 0 | 1 | 3 | 4 |
| 141 | A2/B2 | 1 | 0 | 4,9 | 0 | 1 | 3 | 4 |
| 142 | A2/B2 | 1 | 0 | 1 | 0 | 2 | 3 | 4 |
| 143 | A2/B2 | 1 | 0 | 7 | 0 | 2 | 3 | 4 |
| 144 | A2/B2 | 1 | 0 | 1,4,7 | 0 | 2 | 3 | 4 |
| 145 | A2/B2 | 1 | 0 | 0,2,4,6,8 | 0 | 2 | 3 | 4 |
| 146 | A2/B2 | 1 | 0 | 0,1,2,3,4,5,6,7,8,9 | 0 | 2 | 3 | 4 |
| … | … | … | … | … | … | … | … | … |
| 167 | A3/B3 | 2 | 1 | 2,6,9 | 0 | 2 | 2 | 6 |
| 168 | A3/B3 | 2 | 0 | 4 | 0 | 2 | 2 | 6 |
| 169 | A3/B3 | 1 | 0 | 4 | 0 | 1 | 2 | 6 |
| 170 | A3/B3 | 1 | 0 | 1,6 | 0 | 1 | 2 | 6 |
| 171 | A3/B3 | 1 | 0 | 4,9 | 0 | 1 | 2 | 6 |
| 172 | A3/B3 | 1 | 0 | 1 | 0 | 2 | 2 | 6 |
| 173 | A3/B3 | 1 | 0 | 7 | 0 | 2 | 2 | 6 |
| 174 | A3/B3 | 1 | 0 | 1,4,7 | 0 | 2 | 2 | 6 |
| 175 | A3/B3 | 1 | 0 | 0,2,4,6,8 | 0 | 2 | 2 | 6 |
| 176 | A3/B3 | 1 | 0 | 0,1,2,3,4,5,6,7,8,9 | 0 | 2 | 2 | 6 |

For A1~A3, A3 have 6 sequences and A1 only have 2 sequences, thus, A3 is more general than A1 from verifying receiver behaviour point of view. In other words, if gNB can have good performance for A3, it should have good performance for A1 too. The same principle can be applied for B1 and B4. Thus, A3 and B4 is a good candidate for the PRACH format.

A3 and B4 is a good candidate as the selected PRACH format with short sequence

For the SCS, according to the RAN4 RF discussion, 15 kHz and 30 KHz will be used for FR1 and 60kHz and 120kHz for FR2. Since 15kHz has been verified by format 0, thus, 30 KHz can be selected for FR1. For FR2, we can select 120 KHz. As one potential combination, we can combine 120 KHz with A3 and 30 KHz with B4.

# Test metric

In LTE, the test metric for PRACH requirements are the false alarm probability and the probability of detection. For the false alarm probability, the false alarm probability shall be less than or equal to 0.1% and the probability of detection shall be equal to or exceed 99%. In the definition of the probability of detection, as one error case of the several error cases –correct preamble detection but with the wrong timing estimation, this timing error requirements are correlated with the channel. For AWGN, a timing estimation error occurs if the estimation error of the timing of the strongest path is larger than 1.04us. For ETU70 and EPA1, a timing estimation error occurs if the estimation error of the timing of the strongest path is larger than 2.08us. The strongest path for the timing estimation error refers to the strongest path (i.e. average of the delay of all paths having the same highest gain = 310ns for ETU) in the power delay profile.

There are several issues need to be addressed before reusing this test metric. First, since the channel for NR is not decided yet, how to define the range need more study. For FR2, some of tradition channel model cannot be used directly, since the delay spread is too large relative to the CP length. Second, since OTA test is used for FR2, the propagation delay in the transmission over the air is with more uncertainty compared with conduct test. Thus, RAN4 need more study on the test metric.

RAN4 need more study on the test metric for PRACH at least regarding the timing error definition

# General setup

## Antenna configuration

As a general principle, antenna configuration for PRACH shall keep the same configuration as PUSCH, PUCCH. Further to simplify the maintenance of the performance and test case, we prefer to keep the same antenna configuration for FR1 and FR2. For FR2, only OTA test is feasible. For OTA test, due to the test complexity, we prefer only test 1 Tx and 2 RX.

## Channel model

In LTE, we have defined performance requirements for AWGN and fading channel with frequency offset. In NR, since the channel model need more discussion for both FR1 and FR2, Thus, we propose to define performance requirements for AWGN first and consider fading channel with frequency offset later.

## Restricted sets

In NR, for format 0~3, restricted sets are supported. For PRACH format with short sequence, restricted sets are not supported. From receiver point of view, whether supporting restricted set or not has not too much impact on the receiver behaviour. Thus, we prefer not to configure restricted set in the PRACH performance test.

## Bandwidth

The bandwidth shall be kept the same as PUSCH and PUCCH []. In the companion paper, the bandwidth is proposed.

## Logical sequence index

To generate the PRACH sequence, we need give the number of root sequences (u) required and the cyclic shift length (NCS).  The number of root sequences (u) required and the cyclic shift length (NCS) are dependent on the cell size. For small cell size, only one root sequence can be used to generate all preambles. In Rel-8, we use Ncs=13 for the simulation for format 0. We can reuse the same assumption for Format 0 in NR. For PRACH with short sequence, the cell size is scaled according to the SCS of PRACH. The Ncs value shall be decided based on the configured SCS. As a general principle, Ncs shall satisfy the following condition:

Ncs≥2r3*108-τdsNzcTseq+ng

where Tseq is the PRACH sequence length and duration (measured in s), Nzc is 139 for short sequence, and τds is the maximum delay spread, r is the cell size and ng is the number of additional guard samples due to the receiver pulse shaping filter. For example, when 30KHz is used and Ncs=6, the cell size is about 200 m.

# Conclusion

In this paper, we share our view on how to define PRACH performance requirements, we have the following observations:

Observation 1	A3 and B4 is a good candidate as the selected PRACH format with short sequence

Observation 2	RAN4 need more study on the test metric for PRACH at least regarding the timing error definition

Based on the discussion, we propose the following:

Proposal 1	RAN4 only define performance requirements for selected PRACH formats and SCS combinations

Proposal 2	At most one test case is defined for PRACH with long sequence

Proposal 3	For PRACH with short sequences, multiple test cases can be introduced, but at most one test case is introduced for a given SCS

# References

R4-1806015, Way forward on NR BS demodulation performance, Huawei, Nokia, Nokia Shanghai Bell, Ericsson, Samsung, March 2018.

R4-1806158, General Issues NR BS Demodulation Requirements, Ericsson, May 2018.

R4-080528, Joint proposal for change the PRACH simulation assumptions, Nokia Siemens Networks, Ericsson, NTT DoCoMo, Feb. 2008.