3GPP TSG-RAN WG4 #87	R4-1806160

Busan, Korea, 21 May-26 May 2018

Agenda Item:	6.22.6.1

Source:	Ericsson

Title:	Initial simulation results and discussion for sPUSCH

Document for:	discussion

# Introduction

In this contribution, initial simulation results for sPUSCH are proposed.

# Simulation assumption for sPUSCH

In Table 1 and Table 2, Simulation assumptions are given according to [1] and [2].

Table 1: Simulation assumption

| Parameter | Value |
| --- | --- |
| Number of Tx antennas | 1 |
| Number of Rx antennas | 2, 4 |
| Propagation condition | EPA5 |
| Cyclic prefix | Normal |
| Reference receiver | MRC |
| Channel bandwidth | [5 MHz], 10 MHz, 15 MHz, 20 MHz |
| TTI length | Subslot TTI |
| Number of OFDM-symbols | Subslot TTI: 2 and 3 |
| Fraction of maximum throughput | 70 % |
| Maximum number of HARQ transmissions | 4 |
| DMRS configuration | IFDMA is not configured |
| DMRS sharing pattern | Option 1: - RDD DD DD RD DD RDD Option 2: - RDD DD¦R RD DD¦R RD RDD |
| Number of HARQ processes | 16 |
| RV sequence | 0, 2, 3, 1, 0, 2, 3, 1 |
| Modulation | 16QAM |

Table 2: FRC parameters for performance requirements (16QAM 3/4)

| Reference channel | Ax-1 |  |  | Ax-2 |  | Ax-3 |  | Ax-4 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Allocated resource blocks | 24 |  |  | 48 |  | 72 |  | 100 |  |
| DFT-OFDM Symbols per subframe | 1 | 2 |  | 1 | 2 | 1 | 2 | 1 | 2 |
| Modulation | 16QAM |  |  | 16QAM |  | 16QAM |  | 16QAM |  |
| Code rate | 3/4 |  |  | 3/4 |  | 3/4 |  | 3/4 |  |
| Payload size (bits) | 872 |  | 1736 | 1736 | 3496 | 2536 | 5160 | 3624 | 7224 |
| Transport block CRC (bits) | 24 |  | 24 | 24 | 24 | 24 | 24 | 24 | 24 |
| Code block CRC size (bits) | 24 |  | 24 | 24 | 24 | 24 | 24 | 24 | 24 |
| Number of code blocks - C | 1 |  | 1 | 1 | 1 | 1 | 1 | 1 | 2 |
| Total number of bits per sub-frame | 1152 |  | 2304 | 2304 | 4608 | 3456 | 6912 | 4800 | 9600 |
| Total symbols per sub-frame | 288 |  | 576 | 576 | 1152 | 864 | 1728 | 1200 | 2400 |

# Simulation results

In section 3.1, simulation results for 2 Rx are given for different bandwidth. In section 3.2, simulation results for 4 Rx are given for different bandwidth. The target SNR without any impairment margin is tabulated in Table 3 and Table 4.

## Simulation results for 2 Rx

Figure 1: Simulation results for 5MHz with 2 Rx

Figure 2: Simulation results for 10MHz with 2 Rx

Figure 3: Simulation results for 15MHz with 2 Rx

Figure 4: Simulation results for 20MHz with 2 Rx

Table 3: Target SNR without any impairments margin (2RX)

| Bandwidth (MHz) | Target SNR @ 70% throughput without any margin |
| --- | --- |
| 5 | 8.8 |
| 10 | 8.8 |
| 15 | 8.8 |
| 20 | 8.9 |

## Simulation results for 4 Rx

Figure 5: Simulation results for 5MHz with 4 X

Figure 6: Simulation results for 10MHz with 4 RX

Figure 7: Simulation results for 15MHz with 4 RX

Figure 8: Simulation results for 20MHz with 4 RX

Table 4: Target SNR without any impairments margin (4RX)

| Bandwidth (MHz) | Target SNR (dB) @ 70% throughput without any margin |
| --- | --- |
| 5 | 6.1 |
| 10 | 6.1 |
| 15 | 5.9 |
| 20 | 6.6 |

# Discussion on the DMRS pattern for PUSCH

In last meeting, the only open issue is about the DMRS pattern. According to the agreements, we have two options on the table for the DMRS pattern for sPUSCH:

- Option 1:

  - With DMRS sharing RDD DD DD RD DD RDD

- Option 2:

  - With DMRS sharing: RDD DD|R RD DD|R RD RDD

For option 1, the DMRS overhead is smaller compared with option 2. However, option 2 is a good balance between overhead and performance. Further, since option 2 is the DMRS pattern used for SPS and SPS is quite useful for the sTTI usage scenarios. Thus, we slightly prefer option 2.

.

# Conclusion

In this contribution, initial simulation results are proposed for sPUSCH. We hope the group can consider these simulation results for the performance requirements discussion.

# Reference

R4-1803102, WF on BS demodulation for sTTI and PT, Ericsson, Huawei, Nokia

R4-1803171, Simulation assumption for sPUSCH, Ericsson