3GPP TSG-RAN WG4 Meeting #87		R4-1806258

Busan, South Korea, 21 – 25 May 2018

Agenda item:	7.5.10.4

Source:	Intel Corporation

Title:	ACS requirement for 2DL/1UL intra-band contiguous CA for FR1

Document for:	Discussion

# Introduction

In RAN4 it has been further discussed the Rx requirement for 2DL/1UL intra-band contiguous CA. It has been agreed that the jammer BW used for ACS/IBB should be the aggregated BW of the two component carriers, which complies with previous agreement regarding the interferer bandwidth for single carrier. However, during last meeting a concern was raised about the ACS level considering impact of the larger BW for the intra-band contiguous CA case.
In this contribution we provide our view for the ACS requirement.

# Discussion

In RAN4#86 meeting it was agreed that for the intra-band contiguous CA case the jammer BW is the same as the aggregated BW. However, the value for the ACS is still on brackets. In the next subsection will provide our view considering the higher SU and larger BW.

## ACS requirement intra-band contiguous CA requirement

In [1] simulation results were shown comparing LTE, NR-re-farming and NR bands simulations. The analysis provided the required suppression for NR bands considering the NR spectrum utilization of 97.2%. These results demonstrated that a relaxation for the ACS level for NR bands single carrier was required. The high spectral utilization considered in NR and the larger BW bring further challenges the implementation, which has an impact on the definition of the ACS level for NR.

| Table 1: Resulting Suppression for NR bands - SC LTE; LTE50; LTE100. RB; 250; 500. BW; 50 MHz; 100 MHz. Offset frequency interferer; 48.6 MHz; 98.3 MHz. Suppression; 31.14 dB; 27 dB. Filter Order; 20; 20. Sampling Frequency; 157500000; 315000000. |  |
| --- | --- |

Table 1 shows the resulting suppression required for NR for 100 MHz channel bandwidth. In the TS 38.101-1 the ACS for intra-band contiguous CA is defined from 110 MHz up to 200 MHz. The increase on the bandwidth compared to the single carrier case will impact the filter suppression even more, the digital channel filter will require higher order filter conflicting with the reduced latency in NR and the digital interface must require higher dynamic range from DSP channel filter up to the FFT block to handle increased interference.

The ACS level for both single carrier and intra-band contiguous case is 33 dB for all channel bandwidths. However, it can be observed that considering the NR spectrum utilization for 100 MHz that the average interferer suppression is 27 dB. For the intra-band contiguous CA the suppression of the filter will be worse when increasing the channel BW, which starts at 110 MHz and ends at 200 MHz. The consequence of these difference will challenge (typ. RB/BB) implementation. Therefore, the same ACS level for intra-band contiguous CA and ACS single carrier cannot be considered since the larger BW for CA cause additional challenge to the filter implementation

Observation 1:	 The same ACS level for intra-band contiguous CA and ACS single carrier cannot be considered since the larger BW for CA cause additional challenge to the filter implementation

Another concern highlighted in the WF [2] was that the ACI power level was raised, which could have consequences if this is higher than the power of IBB. This concern was discussed in more detail in [3]. The NR CA configurations and bandwidth combination sets defined for intra-band contiguous CA have an aggregated bandwidth from 110 MHz up to 200 MHz. The REFSENS values for n77, n78, n79 is -85.6/-85.7 dBm for 100 MHz. It can be assumed that for bandwidths from 110 MHz to 200 MHz the REFSENS values will be higher than -85.6/-85.7 dBm as in the case of 100 MHz. If we consider a 10*log( ) extrapolation, the REFSENS for 200 MHz would be around -82.6/-82.7 dBm.

The current assumption for the interferer power level for ACS is REFSENS + [45.5], thus the power level considering the aggregated bandwidth between 110 MHz and 200 MHz may vary between -39.5 and -37.1 dBm. This will make the interferer power level higher than the power level of the IBB which is currently defined as -56 dBm.

Observation 2: The interferer power level for ACS is higher than the IBB power level between aggregated bandwidth 110 MHz and 200 MHz.

Due to the higher power level of ACI than the power level of IBB, a modification for ACS and IBB power levels is required in order to have the ACS and IBB at least at the same level for the intra-band contiguous CA cases.

## Modification of ACS level for intra-band contiguous CA requirement

Based on both arguments explained in subsection 2.1, we propose a relaxation of 3 dB on the ACS requirements for the intra-band contiguous CA. Our proposal for the ACS level is given in Table below.

Proposal 1: A relaxation on the ACS requirements for the intra-band contiguous CA is required as shown in Table 2

Table 2: ACS for intra-band contiguous CA with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz

| RX parameter | Units | Channel bandwidth |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 110 MHz | 120 MHz | 130 MHz | 140 MHz | 150 MHz |
| ACS | dB | 30 | 30 | 30 | 30 | 30 |
| RX parameter | Units | Channel bandwidth |  |  |  |  |
|  |  | 160 MHz | 180 MHz | 200 MHz |  |  |
| ACS | dB | 30 | 30 | 30 |  |  |

# Conclusions

In this contribution we have discussed the ACS requirement for intra-band contiguous CA. In summary, we have made the following observation and proposal:

Observation 1:	 The same ACS level for intra-band contiguous CA and ACS single carrier cannot be considered since the larger BW for CA cause additional challenge to the filter implementation.

Observation 2: The interferer power level for ACS is higher than the IBB power level between aggregated bandwidth 110 MHz and 200 MHz.

Proposal 1: A relaxation on the ACS requirements for the intra-band contiguous CA is required as shown in Table 2

# References

- R4-1714159, “ACS requirement for FR1”, Intel Corporation

- R4-1803383, “WF on UE RF requirements for intra-band contiguous NR CA in FR1”, NTT DoCoMo

- R4-1804141, “ACS and IBB requirements for 2DL/1UL intra-band contiguous CA for FR1”, Intel Corporation