3GPP TSG-RAN WG4 Meeting #87		R4-1806259

Busan, South Korea, 21 – 25 May 2018

Agenda item:	7.5.10.4

Source:	Intel Corporation

Title:	IBB requirement for 2DL/1UL intra-band contiguous CA for FR1

Document for:	Approval

# Introduction

In RAN4 it has been further discussed the Rx requirement for 2DL/1UL intra-band contiguous CA. It has been agreed that the jammer BW used for ACS/IBB should be the aggregated BW of the two component carriers, which complies with previous agreement regarding the interferer bandwidth for single carrier. However, during last meeting a concern was raised about the ACS and IBB requirements considering impact of the larger BW for the intra-band contiguous CA case. In this contribution we provide our view for the IBB requirement.

# Discussion

In RAN4#86 meeting it was agreed that for the intra-band contiguous CA case the jammer BW is the same as the aggregated BW. Since NR support larger bandwidths than in LTE, the CA combinations for 2 components carrier cover a bandwidth of up to 200 MHz, which becomes an additional challenge for the implementation. In this contribution we provide our view for the in-band blocking requirement for 2DL/1UL intra-band contiguous CA.

## IBB requirement intra-band contiguous CA requirement

For the 2DL/1UL intra-band contiguous the requirement is defined from 110 MHz up to 200 MHz. The increase on the channel bandwidth required the adaptation of the filter to meet the requirement. The high spectral utilization considered in NR and the larger BW bring further challenges to the implementation. When the bandwidth increases, the selectivity of the filter becomes worse. As it can be seen in Figure 1, the steepness of the transfer function is a direct function of the filter order. Each of the curves represent a Butterworth filter with filter order “n”.

Figure 1: Butterworth Filter with different filter orders

The target is not to have a receive filter requirement more stringent in order to keep UE power consumption low. The increase on the bandwidth compared to the single carrier case will impact the filter suppression even more, the digital channel filter will require higher order filter conflicting with the reduced latency in NR and the digital interface must require higher dynamic range in order to handle increased interference [1]. The main idea is to adapt the filter bandwidth to the new NR carrier bandwidth while keeping the filter order. Therefore in order to compensate the decrease on the selectivity of the filter, the reference signal must increase as shown in Table 1.

Observation:	 The same IBB requirement parameters for intra-band contiguous CA and IBB  single carrier cannot be considered since the larger BW for CA cause additional challenge to the filter implementation

Based on this argument, we propose a relaxation of 3 dB on the IBB power transmission for the intra-band contiguous CA. Our proposal for the IBB parameter level is given in the table below and highlighted in yellow.

Table 1: In-band blocking parameters for intra-band contiguous CA with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz

| RX parameter | Units | Channel bandwidth |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 110 MHz | 120 MHz | 130 MHz | 140 MHz | 150 MHz |
| Power in transmission bandwidth configuration | dBm | REFSENS + channel specific value below |  |  |  |  |
|  | dB | 9 |  |  |  |  |
| BWinterferer | MHz | 10 | 120 | 130 | 140 | 150 |
| FIoffset, case 1 | MHz | 15 | 180 | 195 | 210 | 225 |
| FIoffset, case 2 | MHz | 25 | 300 | 325 | 350 | 375 |
| RX parameter | Units | Channel bandwidth |  |  |  |  |
|  |  | 160 MHz | 180 MHz | 200 MHz |  |  |
| Power in transmission bandwidth configuration | dBm | REFSENS + channel specific value below | REFSENS + channel specific value below | REFSENS + channel specific value below |  |  |
|  | dB | 6 | 6 | 6 |  |  |
| BWinterferer | MHz | 160 | 180 | 200 |  |  |
| FIoffset, case 1 | MHz | 240 | 270 | 300 |  |  |
| FIoffset, case 2 | MHz | 400 | 450 | 500 |  |  |
| NOTE 1: The transmitter shall be set to 4dB below […]. NOTE 2: The interferer consists of the RMC specified in […] |  |  |  |  |  |  |

Proposal: A relaxation on the IBB requirement for the intra-band contiguous CA is required as shown in Table 1

# Conclusions

In this contribution we have discussed the IBB requirement for intra-band contiguous CA. In summary, we have made the following observation and proposal:

Observation:	 The same IBB requirement parameters for intra-band contiguous CA and IBB  single carrier cannot be considered since the larger BW for CA cause additional challenge to the filter implementation

Proposal: A relaxation on the IBB requirement for the intra-band contiguous CA is required as shown in Table 1

# References

- R4-1714159, “ACS requirement for FR1”, Intel Corporation

- R4-1804141, “ACS and IBB requirements for 2DL/1UL intra-band contiguous CA for FR1”, Intel Corporation