3GPP TSG-RAN WG4#87 Meeting	R4-1806277

Busan, South Korea, 21 – 25 May 2018

Agenda item:	6.20.6

Title: 	Discussion on eFeMTC UE demodulation performance requirements

Source: 	Intel Corporation

Document for:	Discussion

# Introduction

From RAN4#86Bis Meeting, the discussion on eFeMTC UE demodulation/CSI performance requirements started in RAN4. In the scope of the Rel.15 eFeMTC WID [1], a number of enhancements were introduced to enable eFeMTC UEs with higher velocity, improved latency, improved power consumption, improved spectral efficiency and improved load control, compared to previous Rel.14 FeMTC and/or Rel.13 eMTC. A WF [2] was agreed in RAN4#86Bis Meeting, including one open issue that:

- FFS for applicability for non-BL UE supporting coverage enhancement

In this contribution, we share our view on the applicability for non-BL UE supporting coverage enhancement in Rel.15 eFeMTC scope.

# Discussion

Firstly, in our view, the applicability for non-BL UE supporting coverage enhancement is out of the scope of Rel.15 eFeMTC. It is because of two reasons:

- Based on the clear definition in TS36.133 about non-BL/CE UE as: “The requirements for non-BL/CE UE are derived assuming: DL and UL category other than Category 0/M1/M2/NB1/NB2, operation in any LTE system bandwidth but with a channel bandwidth not exceeding 20MHz, transmission bandwidth not exceeding 96RB in downlink and 5MHz in uplink, and dual antenna receiver, when in RRC_IDLE mode camped on a cell acquired using SIB1-BR, or in RRC_CONNECTED configured with CE mode A/B.” Whereas, in Rel.15 eFeMTC WID [1], it states that its objective is to specify improvements for BL/CE UEs. There is clear difference between non-BL/CE and BL/CE UE in RAN4, which we should comply with.

- The fact is that, non-BL/CE requirements are out of the scope of RAN4 RRM session for Rel.15 eFeMTC discussion. Conventionally, the position and consequence would be aligned within all RAN4 sessions.

Therefore, we propose that:

Proposal 1: In RAN4 eFeMTC WI, new demodulation/CSI feedback requirements will be introduced to BL/CE UE only.

Proposal 2: No new demodulation/CSI feedback requirements will be introduced to non-BL/CE UE in RAN4 eFeMTC WI.

# Conclusion

In this contribution, we share our view on the applicability for non-BL UE supporting coverage enhancement in Rel.15 eFeMTC scope.

Proposal 1: In RAN4 eFeMTC WI, new demodulation/CSI feedback requirements will be introduced to BL/CE UE only.

Proposal 2: No new demodulation/CSI feedback requirements will be introduced to non-BL/CE UE in RAN4 eFeMTC WI.

# Reference

[1] RP-172811, “Revised WID on Even further enhanced MTC for LTE”, Ericsson, Qualcomm, Dec. 2017

[2] R4-1805476, “Way forward on UE demodulation requirements for eFeMTC”, Huawei, HiSilicon, Ericsson, Intel, April 2018