3GPP TSG-RAN WG4 Meeting #87	R4-1806576

Busan, Korea, 21th – 25th May, 2018

Source:	BT plc

Title:	TP to TR 37.863-01-01: Introduction of new NR band n7 channel bandwidth options, for DC_3A_n7A

Agenda item:	7.2.3.1

Document for:	Approval

# 1. Introduction

This is a text proposal for 37.863-01-01, introducing two new ‘NR band n7’ channel bandwidth options for DC_3A_n7A.

It is proposed to include 35 MHz and 50 MHz to the existing channel bandwidth options.

# 2. Text Proposal for TR 37.863-01-01

##### ---Start of changes---

## 6.57	DC_3A_n7A

### 6.57.1	Operating bands for DC

Table 6.57.1-1: DC band combination of LTE 1DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3_n7 | 3 | 1710 MHz | – | 1785MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |

### 6.57.2	Channel bandwidths per operating band for DC

Table 6.57.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 35 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_3A_n7A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  | 70 |
|  | n7 | 15 | Yes | Yes | Yes | Yes | Yes |  | Yes |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes | Yes |  | Yes |  |  |  |  |
|  |  | 60 |  | Yes | Yes | Yes | Yes |  | Yes |  |  |  |  |

##### ---End of changes---