3GPP TSG- RAN WG4 Meeting #87 	R4-1806594

# Busan, Korea, 21 – 25 May 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.843 | CR | CRNum | rev | - | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to TR 37.843: AAS OTA REFSENS manufacturers declarations (9.2) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | AASenh_BS_LTE_UTRA-Perf |  |  |  |  |  |  | Date: |  | 2018-05-14 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The draft CR to TS 37.145-2 for Rel-15 AAS BS OTA REFSENS conformance testing have been endorsed in RAN4#86Bis, while the corresponding manufacturers declarations in TR 37.843 have yet to be filled in. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Fill in the manufacturers declarations for Rel-15 eAAS BS OTA REFSENS conformance testing in TR 37.843, based on the endorsed CR for Rel-15 eAAS BS OTA REFSENS conformance testing. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The manufacturers declarations for Rel-15 eAAS BS OTA REFSENS conformance testing in TR 37.843 remain missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<Start of change>

## 9.2	Manufacturers declarations for Rel-15 requirements

Manufacturers declaration introduced or modified in the eAAS WI are listed in table 9.2-1. Those declarations are built upon the already existing Rel-13 AAS BS declaration sets D9.1 – D9.29 and D10.1 – 10.11, as captured in TS 37.145-2 [24].

Table 9.2-1: Manufacturer declarations for Rel-15 eAAS radiated requirements

| Declaration identifier | Declaration | Description |
| --- | --- | --- |
| Dxx.1 | OTA REFSENS identifier | A unique identifier for the OTA REFSENS. |
| Dxx.2 | OTA REFSENS operating band support | Operating band supported by the OTA REFSENS declared for every OTA REFSENS identified in Dxx.1. NOTE 1: As each identified OTA REFSENS has a minimum EISREFSENS value, multiple operating bands can only be declared if they have the same minimum EISREFSENS. |
| Dxx.3 | OTA REFSENS RAT support | RAT(s) supported by the OTA REFSENS for each supported operating band, declared for every OTA REFSENS identified in Dxx.1. NOTE 2: If the OTA REFSENS supports multiple RAT's with different minimum EISREFSENS values and all other parameters are the same, then different EISREFSENS values for different RATs and signal BW's may be applied for an OTA REFSENS. |
| Dxx.4 | OTA REFSENS E-UTRA supported channel BW | The E-UTRA channel BW's supported by each OTA REFSENS. |
| Dxx.5 | OTA REFSENS Range of Angle of Arrival | The OTA REFSENS RoAoA associated with the receiver target reference direction (Dxx.6) for each OTA REFSENS. |
| Dxx.6 | OTA REFSENS receiver target reference direction | For each OTA REFSENS an associated direction inside OTA REFSENS RoAoA (Dxx.5). |
| Dxx.7 | OTA REFSENS Conformance test directions | For each OTA REFSENS four conformance test directions. 1) The direction determined by the maximum φ value achievable inside the OTA REFSENS RoAoA, while θ value being the closest possible to the receiver target reference direction. 2) The direction determined by the minimum φ value achievable inside the OTA REFSENS RoAoA, while θ value being the closest possible to the receiver target reference direction. 3) The direction determined by the maximum θ value achievable inside the OTA REFSENS RoAoA, while φ value being the closest possible to the receiver target reference direction. 4) The direction determined by the minimum θ value achievable inside the OTA REFSENS RoAoA, while φ value being the closest possible to the receiver target reference direction. |

{editors note: to be completed as part of conformance work}

<End of change>