3GPP TSG-RAN WG4 Meeting #87		R4-186780

Busan, Korea, 21st -25th  May, 2018

Source: 	ZTE Corporation

Title: 	TP to TS 38.141-1: Definitions, symbols and abbreviations(Sections 3)

Agenda Item:	7.8.3

Document for:	Approval

1	Introduction

In the last meeting, the contribution[1] to introduce CA/multi-carrier operation related terminology to TS38.104 was agreed, and a TP to TS 38.141-1 for section 1~3[2] was also agreed.

In this contribution, we provides a TP to Section 3 of TS 38.141-1[3] based on TS38.104[4]. The main modifications are:

- Introduction of CA/multi-carrier operation related terminologies

- Deleting some definitions, symbols and abbreviations related to radiated test requirements

- To align with TS38.104, correct the definition of “multi-band connector”, and delete the definition of “single-band antenna connector” and “single-band TAB connector”, which are replaced by the definition of ”single-band connector” .

For bullet 2, due to TS38.141-1 covers conducted test requirements, the definitions, symbols and abbreviations related to radiated test requirements will never be used, thus we do not think we need the these terminologies related to radiate requirements in 141-1 spec.

2	Reference

[1] R4-1805792, Draft CR to TS 38.104: Introduction of CA/Multi-carrier operation related terminology, ZTE Corporation

[2] R4-1805424, TP to TS 38.141-1 v0.1.0 Sections 1-3

[3] 38.141-1 v0.2.0

[4] R4-1804040, Draft CR for 38104: Combined updates(NSA) from RAN4 #86bis, Ericsson

--------------Start of TP  -------------

3	Definitions, symbols and abbreviations

3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Aggregated BS Channel Bandwidth:	 The RF bandwidth in which a Base Station transmits and receives multiple contiguously aggregated carriers. The aggregated BS channel bandwidth is measured in MHz.

antenna connector: connector at the conducted interface of the BS type 1-C

active transmitter unit: transmitter unit which is ON, and has the ability to send modulated data streams that are parallel and distinct to those sent from other transmitter units to a BS type 1-C antenna connector, or to one or more BS type 1-H TAB connectors at the transceiver array boundary

Base Station RF Bandwidth: RF bandwidth in which a base station transmits and/or receives single or multiple carrier(s) within a supported operating band

NOTE:	In single carrier operation, the Base Station RF Bandwidth is equal to the BS channel bandwidth.

Base Station RF Bandwidth edge: frequency of one of the edges of the Base Station RF Bandwidth.

basic limit: emissions limit relating to the power supplied by a single transmitter to a single antenna transmission line in ITU-R SM.329 [2] used for the formulation of unwanted emission requirements for FR1

BS channel bandwidth: RF bandwidth supporting a single NR RF carrier with the transmission bandwidth configured in the uplink or downlink

NOTE 1:	The BS channel bandwidth is measured in MHz and is used as a reference for transmitter and receiver RF requirements.

NOTE 2:	It is possible for the BS to transmit to and/or receive from one or more UE bandwidth parts that are smaller than or equal to the BS transmission bandwidth configuration, in any part of the BS transmission bandwidth configuration.

BS type 1-C:	NR base station operating at FR1 with requirements set consisting only of conducted requirements defined at individual antenna connectors

BS type 1-H:	NR base station operating at FR1 with a requirement set consisting of conducted requirements defined at individual TAB connectors and OTA requirements defined at RIB

Channel edge: lowest or highest frequency of the NR carrier, separated by the BS channel bandwidth.

Carrier aggregation: aggregation of two or more component carriers in order to support wider transmission bandwidths

Carrier aggregation configuration: a set of one or more operating bands across which the BS aggregates carriers with a specific set of technical requirements

co-location reference antenna: a passive antenna used as reference for base station to base station co-location requirements

Contiguous carriers: set of two or more carriers configured in a spectrum block where there are no RF requirements based on co-existence for un-coordinated operation within the spectrum block.

Contiguous spectrum: spectrum consisting of a contiguous block of spectrum with no sub-block gap(s).

Highest Carrier: The carrier with the highest carrier frequency transmitted/received in a specified frequency band.

Inter-band carrier aggregation: carrier aggregation of component carriers in different operating bands.

NOTE:	Carriers aggregated in each band can be contiguous or non-contiguous.

Intra-band contiguous carrier aggregation: contiguous carriers aggregated in the same operating band.

Intra-band non-contiguous carrier aggregation: non-contiguous carriers aggregated in the same operating band.

[Inter RF Bandwidth gap: frequency gap between two consecutive Base Station RF Bandwidths that are placed within two supported operating bands]

Lowest Carrier:	The carrier with the lowest carrier frequency transmitted/received in a specified frequency band.

Lower sub-block edge: frequency at the lower edge of one sub-block.

NOTE:	It is used as a frequency reference point for both transmitter and receiver requirements.

maximum carrier output power: mean power level measured per carrier at the indicted interface, during the transmitter ON period in a specified reference condition

maximum total output power: mean power level measured within the operating band at the indicated interface, during the transmitter ON period in a specified reference condition

measurement bandwidth: RF bandwidth in which an emission level is specified

multi-band connector: BS type 1-C Antenna Connector or BS type 1-H TAB connector associated with a transmitter or receiver that is characterized by the ability to process two or more carriers in common active RF components simultaneously, where at least one carrier is configured at a different operating band than the other carrier(s) and where this different operating band is not a sub-band or superseding-band of another supported operating band

Multi-carrier transmission configuration: set of one or more contiguous or non-contiguous carriers that a NR BS is able to transmit simultaneously according to the manufacturer’s specification.

Non-contiguous spectrum: spectrum consisting of two or more sub-blocks separated by sub-block gap(s).

operating band: frequency range in which NR operates (paired or unpaired), that is defined with a specific set of technical requirements

NOTE:	The operating band(s) for a BS is declared by the manufacturer according to the designations in tables 5.2-1 and 5.2-2.

Radio Bandwidth: frequency difference between the upper edge of the highest used carrier and the lower edge of the lowest used carrier

rated carrier output power: mean power level associated with a particular carrier the manufacturer has declared to be available at the indicated interface, during the transmitter ON period in a specified reference condition

rated total output power: mean power level associated with a particular operating band the manufacturer has declared to be available at the indicated interface, during the transmitter ON period in a specified reference condition

requirement set:	one of the NR base station requirement’s set as defined for BS type 1-C and  BS type 1-H in the present document.

single-band connector: BS type 1-C Antenna Connector or BS type 1-H TAB connector supporting operation either in a single operating band only, or in multiple operating bands but does not meet the conditions for a multi-band connector.

sub-block: one contiguous allocated block of spectrum for transmission and reception by the same base station

NOTE:	There may be multiple instances of sub-blocks within a Base Station RF Bandwidth.

Sub-block bandwidth: bandwidth of one sub-block.

sub-block gap: frequency gap between two consecutive sub-blocks within a Base Station RF Bandwidth, where the RF requirements in the gap are based on co-existence for un-coordinated operation

TAB connector: transceiver array boundary connector

TAB connector RX min cell group: operating band specific declared group of TAB connectors to which BS type 1-H conducted RX requirements are applied

NOTE:	Within this definition, the group corresponds to the group of TAB connectors which are responsible for receiving a cell when the BS type 1-H setting corresponding to the declared minimum number of cells with reception on all TAB connectors supporting an operating band, but its existence is not limited to that condition

TAB connector TX min cell group: operating band specific declared group of TAB connectors to which BS type 1-H conducted TX requirements are applied.

NOTE:	Within this definition, the group corresponds to the group of TAB connectors which are responsible for transmitting a cell when the BS type 1-H setting corresponding to the declared minimum number of cells with transmission on all TAB connectors supporting an operating band, but its existence is not limited to that condition

transceiver array boundary: conducted interface between the transceiver unit array and the composite antenna

transmitter OFF period: time period during which the BS transmitter is not allowed to transmit

transmitter ON period: time period during which the BS transmitter is transmitting data and/or reference symbols

transmitter transient period: time period during which the transmitter is changing from the OFF period to the ON period or vice versa

Upper sub-block edge: frequency at the upper edge of one sub-block.

NOTE:	It is used as a frequency reference point for both transmitter and receiver requirements.

3.2	Symbols

For the purposes of the present document, the following symbols apply:

	Percentage of the mean transmitted power emitted outside the occupied bandwidth on the assigned channel

BWChannel	BS channel bandwidth

BWChannel_CA 	Aggregated BS Channel Bandwidth, expressed in MHz. BWChannel_CA= Fedge_high- Fedge_low.

BWChannel,block 	Sub-block bandwidth, expressed in MHz. BWChannel,block= Fedge,block,high- Fedge,block,low.

BWConfig	Transmission bandwidth configuration, expressed in MHz, where BWConfig = NRB x SCS x 12 kHz

f 	Separation between the channel edge frequency and the nominal -3 dB point of the measuring filter closest to the carrier frequency

fmax 	f_offsetmax minus half of the bandwidth of the measuring filter

ΔFGlobal 	Global frequency raster granularity

ΔfOBUE 	Maximum offset of the operating band unwanted emissions mask from the downlink operating band edge

ΔfOOB	Maximum offset of the out-of-band boundary from the uplink operating band edge

ΔFRaster 	Channel raster granularity

ΔSUL	Channel raster offset for SUL

FC	RF reference frequency on the channel raster, given in table 5.4.2.2-1

FC,block, high	Fc of the highest transmitted/received carrier in a sub-block.

FC,block, low	Fc of the lowest transmitted/received carrier in a sub-block.

FC_low 	The Fc of the lowest carrier, expressed in MHz.

FC_high 	The Fc of the highest carrier, expressed in MHz.

Fedge_low 	The lower edge of Aggregated BS Channel Bandwidth, expressed in MHz. Fedge_low = FC_low - Foffset_low.

Fedge_high 	The upper edge of Aggregated BS Channel Bandwidth, expressed in MHz. Fedge_high = FC_high + Foffset_high.

Fedge,block,low 	The lower sub-block edge, where Fedge,block,low = FC,block,low - Foffset_low.

Fedge,block,high 	The upper sub-block edge, where Fedge,block,high = FC,block,high + Foffset_high.

Foffset_high 	Frequency offset from FC_high to the upper Base Station RF Bandwidth edge, or from F C,block, high to the upper sub-block edge

Foffset_low 	Frequency offset from FC_low to the lower Base Station RF Bandwidth edge, or from FC,block, low to the lower sub-block edge.

f_offset	Separation between the channel edge frequency and the centre of the measuring filter

f_offsetmax	The offset to the frequency ΔfOBUE outside the downlink operating band

FREF	RF reference frequency

FREF,SUL		RF reference frequency for Supplementary Uplink (SUL) bands

FDL_low	The lowest frequency of the downlink operating band

FDL_high	The highest frequency of the downlink operating band

FUL_low	The lowest frequency of the uplink operating band

FUL_high	The highest frequency of the uplink operating band

Ncells	The declared number corresponding to the minimum number of cells that can be transmitted by an BS type 1-H in a particular operating band

NRB	Transmission bandwidth configuration, expressed in resource blocks

NREF	NR Absolute Radio Frequency Channel Number (NR-ARFCN)

NRXU,active	The number of active receiver units. The same as the number of demodulation branches to which compliance is declared for chapter 8 performance requirements

NRXU,counted	The number of active receiver units that are taken into account for conducted Rx spurious emission scaling, as calculated in subclause 7. 6.1

NRXU,countedpercell	The number of active receiver units that are taken into account for conducted RX spurious emissions scaling per cell, as calculated in subclause 7.6..1

NTXU,counted	The number of active transmitter units as calculated in subclause 6.1, that are taken into account for conducted TX output power limit in subclause 6.2.1, and for unwanted TX emissions scaling

NTXU,countedpercell	The number of active transmitter units that are taken into account for conducted TX emissions scaling per cell, as calculated in subclause 6.1

Pmax,c,AC	Maximum carrier output power measured per antenna connector

Pmax,c,cell	The maximum carrier output power per TAB connector TX min cell group

Pmax,c,TABC	The maximum carrier output power per TAB connector

Prated,c,AC	The rated carrier output power per antenna connector

Prated,c,sys	The sum of Prated,c,TABC for all TAB connectors for a single carrier

Prated,c,TABC	The rated carrier output power per TAB connector

Prated,t,AC	The rated total output power declared at the antenna connector

Prated,t,TABC	The rated total output power declared at TAB connector

PREFSENS	Conducted Reference Sensitivity power level

SSREF	SS block reference frequency position

Wgap	Sub-block gap or Inter RF Bandwidth gap size

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

AAS	Active Antenna System

ACLR	Adjacent Channel Leakage Ratio

ACS	Adjacent Channel Selectivity

AWGN	Additive White Gaussian Noise

BS	Base Station

BW	Bandwidth

CA	Carrier Aggregation

CACLR	Cumulative ACLR

CW	Continuous Wave

E-UTRA	Evolved UTRA

EVM	Error Vector Magnitude

FDD	Frequency Division Duplex

FR	Frequency Range

GSCN	Global Synchronization Channel Number

GSM	Global System for Mobile communications

ITU-R	Radiocommunication Sector of the International Telecommunication Union

ICS	In-Channel Selectivity

LA	Local Area

LNA	Low Noise Amplifier

MR	Medium Range

NR	New Radio

NR-ARFCN	NR Absolute Radio Frequency Channel Number

OBUE	Operating Band Unwanted Emissions

RDN				Radio Distribution Network

REFSENS	Reference Sensitivity

RF	Radio Frequency

RMS	Root Mean Square (value)

RX	Receiver

SCS	Sub-Carrier Spacing

SDL	Supplementary Downlink

SUL	Supplementary Uplink

TAB	Transceiver Array Boundary

TAE	Time Alignment Error

TDD	Time division Duplex

TX	Transmitter

--------------End of TP ------------