3GPP TSG-RAN WG4 Meeting #87	R4-1806951

# Busan, KR, 21st – 25th May, 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.843 | CR | CRNum | rev | - | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR for TR37.843 -Indoor anechoic chamber test method procedure for OTA receiver intermodulation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | AASenh_BS_LTE_UTRA |  |  |  |  |  |  | Date: |  | 2018-05-14 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | For adding an indoor anechoic chamber test method on OTA receiver intermodulation requirement |  |  |  |  |  |  |  |  |
| Summary of change: |  | An anechoic chamber test method and MU for receiver intermodulation is described |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | An indoor anechoic chamber test method is not available for receiver intermodulation OTA test. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

10.3.6  OTA receiver intermodulation

10.3.6.1 General

10.3.6.2 Indoor anechoic chamber test method

10.3.6.2.1 General

This method measures the receiver intermodulation in an anechoic chamber with the separation between the manufacturer declared coordinate system reference point of the AAS BS and the phase centre of the receiving antenna of no less than 2D2/λ, where D is the largest dimension of the antenna of AAS BS and λ is the wavelength. The measurement system setup is as depicted in figure 10.3.6.2.1.

# Figure 10.3.6.2.1: Indoor Anechoic Chamber measurement system setup for  OTA receiver intermodulaiton

10.3.6.2.2 Calibration

Calibration shall be done with the procedure shown in 10.3.2.1.2 for the frequencies of wanted and unwanted signals with power combier/coupler. Calculate the calibration value for wanted and unwanted signal from A to D, from A to E, and from A to F.

LWanted_cal, A→D:  Calibration value for wanted signal between A and D in figure 10.3.6.2.1.

LUnanted_cal, A→E:  Calibration value for unwanted signal between A and E in figure 10.3.6.2.1.

LUnanted_cal, A→F:  Calibration value for unwanted signal between A and F in figure 10.3.6.2.1.

10.3.6.2.3 Procedure

1)	Uninstall the reference antenna and install the AAS with its manufacturer declared coordinate system reference point in the same place as the phase centre of the reference antenna. The manufacturer declared coordinate system orientation of the AAS is set to be aligned with testing system.

2)	Set the AAS BS to be satisfied with the RoAoA desired by the requirement covering conformance testing receiving direction.

3)	Rotate the AAS to make the conformance testing receiving direction aligned with the beam peak direction of the reference antenna at the calibration stage.

4)	For FDD AAS BS start BS transmission at the required condition.

5)	Set the test signal mean power at the RF signal source generator for wanted signal as the required level plus LWanted_cal, A→D  and the reference measurement channel.

6) Set the test signal mean power at the RF signal source generators for unwanted signal as the required level plus LUnwanted_cal, A→E  or plus LUnwanted_cal, A→F and the intereferece signal channel.

6)	Measure the throughput according to annex E of the 3GPP TS 36.141 [??].

7)	Repeat the above steps 1)~6) per conformance testing direction.

10.3.6.2.4 MU assesment

10.3.6.2.4.1 MU Budget

MU Budget for receiver intermodulation is the same as Table 10.3.5.2.4.1

10.3.6.2.4.2 MU Value

MU values of received singal level and calibration for OTA receiver intermodulaiton is the same as Table 10.3.5.2.4.2-1

The test system uncertainty is obtained by the following equation based on sub-clause 4.1.2.2 of 3GPP TS 36.141[22]:

Test system standard uncertainty=wanted_signal_error2+unwanted_signal1_error2+unwanted_signal2_error2+calibration_mesurement_error2
+ACLR effect

Here, ACLR effect for ACS, narrow-band blocking, in-band blocking is 0.4 as shown in sub-clause 4.1.1.2 of 3GPP TS 36.141[22].

The wanted_signal_level_error, unwanted_signal1_error , unwanted_signal2_error, and calibration_measuremnt_error are standard uncertainty of the received signal level by AAS BS for OTA as shown in Table10.3.5.2.4.2-1.

Total test system MU for OTA receiver intermodulation is shown in Table 10.3.6.2.4.2-2

Table 10.3.6.2.4.2-2 MU Value of receiver intermodulation

.

| OTA Requirements | Receiver intermodulation |  |
| --- | --- | --- |
| Frequency range | f ≦ 3 GHz | 3 GHz < f ≦ 4.2 GHz |
| Test system standard uncertainty (1) | 1.36 | 1.39 |
| Test system expand standard uncertainty (1.96) | 2.67 | 2.72 |