3GPP TSG-RAN WG4 Meeting #87															R4-1806980

Busan, Korea (Republic of), 21 - 25 May 2018

Source: 	Nokia

Title: 					MSD for DC_(n)71B

Agenda Item: 			7.2.3.10.2

Document for:	Discussion

# 1	Introduction

This contribution presents simulation results for MSD of DC_(n)71B.

# 2	Discussion

MSD is presented as a function of output power back off. Once the A-MPR is settled then correct MSD can be derived for the table.

Table 1:MSD test points

| E-UTRA and NR Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |
| --- | --- | --- | --- | --- |
| DC configuration | DC_71b-n71b |  |  |  |
| Access | OFDMA |  | DFT-S-OFDMA |  |
| EUTRA/NR band | 71 | n71 | 71 | n71 |
| UL fc (MHz) | 668 | 678 | 668 | 678 |
| UL/DL BW (MHz) | 10 | 10 | 10 | 10 |
| UL LCRB (RB) | 10 (RBstart=0) | 10 (RBend=51) | 10 (RBstart=0) | 10 (RBend=51) |
| DL fc (MHz) | 622 | 632 | 622 | 632 |
| Duplex mode | FDD |  | FDD |  |

Table 2:MSD as a function of output power back off

| MPR (dB) | MSD (dB) |  | MSD (dB) |  |
| --- | --- | --- | --- | --- |
| 0.0 | 24.0 | 35.2 | 24.3 | 34.5 |
| 0.5 | 23.1 | 34.4 | 23.5 | 34.6 |
| 1.0 | 21.9 | 33.8 | 22.3 | 34.8 |
| 1.5 | 20.5 | 33.2 | 21.1 | 34.7 |
| 2.0 | 19.1 | 32.5 | 19.5 | 34.3 |
| 2.5 | 17.5 | 31.8 | 18.0 | 33.8 |
| 3.0 | 16.1 | 30.9 | 16.4 | 33.0 |
| 3.5 | 14.5 | 29.9 | 14.9 | 31.9 |
| 4.0 | 13.0 | 28.9 | 13.3 | 30.7 |
| 4.5 | 11.6 | 27.7 | 11.8 | 29.4 |
| 5.0 | 10.1 | 26.6 | 10.2 | 28.0 |
| 5.5 | 8.8 | 25.3 | 8.6 | 26.5 |
| 6.0 | 7.5 | 23.9 | 7.3 | 24.9 |
| 6.5 | 6.3 | 22.5 | 6.2 | 23.3 |
| 7.0 | 5.2 | 21.1 | 5.2 | 21.7 |
| 7.5 | 4.3 | 19.6 | 4.5 | 20.1 |
| 8.0 | 3.5 | 18.0 | 3.9 | 18.4 |

# 3	Conclusion

In this contribution we have presented simulation results for MSD of DC_(n)71B.