3GPP TSG-RAN WG4 Meeting #87													R4-1806981

Busan, Korea (Republic of), 21 - 25 May 2018

Source: 	Nokia

Title: 						TP for 37.863-01-01: DC_(n)71B

Agenda Item: 			9.2.2.5

Document for:	Approval

# 1	Introduction

This contribution completes the information of DC_(n)71B in technical report.

*************************Start of the TP*******************************************

## 6.69	DC_(n)71B

### 6.69.1	Operating bands for DC

Table 6.69.1-1: DC band combination of LTE 1DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band | Downlink (DL) band | Duplex mode |
| --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit | BS transmit / UE receive |  |
|  |  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| DC_(n)71B | 71 | 663 MHz – 698 MHz | 617 MHz – 652 MHz | FDD |
|  | n71 |  |  |  |

### 6.69.2	Channel bandwidths per operating band for DC

Table 6.69.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + one NR band

| EN-DC configuration / Bandwidth combination set |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Downlink EN-DC configuration | Uplink EN-DC configuration | Component carriers in order of increasing carrier frequency |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
|  |  | Channel bandwidths for LTE carrier [MHz] | Channel bandwidths for NR carrier [MHz] |  |  |
| DC_(n)71B | DC_(n)71B | 15 | 5 | 20 | 0 |
|  |  | 10 | 5, 10 |  |  |
|  |  | 5 | 5, 10, 15 |  |  |

### 6.69.3	Co-existence studies

DC_(n)71B is a contiguous allocation thus only uplink harmonic analysis is needed.

Table 6.69.3-1: DC_(n)71B UL harmonics

| Uplink band |  |  | 2nd Harmonic |  | 3rd Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| 71 | 663 | 698 | 1326 | 1396 | 1989 | 2094 |

From Table 6.69.3-1 it can be seen that uplink harmonics 3rd harmonic overlaps with bands 3, 25, 34 and 70.

### 6.69.4	∆TIB and ∆RIB values

No dTib or dRib is required for DC_(n)71B as it is intraband DC.

### 6.69.5	MSD

Table 6.69.5-1: MSD for the DC_(n)71B configuration

| MSD / DC bandwidth class B |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DC configuration | E-UTRA/NR band | FC (UL) (MHz) | Channel bandwidth (MHz) | UL allocation [LCRB] | FC (DL) (MHz) | MSD (dB) | Duplex mode |
| DC_(n)71B | 71 | 665.5 | 5 | 5 (RBend =24) | 619.5 | 0 | FDD |
|  | n71 | 675.5 | 15 | 15 (RBstart = 0) | 629.5 | 1.8 |  |
|  | 71 | 670.5 | 15 | 15 (RBend = 74) | 624.5 | 0 |  |
|  | n71 | 680.5 | 5 | 5 (RBstart = 0) | 634.5 | 1.6 |  |
|  | 71 | 668 | 10 | 10 (RBend = 49) | 622 | 0 |  |
|  | n71 | 678 | 10 | 10 (RBstart = 0) | 632 | 1.7 |  |
|  | 71 | 668 | 10 | 10 (RBstart = 0) | 622 | [24.3] |  |
|  | n71 | 678 | 10 | 10 (RBend = 51) | 632 | [35.2] |  |

### 6.69.6	SEM for DC_(n)71B

FCC has defined spectrum emission maks that is relevant for operation on band 71 frequency range. SEM is presented in Table 6.69.6-1.

6.69.6-1: SEM for band 71 frequency range

| ΔfOOB (MHz) | Frequency offset of measurement filter centre frequency, f_offset | Minimum requirement [dBm] | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 0.1 MHz | 0.015 MHz f_offset < 0.085 MHz | -13 | 30 kHz |
| 0.1 MHz f < ENBW | 0.15 MHz f_offset < ENBW – 0.05 MHz | -13 | 100 kHz |
| ENBW f < ENBW + 5 MHz | ENBW+0.5 MHz f_offset < ENBW + 4.5 MHz | -25 | 1 MHz |
| NOTE 1: ENBW is the aggregated bandwidth of an E-UTRA sub-block and an adjacent NR sub-block; there is no frequency separation between the said sub-blocks. The sub-block bandwidths include any internal guard bands. |  |  |  |

| - |  |  |  |
| --- | --- | --- | --- |
| - |  |  |  |

### 6.69.7	A-MPR

Figure 6.69.7-1 shows required back-off as function of allocation ratio. Figure contains data for all included bandwidth combinations, SCSs in the worst-case position (upper band edge). From this data we have derived a unified A-MPR definition for the whole band n71.

Figure 6.69.7-1: Back-off vs. allocation ratio for entire band n71 (all bandwidth combinations and SCSs)

When LTE and NR know each other’s allocation, the A-MPR(1) is defined in terms of the total transmission powers as

AMPRDC=Ppowerclass-PTX=CEILMA,DC(A), 0.5

where

MA,DC(A)=5.50  - 5.71 A;0.00 < A <= 0.353.50;                     0.35 < A <= 1.00

The allocation ratio is

A=LCRB,LTE+LCRB,NRNRB,LTE+NRB,NR

When LTE and NR do not know each other’s allocation, The A-MPR(2) is defined carrier wise as

AMPRLTE=Ppowerclass-PLTE=CEIL{MA,LTE, 0.5}

AMPRNR=Ppowerclass-PNR=CEIL{MA,NR, 0.5}

where

MA,LTE=MA,DC(ALTE,wc)-LTE

MA,NR=MA,DC(ANR,wc)-NR

Above, the worst-case allocation ratios for each carrier are as follows where LTE modems assumes that NR allocation is 1 as it will lead to maximum needed A-MPR and vice versa.

ALTE,wc=LCRB,LTE+1NRB,LTE+NRB,NR

ANR,wc=1+LCRB,NRNRB,LTE+NRB,NR

and the worst-case division of power between carriers is represented by

∆LTE=10 log10LCRB,LTELCRB,LTE+NRB,NR

and

∆NR=10 log10LCRB,NRNRB,LTE+LCRB,NR

Table 6.69.7-1. Symbols for DC_(n)71B A-MPR

| PTX | Total transmission power (LTE+NR) |
| --- | --- |
| PLTE | Transmission power of the LTE channel |
| PNR | Transmission power of the NR channel |
| LCRB,LTE | Allocation size (in RB) in LTE channel |
| LCRB,NR | Allocation size (in RB) in NR channel |
| NRB,LTE | Transmission bandwidth configuration in LTE channel |
| NRB,NR | Transmission bandwidth configuration in NR channel for its configured SCS |
| NRB,NR | Transmission bandwidth configuration in NR channel for the SCS assumed by the LTE branch (minimum SCS valid for the NR channel bandwidth) |