3GPP TSG-RAN WG4 Meeting #87															R4-1807061

Busan, Korea (Republic of), 21 - 25 May 2018

Source: 	Nokia

Title: 					TP to TR37.863-02-01: DC_2A-5A_n260A

Agenda Item: 			7.2.3.2.3

Document for:	Approval

# 1	Introduction

This contribution introduces DC_2A-5A_n260A into technical report.

**************************Start of TP ********************************************

## 6.x	DC_2A-5A-n260A

### 6.x.1	Operating bands for DC

Table 6.x.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_2A-5A_n260A | 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | 5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | n260 | 37000 MHz | – | 40000 MHz | 37000 MHz | – | 40000 MHz | TDD |

### 6.x.2	Channel bandwidths per operating band for DC

Table 6.x.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | [40 MHz] | 50 MHz | [60 MHz] | 80 MHz | 100 MHz | [200 MHz] | 400 MHz | Maximum aggregated bandwidth [MHz] |  |
| DC_2A-5A_n260A | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  | n260 | 60 |  |  |  |  |  | Yes |  |  | Yes | Yes |  | 230 |  |
|  |  | 120 |  |  |  |  |  | Yes |  |  | Yes | Yes | Yes | 430 |  |

### 6.x.3	Co-existence studies

Table 6.x.3-1 lists Band 2A-5A +Band n260A 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.x.3-1: Band 2 and Band n260 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1850 | 1910 | 37000 | 40000 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3700 | 3820 | 74000 | 80000 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5550 | 5730 | 111000 | 120000 |
| 2nd order IMD products | ¦fy_high – fx_low¦ | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 38150 | 35090 | 38850 | 41910 |
| Two-tone 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 36300 | 33180 | 72090 | 78150 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 40700 | 43820 | 75850 | 81910 |
| Two-tone 3rd order IMD products | fx_low – Max BW-fy | fx_high + Max BW-fy | fy_low – Max BW-fx | fy_high + Max BW-fx |
| IMD frequency limits (MHz) | 1450 | 2310 | 36980 | 40020 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 76300 | 70180 | 77700 | 83820 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 34450 | 31270 | 109090 | 118150 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 42550 | 45730 | 112850 | 121910 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 158150 | 146090 | 29360 | 32600 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 149850 | 161910 | 44400 | 47640 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 116300 | 107180 | 68270 | 74450 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 114700 | 123820 | 79550 | 85730 |

Based on Table 6.x.3-1

- 2nd order harmonics may fall into LTE frequencies of Band 43

- 3rd order harmonics may fall into LTE frequencies of Band 46

- 2nd order IMD products may fall into own Rx frequencies of band n260

- 3rd order IMD products may fall into Rx frequencies of Band 1,  3, 4, 9, 10, 11, 21, 23, 24, 25, 32, 33, 34, 35, 36, 37, 39, 40, 45, 50, 65, 66, 70, 74, 75 and band n257

- 3rd order IMD products may fall into own Rx frequencies of Band 2 and n260

- 5th order IMD products may fall into Rx frequencies of band n257

Table 6.x.3-2: Band 5 and Band n260 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1850 | 1910 | 824 | 849 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3700 | 3820 | 1648 | 1698 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5550 | 5730 | 2472 | 2547 |
| Two tone 2nd order IMD products | ¦fy_low - fx_high¦ | ¦fy_high - fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1086 | 1001 | 2674 | 2759 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2851 | 2996 | 262 | 152 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4524 | 4669 | 3498 | 3608 |
| Two-tone 4th order IMD products | ¦3*fx_low - fy_high¦ | ¦3*fx_high - fy_low¦ | ¦3*fy_low - fx_high¦ | ¦3*fy_high - fx_low¦ |
| IMD frequency limits (MHz) | 4701 | 4906 | 562 | 697 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6374 | 6579 | 4322 | 4457 |
| Two-tone 4th order IMD products | ¦2*fx_low - 2*fy_high¦ | ¦2*fx_high - 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 2002 | 2172 | 5348 | 5518 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 1546 | 1386 | 6816 | 6551 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 5146 | 5306 | 8224 | 8489 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 1153 | 1348 | 4082 | 3852 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6172 | 6367 | 7198 | 7428 |

Based on Table 6.x.3-2

- 3rd order harmonic may fall into frequencies of band n41

- 2nd order IMD products may fall into own Rx frequencies of band n260

- 3rd order IMD products may fall into own Rx frequencies of band n260

- 3rd order IMD products may fall into frequencies of band 6, 8, 12, 13, 14, 17, 18, 19, 20, 26, 27, 28, 29, 31, 44, 67, 68, 71, and 72

- 4th order IMD products may fall into own Rx frequencies of band n260

When 2UL inter-band DC UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.x.3-3 and Table 6.x.3-4 list if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.x.3-3: 2UL Band 2 + Band n260 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | YES |  | IMD3 |
| Galileo | 1559 | - | 1591 | YES |  | IMD3 |
| GLONASS | 1591 | - | 1610 | YES |  | IMD3 |
| GPS | 1563 | - | 1587 | YES |  | IMD3 |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | No | US/Europe |  |
|  | 2400 | - | 2494 | No | Asia |  |
| ISM band (5GHz) | 5150 | - | 5925 | YES | US | 3rd order harmonic |
|  | 5150 | - | 5350 | No | Europe |  |
|  | 5470 | - | 5725 | YES |  | 3rd order harmonic |
|  | 5150 | - | 5825 | YES | Asia | 3rd order harmonic |
| 45GHz Unlicensed Bands | 42300 | - | 47000 | YES | China | 3rd order harmonic |
|  | 47200 | - | 48400 | YES | China | 5th IMD |
| 60GHz Unlicensed Bands | 57000 | - | 66000 | No | Europe |  |
|  | 57050 | - | 64000 | No | USA - Canada |  |
|  | 57000 | - | 64000 | No | South Korea |  |
|  | 59000 | - | 66000 | No | Japan |  |
|  | 59000 | - | 64000 | No | China |  |
|  | 59400 | - | 62900 | No | Australia |  |

Table 6.x.3-4: 2UL Band 5 + Band n260 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | No |  |  |
| Galileo | 1559 | - | 1591 | No |  |  |
| GLONASS | 1591 | - | 1610 | No |  |  |
| GPS | 1563 | - | 1587 | No |  |  |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | YES | US/Europe | 3rd order harmonic |
|  | 2400 | - | 2494 | YES | Asia | 3rd order harmonic |
| ISM band (5GHz) | 5150 | - | 5925 | No | US |  |
|  | 5150 | - | 5350 | No | Europe |  |
|  | 5470 | - | 5725 | No |  |  |
|  | 5150 | - | 5825 | No | Asia |  |

Table 6.x.3-5 and Table 6.x.3-6 list the protected bands required for the dual connectivity configuration.

Table 6.x.3-5: Protected bands for the dual connectivity configuration

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_2A-n260A | Band 4, 5, 10, 12, 13, 14, 17, 24, 26, 27, 28, 29, 30, 41, 42, 48, 50, 51, 66, 70, 71, 74, 85, n77, n78 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Band 2, 25 | FDL_low | - | FDL_high | -50 | 1 | x |
|  | Band 43 | FDL_low | - | FDL_high | -50 | 1 | 1* |
| * Table 6.5.3.2-1 of TS38.101 Note x: NOTE 15 in Table 6.6.3.2-1 of 36.101 will be applied here. Additional requirements for the frequency ranges that are less than FOOB (MHz) in Table 6.x.3.1-1 need further study. |  |  |  |  |  |  |  |

Table 6.x.3-6: Protected bands for the dual connectivity configuration

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_5A_n260A | Band 1, 2, 3, 4, 5, 7, 8, 10, 12, 13, 14, 17, 24, 25, 28, 29, 30, 31, 34, 38, 40, 42, 43, 45, 48, 50, 51, 65, 66, 70, 71, 74, n77, n78 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Band 26 | FDL_low | - | FDL_high | -27 | 1 |  |
|  | Band 41 | FDL_low | - | FDL_high | -50 | 1 | 1* |
| * Table 6.5.3.2-1 of TS38.101 |  |  |  |  |  |  |  |

### 6.x.4	∆TIB and ∆RIB values

For DC_2A-5A_n260A, the TIB,c and RIB values are given in the tables below.

Table 6.x.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2A-5A_n260A | 2 | 0.3 |
|  | 5 | 0.3 |
|  | n260 | 0 |

Table 6.x.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2A-5A_n260A | 2 | 0 |
|  | 5 | 0 |
|  | n260 | 0 |

### 6.x.5	MSD

No IMD issues are expected for DC configuration between the LTE bands below 6GHz and FR2 NR band. Furthermore, up to 7th order harmonics have no impact to Rx frequency of both LTE and NR bands.

**************************End of TP ********************************************