3GPP TSG-WG4 RAN#87 	R4-1807407

# Busan, South Korea, 21-25 May 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | CRNum | rev | - | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to TS 38.101-1: Inclusion of Simultaneous RxTx UE capability for some band combinations |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | RAN4 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Ericsson, Vodafone, Orange, [ … ] |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-05-14 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In RAN4#86bis, agreement was made that, RAN4 will consider on band combination basis on whether to mandate simultaneous RxTx UE capability for NR CA combnations. In this CR, band combinations for which simultaneous RxTx is mandatory are indicated. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updates in Section 5.2A and Section 5.2B based on latest version of 38.101-1 as approved online in R4-1806026. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Simultaneous RxTx UE capability will not supported for the band combinations which are needed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Section 5.2A, Section 5.2B |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  |  | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

## 5.2A	Operating bands for CA

### 5.2A.1	Intra-band CA

NR intra-band contiguous carrier aggregation is designed to operate in the operating bands defined in Table 5.2A.1-1, where all operating bands are within FR1.

Table 5.2A.1-1: Intra-band contiguous CA operating bands in FR1

| NR CA Band | NR Band (Table 5.2-1) |
| --- | --- |
| CA_n77 | n77 |
| CA_n78 | n78 |
| CA_n79 | n79 |

### 5.2A.2	Inter-band CA

NR inter-band carrier aggregation is designed to operate in the operating bands defined in Table 5.2A.2-1, where all operating bands are within FR1.

Table 5.2A.2-1: Inter-band CA operating bands involving FR1 (two bands)

| NR CA Band | NR Band (Table 5.2-1) |
| --- | --- |
| CA_n3A-n78A | n3, n78 |
| CA_n28A-n78A1 | n28, n78 |
| CA_n41A-n78A | n41, n78 |
| CA_n77A-n79A | n77, n79 |
| CA_n78A-n79A | n78, n79 |
| CA_n75A-n78A1 | n75, n78 |
| CA_n76A-n78A1 | n76, n78 |
| NOTE 1: Applicable for UE supporting inter-band carrier aggregation with mandatory simultaneous Rx/Tx capability. |  |

## 5.2B	Operating bands for DC

### 5.2B.1	General

NR dual connectivity is designed to operate in the operating bands defined in Table 5.2B-1, where all operating bands are within FR1.

Table 5.2B-1: Inter-band DC operating bands involving FR1 (two bands)

| NR DC Band | NR Band (Table 5.2-1) |
| --- | --- |
| NOTE 1: Applicable for UE supporting inter-band dual connectivity with mandatory simultaneous Rx/Tx capability. |  |

## 5.2C	Operating band combination for SUL

NR operation is designed to operate in the operating band combination defined in Table 5.2C-1, where all operating bands are within FR1.

Table 5.2C-1: Operating band combination for SUL in FR1

| NR Band combination for SUL | NR Band (Table 5.2-1) |
| --- | --- |
| SUL_n78-n802 | n78, n80 |
| SUL_n78-n812 |  |  | n78, n81 |  |
| SUL_n78-n822 | n78, n82 |
| SUL_n78-n832 |  |  | n78, n83 |  |
| SUL_n78-n84 | n78, n84 |
| SUL_n78-n86 |  |  | n78, n86 |  |
| SUL_n79-n802 |  |  | n79, n80 |  |
| SUL_n79-n812 | n79, n81 |
| NOTE 1: If a UE is configured with both NR UL and NR SUL carriers in a cell, the switching time between NR UL carrier and NR SUL carrier is 0us. NOTE 2: Applicable for UE supporting SUL with mandatory simultaneous Rx/Tx capability. |  |