3GPP TSG RAN WG4 Meeting #87	R4-1807420

Busan, Korea, May 21 – 25, 2018

Agenda Item:	7.4.2.3

Source:	Huawei, HiSilicon

Title:	Detailing the GSCN calculation

Document for:	Approval

# Introduction

In the last meeting, decisions finalizing the synchronization raster design were made [1][3][4]. In [2], a procedure to determine the parameters for the GSCN within a band was presented. However, with the decisions made, the procedure needs to be revised accordingly. The contribution details the revised procedure and provides the basis for the text proposal in [5].

# Discussion

## Background

For discussion purposes, the following table from the CR is copied [1].

Table 5.4.3.1-1: GSCN parameters for the global frequency raster

| Frequency range | SS block frequency position SSREF | GSCN | Range of GSCN |
| --- | --- | --- | --- |
| 0 – 3000 MHz | N * 1200kHz + M * 50 kHz, N=1:2499, M ϵ {1,3,5} (Note 1) | 3N + (M-3)/2 | 2 – 7498 |
| 3000 – 24250 MHz | 3000 MHz + N * 1.44 MHz N= 0:14756 | 7499 + N | 7499 – 22255 |
| 24250 – 100000 MHz | 24250.08 MHz + N * 17.28 MHz N= 0:4383 | 22256+ N | 22256– 26639 |
| NOTE 1: The default value for operating bands with SCS spaced channel raster is M=3. |  |  |  |

As stated in [2], for a given band, the GSCN is computed based on the band edge, the size of the synchronization signal block (SSB), and the guard band. The GSCN indicates the center of the SSB. For the lower band edge, the lowest frequency location for the center of the GSCN (fmin) is

fmin=bandlow+∆fNSCRBNRBSSB/2+guardSCSminBW	(1)

where bandlow is the lowest frequency of the band, ∆f is the subcarrier spacing, NSCRB=12 is the number of REs per RB, NRBSSB=20 is the number of RBs in the SSB, and guardSCSminBW is the size of the guard band for the subcarrier spacing and the minimum bandwidth. A similar formula is used for the upper band edge (fmax).

fmax=bandhigh-∆fNSCRBNRBSSB2-1-guardSCSminBW	(2)

where bandhigh is the upper frequency of the band.

## Derivation of GSCN

The procedure in [2] is modified to account for the different range of values for M. For representation purposes, the parameters and notation in Table 1 are used.

Table 1: GSCN parameters for the global frequency raster

| Frequency range, MHz | SS block frequency position SSREF | Flow, MHz | Fraster, MHz | Fshift, MHz |
| --- | --- | --- | --- | --- |
| 0 – 3000 | N * 1200kHz + M * 50 kHz, M {1,3,5} or M=3 for note 1 | 0 | 1.2 | 0.05 |
| 3000 – 24250 | 3000 MHz + N * 1.44 MHz | 3000 | 1.44 | 0 |
| 24250 – 100000 | 24250.08 MHz + N * 17.28 MHz | 24250.08 | 17.28 | 0 |

### Minimum value of GSCN

The generalized formula for finding GSCNmin is to find the set of N and M that satisfies

fmin≤Flow+N∆Fraster+M∆Fshift	(3)

as well as satisfying raster step requirements. For frequencies above 3000 MHz, M is not used. However, to keep the notation consistent, M is used but ∆Fshift=0.

The resulting integer programming problem is

Nmin≥fmin-Flow-M∆Fshift∆Fraster

The smallest value for Nmin is

Nmin≥1∆Frasterfmin-Flow-max⁡M∆Fshift	(4)

For ∆Fshift=0, no calculation for M is needed. Otherwise, to determine M

M≥1∆Fshiftfmin-Flow-Nmin∆Fraster	(5)

### Maximum value of GSCN

A similar process for the maximum value is followed.

fmax≥Flow+N∆Fraster+M∆Fshift	(6)

The resulting integer programming problem is

Nmax≤1∆Frasterfmin-Flow-M∆Fshift	(7)

and the largest value of N is

Nmax≤fmax-Flow-minM∆Fshift∆Fraster

Note if ∆Fshift=0, there is no calculation for M. Otherwise, to determine M

M≤fmax-Flow-Nmax∆Fraster∆Fshift

### Summary for revised calculation of parameters of GSCN

Table 2 captures the revised formulas to compute the GSCN. The computation for M uses minimum and maximum functions.

Table 2. Formulas to compute the minimum and maximum values of GSCN

| Type | Parameter | Range | Formula |
| --- | --- | --- | --- |
| For GSCNmin | N (compute first) | 0-3000 MHz | N=fmin-Flow-maxM∆Fshift∆Fraster,Mϵ1,3,53Note 1 above |
|  |  | ≥ 3000 MHz | N=fmin-Flow∆Fraster |
|  | M | 0-3000 MHz | M=maxfmin-Flow-N∆Fraster∆Fshift,1 |
|  |  | 0-3000 MHz, note 1 above | 3 |
|  |  | ≥ 3000 MHz | N/A |
| For GSCNmax | N (compute first) | 0-3000 MHz | N=fmax-Flow+minM∆Fshift∆Fraster,Mϵ1,3,53Note 1 above |
|  |  | ≥ 3000 MHz | N=fmax-Flow∆Fraster |
|  | M | 0-3000 MHz | M=minfmax-Flow-N∆Fraster∆Fshift,5 |
|  |  | 0-3000 MHz, note 1 above | 3 |
|  |  | ≥ 3000 MHz | N/A |

With the parameters for N (and M), the GSCN can be computed using Table 5.4.3.1-1. To incorporate step sizes for the GSCN, the following equations are

GSCNmin'≤RGSCNminR	(8)

GSCNmax'≥RGSCNmaxR	(9)

where R is the raster step size.

## Relationship of GSCN and global frequency raster

The synchronization raster is located on the global frequency raster. To determine the relationship between the GSCN and ARFCN, first equate the two relationships

FREF-Offsi+∆FGlobaliNREF-NREF-Offsi=Flowi+N∆Frasteri+M∆Fshifti	(10)

Re-expressing the equation as

NREF=NREF-Offsi+1∆FGlobaliFlowi-FREF-Offsi+∆Frasteri∆FGlobaliN+∆Fshifti∆FGlobaliM	(11)

The parameters and simplification of (11) are listed in Table 3 and Table 4.

Table 3. Parameters for computing the ARFCN from GSCN

| i | Range, MHz | FREF-Offsi, MHz | ∆FGlobali, kHz | NREF-Offsi | Flowi, MHz | ∆Frasteri, kHz | ∆Fshifti, kHz |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 – 3000 | 0 | 5 | 0 | 0 | 1200 | 50 |
| 1 | 3000 – 24250 | 3000 | 15 | 600000 | 3000 | 1440 | 0 |
| 2 | 24250 – 100000 | 24250.08 | 60 | 2016667 | 24250.08 | 17280 | 0 |

Table 4. Simplification of Eq. 11 where N and M are used to compute GSCN

| i | Range, MHz | NREF | N | M |
| --- | --- | --- | --- | --- |
| 0 | 0 – 3000 | 240N+10M | 0 - 2499 | 1, 3, 5 |
| 1 | 3000 – 24250 | 600000+96N | 0 - 14756 | - |
| 2 | 24250 – 100000 | 2016667+288N | 0 - 4383 | - |

# Conclusion

The contribution presented a derivation and closed form expressions for computing parameters for the GSCN according to the frequency band. In addition,

Proposal: The table captures the formula for determining the parameters for the GSCN and should be captured in a technical report.

Table 2. Formulas to compute the minimum and maximum values of GSCN

| Type | Parameter | Range | Formula |
| --- | --- | --- | --- |
| For GSCNmin | N (compute first) | 0-3000 MHz | N=fmin-Flow-maxM∆Fshift∆Fraster,Mϵ1,3,53Note 1 above |
|  |  | ≥ 3000 MHz | N=fmin-Flow∆Fraster |
|  | M | 0-3000 MHz | M=maxfmin-Flow-N∆Fraster∆Fshift,1 |
|  |  | 0-3000 MHz, note 1 above | 3 |
|  |  | ≥ 3000 MHz | N/A |
| For GSCNmax | N (compute first) | 0-3000 MHz | N=fmax-Flow+minM∆Fshift∆Fraster,Mϵ1,3,53Note 1 above |
|  |  | ≥ 3000 MHz | N=fmax-Flow∆Fraster |
|  | M | 0-3000 MHz | M=minfmax-Flow-N∆Fraster∆Fshift,5 |
|  |  | 0-3000 MHz, note 1 above | 3 |
|  |  | ≥ 3000 MHz | N/A |

# References

R4-1805983, “Draft CR for TS 38.104: Sync raster offset in re-farming bands (5.4.3)”, TS38.104, Ericsson, Samsung, Apr. 16-20, 2018.

R4-1804217, “Clarifying the GSCN calculation”, Huawei, Apr. 16-20, 2018.

R4-1805984, “TP to TR 38.817-01: Sync raster open issues (4.3.1)”, Ericsson, Apr. 16-20, 2018.

R4-1805980, “WF on sync raster for NR,” Ericsson, Apr. 16-20, 2018.

R4-1807423, “TP to capture the GSCN calculation”, Huawei, HiSilicon, RAN4#87, May 21-25, 2018.