3GPP TSG-RAN WG4 Meeting #87	R4-1807715

## Busan, Korea, 21 - 25 May, 2018

Source: 	Huawei

Title:	Correction on Occupied BS test applicability for CA

Agenda Item:	6.28.3.1 (AAS: RF conformance)

Document for:	Approval

# Introduction

During last RAN4#86bis meeting, Draft CR to the TS 37.145-2 was Endorsed for the OTA occupied bandwidth. It was observed that the specification of the Channel Bandwidth > 20MHz is used for the E-UTRA requirement specification.

In this contribution it is proposed to agree on the correction of the Occupied BW requirement (TS 36.104, TS 37.145-1) and OTA occupied BW test requirement (TS 37.145-2) for the CA operation.

# Discussion

Based on [1] it was observed that for the OTA occupied BS test requirement, term “> 20” was used for the E-UTRA BWchannel. Clearly, it shall be clarified that it refers to the aggregated channel bandwidth for CA operation.

The same issues exist in the TS 37.145-1 (back in Rel-13) and TS 36.141 (back in Rel-10) specifications. Therefore, before proceeding to CRs, it was found more efficient to first agree on the technical corrections (which can later be incorporated into CRs, or potentially merged with other corrections).

Below, we present the proposed fix for the Occupied BS test for the CA case (Based on TS 37.145-1 specification):

Table 1: Proposed correction of the Occupied bandwidth test requirement table on Span and number of measurement points (Based on TS 37.145-1)

------------------------------ Modified section ------------------------------ IoT Narrowband – Internet of Things OBW Occupied Band Width OFDM Orthogonal Frequency Division Multiplex ------------------------------ Next modified section ------------------------------ Table 6.6.2.4.2.4-1: Span and number of measurement points for OBW measurements Bandwidth; E-UTRA channel bandwidth BWChannel (MHz); Aggregated channel bandwidth BWChannel_CA (MHz). 1.4; 3; 5; 10; 15; 20; > 20. Span (MHz); 10; 10; 10; 20; 30; 40. Minimum number of measurement points; 1429; 667; 400; 400; 400; 400.

# Conclusion

Based on above, the following is proposed:

Proposal 1: agree on technical content of the Occupied BW test correction and implement the correction in TS 36.141, TS 37.145-1 and TS 37.145-2.

During this meeting, it is proposed to focus on the eAAS specifications corrections. Companies CR / Draft CR were submitted in [2 – 4].

The maintenance corrections of the E-UTRA specification in TS 36.141 are suggested to be postponed till the August RAN4 meeting.

# References

[1]	R4-1805859	Draft CR to TS 37.145-2 – adding a section 6.7.2 OTA occupied bandwidth, RAN4#86bis, NTT DOCOMO, INC.

[2]	R4-1807716	CR to TS 37.145-1: Correction on Occupied BS test applicability for CA, Rel-13	Huawei

[3]	R4-1807717	CR to TS 37.145-1: Correction on Occupied BS test applicability for CA, Rel-14	Huawei

[4]	R4-1807718	Draft CR to TS 37.145-2: Correction on Occupied BS test applicability for CA, Rel-15	Huawei