3GPP TSG-RAN WG4 Meeting #87	R4-1807734

## Busan, Korea, 21 - 25 May, 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.105 | CR | CRNum | rev | - | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to TS 37.105: BS demodulation requirements for OTA AAS BS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | AASenh_BS_LTE_UTRA-Perf |  |  |  |  |  |  | Date: |  | 2018-05-11 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In this Cat. F DraftCR, conducted BS demodulation requirements are corrected, and the radiated BS demodulation requirements section is completed with the information on the limited set of BS demodulation requirements applicable to OTA AAS BS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - 8: - multiple correction to distinguish the hybrid AAS BS - 11: - New text in 11.1 subclause (General) - Removal of UTRA TDD requirements for OTA AAS BS - multiple correction to distinguish the OTA AAS BS - correction of subclause references - text capturing limited set of BS demod requirements applicable to OTA AAS BS |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Specification of the conducted and radiated BS demodulation requirements for hybrid AAS BS and OTA AAS BS would be incomplete and misleading. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1, 8.1.1, 8.1.2, 8.2, 8.3, 8.4, 11, 11.1, 11.1.1, 11.1.2, 11.1.3, 11.2, 11.3, 11.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | This contribution is a resubmission of R4-1804920 (not treated during RAN4#86bis). Resubmission is aligned with the latest version of TS 37.105 in R4-1806002, ,i.e. no conflicting changes. |  |  |  |  |  |  |  |  |

------------------------------ Modified section ------------------------------

# 8	Performance requirements

## 8.1	General

Performance requirements specify the ability of the hybrid AAS BS to correctly demodulate signals in various conditions and configurations.

The demodulation requirements for a hybrid AAS BS are the same as non-AAS BS demodulation requirements specified for:

-	Single RAT UTRA operation in 3GPP TS 25.104 [9] clause 8 for FDD operation, and in 3GPP TS 25.105 [10] clause 8 for TDD operation,

-	Single RAT E-UTRA operation in 3GPP TS 36.104 [11], subclauses 8.1 – 8.4.

### 8.1.1	UTRA operation

Performance requirements for single RAT UTRA operation in FDD are specified for the measurement channels defined in 3GPP TS 25.104 [2] and 3GPP TS 25.105 [3]. The requirements only apply to those measurement channels that are supported by hybrid AAS BS. For FRC8 in 3GPP TS 25.104 [2] the non E-DPCCH boosting and E-DPCCH boosting requirement only apply for the option supported by the hybrid AAS BS. The performance requirements for the high speed train scenarios defined in 3GPP TS 25.104 [2] and 3GPP TS 25.105 [3] are optional.

Unless stated otherwise, performance requirements apply for a single cell only. Performance requirements for an hybrid AAS BS supporting UTRA FDD DC-HSUPA or DB-DC-HSUPA and UTRA TDD MC_HSUPA are defined in terms of single carrier requirements. For FDD operation the requirements in clause 8 shall be met with the transmitter unit(s) associated with the TAB connectors(s) in the operating band ON.

NOTE:	In normal operating conditions the TAB connectors(s) in UTRA FDD operation are configured to transmit and receive at the same time. The transmitter unit(s) associated with the TAB connectors may be OFF for some of the tests as specified in 3GPP TS 37.145 [13].

In the referred UTRA specifications and in this section, the term BS with RX diversity refers to performance requirements for two demodulation branches, and BS without RX diversity refers to performance requirements for one demodulation branch.

For hybrid AAS BS with RX diversity, only the BS performance requirements with RX diversity apply, the required Eb/N0 for UTRA FDD and Îor/Ioc for UTRA TDD shall be applied separately for each demodulation branch.

For hybrid AAS BS without RX diversity, only the BS performance requirements without RX diversity apply. The required Eb/N0 for UTRA FDD and Îor/Ioc for UTRA TDD shall be applied for each AAS BS demodulation branch.

The Eb/N0 used for UTRA FDD is defined as:

Where:

is the received total energy of DPDCH, DPCCH, S-DPCCH, HS-DPCCH, E-DPDCH, S-E-DPDCH, E-DPCCH and S-E-DPCCH per PN chip per demodulation branch from all branches

is the total one-sided noise power spectral density due to all noise sources

is the number of chips per frame

is the number of information bits in DTCH excluding CRC bits per frame

Table 8.1.1-1: Summary of hybrid AAS BS performance targets for single RAT UTRA operation

| Physical channel | Measurement channel | Static | Multi-path Case 1 | Multi-path Case 2 | Multi-path Case 3 | Moving (NOTE 1) | Birth / Death (NOTE 1) | High Speed Train |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Performance metric |  |  |  |  |  |  |
| DCH | 12.2 kbps | BLER<10-2 | BLER<10-2 | BLER<10-2 | BLER<10-2 | BLER<10-2 | BLER<10-2 | BLER<10-2 |
|  | 64 kbps | BLER< 10-1,10-2 | BLER< 10-1, 10-2 | BLER< 10-1,10-2 | BLER< 10-1, 10-2,10-3 | BLER< 10-1,10-2 | BLER< 10-1,10-2 | BLER< 10-1, 10-2 (NOTE 2) |
|  | 144 kbps | BLER< 10-1,10-2 | BLER< 10-1,10-2 | BLER< 10-1,10-2 | BLER< 10-1, 10-2,10-3 | - | - |  |
|  | 384 kbps | BLER< 10-1,10-2 | BLER< 10-1,10-2 | BLER< 10-1,10-2 | BLER< 10-1, 10-2,10-3 | - | - |  |
| NOTE 1: UTRA FDD only. NOTE 2: UTRA TDD only. NOTE 3: If not stated otherwise, the above performance targets are applicable to UTRA TDD and to UTRA FDD. NOTE 4: In case of multiple BLER level thresholds listed for single requirement and measurement cannel combination, those BLER level values are reflected by set multiple requirements in 3GPP TS 25.104 [2], or 3GPP TS 25.105 [3]. |  |  |  |  |  |  |  |  |

### 8.1.2	E-UTRA operation

Performance requirements for the hybrid AAS BS are specified for the fixed reference channels (FRC) and propagation conditions defined in 3GPP TS 36.104 [8] annex A and annex B, respectively. The requirements only apply to those FRCs that are supported by the hybrid AAS BS.

Unless stated otherwise, performance requirements apply for a single carrier only. Performance requirements for an hybrid AAS BS in E-UTRA operation supporting carrier aggregation are defined in terms of single carrier requirements. For FDD operation the requirements shall be met with the transmitter unit(s) associated with the TAB connectors(s) in the operating band ON.

NOTE:	In normal operating conditions TAB connectors in FDD operation are configured to transmit and receive at the same time. The transmitter unit(s) associated with the TAB connectors may be OFF for some of the tests as specified in 3GPP TS 37.145 [13].

In the referred E-UTRA specification, the term "RX antennas" refers to demodulation branches (and not physical antennas).

The SNR used in this clause is specified based on a single carrier and defined as:

SNR = S / N

Where:

S 	is the total signal energy in the subframe on a single TAB connector.

N	is the noise energy in a bandwidth corresponding to the transmission bandwidth over the duration of a subframe.

For enhanced performance requirements type A, the SINR used in this clause is specified based on a single carrier and defined as:

Where:

is the total signal energy in the subframe on a single TAB connector.

is the summation of the received energy of the strongest interferers explicitly defined in a test procedure plus the white noise energy N, in a bandwidth corresponding to the transmission bandwidth over the duration of a subframe on a single TAB connector. The respective energy of each interferer relative to  is defined by its associated DIP value.

## 8.2	Minimum requirements for MSR operation

For hybrid AAS BS in single RAT UTRA operation, minimum requirements for conducted demodulation performance are specified in subclause 8.3.

For hybrid AAS BS in single RAT E-UTRA operation, minimum requirements for conducted demodulation performance are specified in subclause 8.4.

## 8.3	Minimum requirements for UTRA operation

For hybrid AAS BS in single RAT UTRA operation in FDD, all mandatory BS demodulation performance requirements specified in subclauses 8.2 to 8.12 of 3GPP TS 25.104 [6] shall be fulfilled.

For hybrid AAS BS in single RAT UTRA operation in TDD, all mandatory BS demodulation performance requirements specified in subclauses 8.2 to 8.5 of 3GPP TS 25.105 [7] shall be fulfilled.

In the above referred UTRA specifications, the term BS with RX diversity refers to performance requirements for two demodulation branches, and BS without RX diversity refers to performance requirements for one demodulation branch.

## 8.4	Minimum requirements for E-UTRA operation

For hybrid AAS BS in single RAT E-UTRA operation, all mandatory BS demodulation performance requirements specified in subclauses 8.2 to 8.4 of 3GPP TS 36.104 [8] shall be fulfilled.

In the above referred E-UTRA specification, the term "RX antennas" refers to demodulation branches (i.e. not physical antennas).

------------------------------ Next modified section ------------------------------

# 11	Radiated performance requirements

## 11.1	General

Radiated performance requirements specify the ability of the OTA AAS BS to correctly demodulate radiated signals in various conditions and configurations.

The demodulation requirements for an OTA AAS BS are limited by two OTA demodulations branches as described in subcluse 11.1.1, and are specified for:

-	Single RAT UTRA operation in 3GPP TS 25.104 [9] clause 8 for FDD operation,

-	Single RAT E-UTRA operation in 3GPP TS 36.104 [11], subclauses 8.1 – 8.4.

The present document does not establish radiated performance requirements for NB-IoT in band, NB-IoT guard band, or standalone NB-IoT operation, for OTA AAS BS in single RAT E-UTRA operation or in MSR operation using E-UTRA.

### 11.1.1	OTA demodulation branches

OTA performance requirements are only specified for up to 2 demodulation branches (i.e. up to 1TX-2RX test setup, as captured in 3GPP TS 37.145-2 []) if the OTA AAS BS uses polarisation diversity and has the ability to maintain isolation between the performance requirements signals for each of the demodulation branches.

If the OTA AAS BS does not use polarisation diversity then performance requirements only apply to a single demodulation branch (i.e. 1TX-1RX test setup).

### 11.1.2	UTRA operation

Radiated performance requirements for single RAT UTRA operation in FDD are specified for the measurement channels defined in 3GPP TS 25.104 [2]. The requirements only apply to those measurement channels that are supported by OTA AAS BS. For FRC8 in 3GPP TS 25.104 [2] the non E-DPCCH boosting and E-DPCCH boosting requirement only apply for the option supported by the OTA AAS BS. The performance requirements for the high speed train scenarios defined in 3GPP TS 25.104 [2] are optional.

Unless stated otherwise, performance requirements apply for a single cell only. Performance requirements for an OTA AAS BS supporting UTRA FDD DC-HSUPA or DB-DC-HSUPA are defined in terms of single carrier requirements. For FDD operation the requirements in clause 11 shall be met with the transmitter unit(s) associated with the RIB in the operating band ON.

NOTE:	In normal operating conditions the transceiver units in UTRA FDD operation are configured to transmit and receive at the same time. The transmitter unit(s) associated with the RIB may be OFF for some of the tests as specified in 3GPP TS 37.145 [13].

In the referred UTRA specification 3GPP TS 25.104 [2] and in this section, the term BS with RX diversity refers to performance requirements for two demodulation branches, and BS without RX diversity refers to performance requirements for one demodulation branch.

For OTA AAS BS with RX diversity, only the BS performance requirements with RX diversity apply, the required Eb/N0 for UTRA FDD shall be applied separately for each demodulation branch.

For OTA AAS BS without RX diversity, only the BS performance requirements without RX diversity apply. The required Eb/N0 for UTRA FDD shall be applied for each AAS BS demodulation branch.

The Eb/N0 used for UTRA FDD is defined as:

Where:

is the received total energy of DPDCH, DPCCH, S-DPCCH, HS-DPCCH, E-DPDCH, S-E-DPDCH, E-DPCCH and S-E-DPCCH per PN chip per demodulation branch from all branches

is the total one-sided noise power spectral density due to all noise sources

is the number of chips per frame

is the number of information bits in DTCH excluding CRC bits per frame

Table 11.1.2-1: Summary of OTA AAS BS performance targets for single RAT UTRA FDD operation

| Physical channel | Measurement channel | Static | Multi-path Case 1 | Multi-path Case 2 | Multi-path Case 3 | Moving | Birth / Death | High Speed Train |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Performance metric |  |  |  |  |  |  |
| DCH | 12.2 kbps | BLER<10-2 | BLER<10-2 | BLER<10-2 | BLER<10-2 | BLER<10-2 | BLER<10-2 | BLER<10-2 |
|  | 64 kbps | BLER< 10-1,10-2 | BLER< 10-1, 10-2 | BLER< 10-1,10-2 | BLER< 10-1, 10-2,10-3 | BLER< 10-1,10-2 | BLER< 10-1,10-2 | - |
|  | 144 kbps | BLER< 10-1,10-2 | BLER< 10-1,10-2 | BLER< 10-1,10-2 | BLER< 10-1, 10-2,10-3 | - | - | - |
|  | 384 kbps | BLER< 10-1,10-2 | BLER< 10-1,10-2 | BLER< 10-1,10-2 | BLER< 10-1, 10-2,10-3 | - | - | - |
| NOTE: In case of multiple BLER level thresholds listed for single requirement and measurement cannel combination, those BLER level values are reflected by set multiple requirements in 3GPP TS 25.104 [2]. |  |  |  |  |  |  |  |  |

### 11.1.3	E-UTRA operation

Radiated performance requirements for the OTA AAS BS are specified for the fixed reference channels (FRC) and propagation conditions defined in 3GPP TS 36.104 [8] annex A and annex B, respectively. The requirements only apply to those FRCs that are supported by the OTA AAS BS.

Unless stated otherwise, performance requirements apply for a single carrier only. Performance requirements for an OTA AAS BS in E-UTRA operation supporting carrier aggregation are defined in terms of single carrier requirements. For FDD operation the requirements shall be met with the transmitter unit(s) associated with the RIB in the operating band ON.

NOTE:	In normal operating conditions the transceiver units in FDD operation are configured to transmit and receive at the same time. The transmitter unit(s) associated with the RIB may be OFF for some of the tests as specified in 3GPP TS 37.145 [13].

In the referred E-UTRA specification, the term "RX antennas" refers to demodulation branches (and not physical antennas).

The SNR used in this clause is specified based on a single carrier and defined as:

SNR = S / N

Where:

S 	is the total signal energy in the subframe.

N	is the noise energy in a bandwidth corresponding to the transmission bandwidth over the duration of a subframe.

For enhanced performance requirements type A, the SINR used in this clause is specified based on a single carrier and defined as:

Where:

is the total signal energy in the subframe.

is the summation of the received energy of the strongest interferers explicitly defined in a test procedure plus the white noise energy N, in a bandwidth corresponding to the transmission bandwidth over the duration of a subframe. The respective energy of each interferer relative to  is defined by its associated DIP value.

## 11.2	Minimum requirements for MSR operation

For OTA AAS BS in single RAT UTRA operation, minimum requirements for radiated demodulation performance are specified in subclause 11.3.

For OTA AAS BS in single RAT E-UTRA operation, minimum requirements for radiated demodulation performance are specified in subclause 11.4.

## 11.3	Minimum requirements for UTRA operation

For OTA AAS BS in single RAT UTRA operation in FDD, all mandatory BS demodulation performance requirements specified in subclauses 8.2 to 8.12 of 3GPP TS 25.104 [6] shall be fulfilled, considering limitation of up to two demodulation branches as captured in subclause 11.1.1.

NOTE:	There are also optional BS demodulation performance requirements specified in 3GPP TS 25.104 [6], which are subject to manufacturer’s declarations.

In the referred UTRA specification, the term BS with RX diversity refers to performance requirements for two demodulation branches, and BS without RX diversity refers to performance requirements for one demodulation branch.

## 11.4	Minimum requirements for E-UTRA operation

For OTA AAS BS in single RAT E-UTRA operation, all mandatory BS demodulation performance requirements specified in subclauses 8.2 to 8.4 of 3GPP TS 36.104 [8] shall be fulfilled, considering limitation of up to two demodulation branches as captured in subclause 11.1.1.

NOTE:	There are also optional BS demodulation performance requirements specified in  3GPP TS 36.104 [8], which are subject to manufacturer’s declarations.

In the referred E-UTRA specification, the term "RX antennas" refers to demodulation branches (i.e. not physical antennas).

------------------------------ End of modified section ------------------------------