3GPP TSG-RAN WG4 Meeting #87                 	R4-1807756

Busan, Korea (Republic of), 21-25 May 2018

Source:	Nokia, Nokia Shanghai Bell

Title:	Simulation results and remaining parameters for SPUSCH

Agenda item:	6.22.6.1

Document for:	Discussion

Introduction

In this contribution we discuss DMRS pattern for SPUSCH and introduce simulation results using the parameters as agreed in [1].

Simulation assumptions

Simulation assumptions as agreed for SPUSCH in [1] are shown below.

Table 1: Simulation assumptions for SPUSCH

| Parameter | Value |
| --- | --- |
| Number of Tx antennas | 1 |
| Number of Rx antennas | 2, 4 |
| Propagation condition | EPA5 |
| Cyclic prefix | Normal |
| Reference receiver | MRC |
| Channel bandwidth | [5 MHz], 10 MHz, 15 MHz, 20 MHz |
| TTI length | Subslot TTI |
| Number of OFDM-symbols | Subslot TTI: 2 and 3 |
| Fraction of maximum throughput | 70 % |
| Maximum number of HARQ transmissions | 4 |
| DMRS configuration | IFDMA is not configured |
| DMRS sharing pattern | Option 1: - RDD DD DD RD DD RDD Option 2: - RDD DD¦R RD DD¦R RD RDD |
| Number of HARQ processes | 16 |
| RV sequence | 0, 2, 3, 1, 0, 2, 3, 1 |
| Modulation | 16QAM |

Additionally, the following FRCs were tentatively agreed in the last RAN4 meeting:

# Table A.4-1 FRC parameters for performance requirements (16QAM 3/4)

| Reference channel | Ax-1 |  |  | Ax-2 |  | Ax-3 |  | Ax-4 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Allocated resource blocks | 24 |  |  | 48 |  | 72 |  | 100 |  |
| DFT-OFDM Symbols per subframe | 1 | 2 |  | 1 | 2 | 1 | 2 | 1 | 2 |
| Modulation | 16QAM |  |  | 16QAM |  | 16QAM |  | 16QAM |  |
| Code rate | 3/4 |  |  | 3/4 |  | 3/4 |  | 3/4 |  |
| Payload size (bits) | 872 |  | 1736 | 1736 | 3496 | 2536 | 5160 | 3624 | 7224 |
| Transport block CRC (bits) | 24 |  | 24 | 24 | 24 | 24 | 24 | 24 | 24 |
| Code block CRC size (bits) | 24 |  | 24 | 24 | 24 | 24 | 24 | 24 | 24 |
| Number of code blocks - C | 1 |  | 1 | 1 | 1 | 1 | 1 | 1 | 2 |
| Total number of bits per sub-frame | 1152 |  | 2304 | 2304 | 4608 | 3456 | 6912 | 4800 | 9600 |
| Total symbols per sub-frame | 288 |  | 576 | 576 | 1152 | 864 | 1728 | 1200 | 2400 |

The only parameter that is still open is the DMRS pattern. In our view, both proposed patterns are feasible and can be used in demodulation tests. In our simulations we have used Option 1, for which results are shown in section 3 below. In our view it is ok to use Option 1 in SPUSCH demodulation tests, as this option is simpler of the two proposed options.

Use Option 1 RDD DD DD RD DD RDD as the DMRS pattern for SPUSCH demodulation tests.

We have also used the FRCs from Table A.4-1 in our simulations, and think these FRCs are feasible for SPUSCH demodulation tests.

Simulation results

Simulation results for SPUSCH using the parameters from Table 1, DMRS pattern RDD DD DD RD DD RDD, and FRCs as in Table A.4-1 are shown below in Table 2.

Table 2: Initial simulation results for SPUSCH.

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix | FRC | Fraction of maximum throughput | SNR, dB |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | EPA 5Hz Low | Ax-1 | 70% | 10.65 |
|  |  |  |  | Ax-2 | 70% | 10.61 |
|  |  |  |  | Ax-3 | 70% | 10.74 |
|  |  |  |  | Ax-4 | 70% | 10.96 |
|  | 4 | Normal | EPA 5Hz Low | Ax-1 | 70% | 6.96 |
|  |  |  |  | Ax-2 | 70% | 6.84 |
|  |  |  |  | Ax-3 | 70% | 6.8 |
|  |  |  |  | Ax-4 | 70% | 7.03 |

Conclusion

In this contribution we have presented simulation results for SPUSCH.

Considering simulation assumptions, we have made the following proposal:

- Use Option 1 RDD DD DD RD DD RDD as the DMRS pattern for SPUSCH demodulation tests.

- References

- R4-1805495, Simulation assumptions for sPUCCH and sPUSCH, Ericsson, 3GPP TSG-RAN WG4 #86bis, Melbourne, April 2018.