3GPPRAN4#87	R4-1807794

Busan, Korea, May 21st– 25th

Title: 	BWP SCS restriction based on UE CC BW

Source: 	Qualcomm Incorporated

Agenda item:	7.4 System Parameters [NR_newRAT-Core]

Document for:	Discussion

# Introduction

In this contribution, we specify restrictions that need to be placed on SCS of a BWP based on the configured UE CC BW.

# Discussion

## Background

In NR we have the concept of BWP for a UE where the gNodeB can configure the UE to a smaller chunk of BW within a CC rather than the full CC BW. From the UE perspective, this would be useful for power savings as the network can configure the UE with a BWP that has BW smaller than UE CC BW when there are no grants and then switch to a BWP with wider BW when there are grants. Based on previous discussion, a BWP change could potentially entail a change to SCS, BW and LO frequency, or any subset of those. However, it is up to the UE to decide whether it wants to switch to a smaller BW to avail itself of the power savings or continue to operate in the CC BW to meet the RF requirements [1]. From the network perspective a UE that is switching to a smaller BW and one that is always operating in the CC BW are identical.

Since the UE can choose to forego the power savings and always operate in the CC BW, this places certain limitations on the SCS’s of BWP that can be configured. As an example, consider a UE with a maximum capability of 4K point FFT. From a demodulation perspective, this UE can support CC BW of 100MHz. However, with a configured UE CC BW of 100MHz, UE cannot be configured with a BWP having 15 kHz SCS. The reason here is that if the UE decides to forego the power savings and keep operating in 100MHz RF BW, it would need to use an 8K point FFT, which would be beyond the capabilities. Note, that this restriction is only applicable when the UE CC BW is larger than the BW supported for that SCS.

Proposal 1: SCS of a BWP should be restricted such that UE can always operate in CC BW, irrespective of the BWP configured.

# Conclusions

Proposal 1: SCS of a BWP should be restricted such that UE can always operate in CC BW, irrespective of the BWP configured.

# References

[1] R4-1801006, WF on wideband operation of NR UE, 3GPP TSG-RAN WG4 #AH-1801 Meeting, San Diego, California, USA, Jan. 22 – 26, 2018