3GPP TSG-RAN WG4 Meeting #87													        R4-1807801

Busan, Korea (Republic of), 21 - 25 May 2018

Source: 	Huawei, HiSilicon

Title: 					n51 A-MPR

Agenda Item: 			7.2.2.1.1

Document for:	Approval

# 1	Introduction

As discussed during the last RAN4 meeting and approved, the contribution [1] provided all technical definitions to be able to make the A-MPR studies to protect the EESService. Thus, A-MPR evaluation for n50 is conducted as defined just below:

- Proposal 1:

  - EESS protection level => -32dBm/27MHz1  in the 1400-1427MHz

- Proposal 2: A-MPR evaluation for n51 is conducted as

  - Definition of the study => Study the Tx only in the 1427-1432MHz band to protect EESS which is in 1400-1427MHz band

  - Tx CBW [MHz] => 5, 10, 15, 20, 40, 50, 60

  - Protected Band [MHz] => 1400-1427

This contribution presents the simulation results for the above scenarios.

# 2	Discussion

## 2.1	Simulation assumptions

A-MPR study for EESS protection has been defined as:

- Band range:

  - start:1432-1517MHz BW:5

- SCS:60M 30k, 15k for other BW

- SE limitation1:EESS: -32dBm/27MHz1  in the 1400-1427MHz

- ACLR: EUTRA limit 30dB

SEM:

MPR:

Table 6.2.2-1 Maximum power reduction (MPR) for power class [2] and 3

| Modulation | MPR (dB) |  |
| --- | --- | --- |
|  | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM PI/2 BPSK | ≤ 0.5 | 0 |
| DFT-s-OFDM QPSK | ≤ 1 | 0 |
| DFT-s-OFDM 16 QAM | ≤ 2 | ≤ 1 |
| DFT-s-OFDM 64 QAM | ≤ 2.5 |  |
| DFT-s-OFDM 256 QAM | 4.5 |  |
| CP-OFDM QPSK | ≤ 3 | ≤ 1.5 |
| CP-OFDM 16 QAM | ≤ 3 | ≤ 2 |
| CP-OFDM 64 QAM | ≤ 3.5 |  |
| CP-OFDM 256 QAM | ≤ 6.5 |  |

## 2.2 	EESS protection

# 2	AMPR results for EESS

| Modulation | AMPR |  |
| --- | --- | --- |
|  | 5M |  |
|  | Outer | inner |
| DFT-s-OFDM QPSK | 15.5 | 12 |
| DFT-s-OFDM 16 QAM | 14.5 | 11 |
| DFT-s-OFDM 64 QAM | 14.5 | 10 |
| DFT-s-OFDM 256 QAM | 12.5 | 7.5 |
| CP-OFDM QPSK | 14.5 | 10 |
| CP-OFDM 16 QAM | 14.5 | 10 |
| CP-OFDM 64 QAM | 14 | 8 |
| CP-OFDM 256 QAM | 11 | 5.5 |

# 2	Figures to protect EESS in 1400-1427MHz band

# Below, 8 A-MPR studies are proposed. At the end of each paragraph, there is a table to summary the A-MPR which need to implement to protect EESService for DFT-S-OFDM and CP-OFDM.

## 2.3 	DFT-S-OFDM

We note that in the case of different QAM, the result is alsomost the same. Thus, we make only one A-MPR table for all QAM cases above.

The table 1 just below is the summary of the A-MPR needed to protect EESService.

Table 1: A-MPR for "NS_7" for DFT-S-OFDM case

| Channel bandwidth [MHz] | Parameters | Region A |
| --- | --- | --- |
| 5 | RBstart | 8 – 15 |
|  | LCRB [RBs] | 0 - 6 |
|  | A-MPR [dB] | ≤ 12.5 |

## 2.3 	CP-OFDM

As for the DFT-S-OFDM, for all QAM cases the A-MPR results were very similars, to study CD-OFDM case, we focus only on one case.

Just below, the table 2 is the summary of the A-MPR needed to protect EESService in the case of the CP-OFDM nodulation.

Table 2: A-MPR for "NS_7" for CP-OFDM case

| Channel bandwidth [MHz] | Parameters | Region A |
| --- | --- | --- |
| 5 | RBstart | 8 – 15 |
|  | LCRB [RBs] | 0 – 6 |
|  | A-MPR [dB] | ≤ 14.5 |

# 3	Conclusion

In this contribution we have presented NR A-MPR back off for Band n51. As the difference between DFT-S-OFDM and CP-OFDM is small, it is proposed:

Proposal 1: take into account only the DFT-S-OFDM modulation

Proposal 2: The tables 3 & 4 below propose protection A-MPR levels for EESS. These tables can be the reference tables for A-MPR parameters in the TS to protect EESS in the 1400-1427MHz band:

Table 3: A-MPR for "NS_7"

| Channel bandwidth [MHz] | Parameters | Region A |
| --- | --- | --- |
| 5 | RBstart | 8 – 15 |
|  | LCRB [RBs] | 0 – 6 |
|  | A-MPR [dB] | ≤ 14.5 |

Table 4: AMPR results for EESS

| Modulation | AMPR |  |
| --- | --- | --- |
|  | 5M |  |
|  | Outer | inner |
| DFT-s-OFDM QPSK | 15.5 | 12 |
| DFT-s-OFDM 16 QAM | 14.5 | 11 |
| DFT-s-OFDM 64 QAM | 14.5 | 10 |
| DFT-s-OFDM 256 QAM | 12.5 | 7.5 |
| CP-OFDM QPSK | 14.5 | 10 |
| CP-OFDM 16 QAM | 14.5 | 10 |
| CP-OFDM 64 QAM | 14 | 8 |
| CP-OFDM 256 QAM | 11 | 5.5 |

# 4	Reference

[1] R4-1805727, WF on NR A-MPR evaluation scenarios for n50, n51, and n74, NTT DOCOMO, INC., Huawei, Etisalat, RAN4#86bis