3GPP TSG-RAN4 Meeting #87 	R4-1807971

# Busan, Korea, 21 – 25 May, 2018

Title:	LS to RAN2 on measurement gap applicability

Response to:

Release:	Rel-15

Work Item:	NR_newRAT-Core

Source:	RAN WG 4

To:	RAN WG 2

CC:

Contact Person:

#### Name:	Jie Cui

E-mail Address:	jie.cui@intel.com

Attachments:	N/A

1. Overall Description:

RAN4 would like to inform RAN2 that RAN4 made the following revision to measurement gap applicability for Rel-15 to TS36.133 and TS38.133

- Revision to MG pattern table in LTE RRM spec TS36.133

Table 8.1.2.1-1: Gap Pattern Configurations supported by the UE Gap Pattern Id; MeasurementGap Length (MGL, ms); Measurement Gap Repetition Period (MGRP, ms); Minimum available time for inter-frequency and inter-RAT measurements during 480ms period (Tinter1, ms); Measurement Purpose. 0; 6; 40; 60; Inter-Frequency E-UTRAN FDD and TDD, UTRAN FDD, GERAN, LCR TDD, HRPD, CDMA2000 1x, inter-RAT NR. 1; 6; 80; 30; Inter-Frequency E-UTRAN FDD and TDD, UTRAN FDD, GERAN, LCR TDD, HRPD, CDMA2000 1x, inter-RAT NR. 2; 3; 40; 24NOTE 1,2; Inter-Frequency E-UTRAN FDD and TDD for cells with time difference as specified below. inter-RAT NR. 3; 3; 80; 12NOTE 1,2; Inter-Frequency E-UTRAN FDD and TDD for cells with time difference according as specified below. inter-RAT NR. [4]; [6]; [20]; Note 3; inter-RAT NR. [5]; 6; 160; Note 3; inter-RAT NR. [6]; [4]; [20]; Note 3; inter-RAT NR. [7]; 4; 40; Note 3; inter-RAT NR. [8]; 4; 80; Note 3; inter-RAT NR. [9]; [4]; [160]; Note 3; inter-RAT NR. [10]; 3; 20; Note 3; inter-RAT NR. [11]; 3; 160; Note 3; inter-RAT NR. [12]; [5.5]; [20]; Note 3, 4; inter-RAT NR. [13]; [5.5]; 40; Note 3, 4; inter-RAT NR. [14]; [5.5]; 80; Note 3, 4; inter-RAT NR. [15]; [5.5]; 160; Note 3, 4; inter-RAT NR. [16]; [3.5]; 20; Note 3, 4; inter-RAT NR. [17]; [3.5]; 40; Note 3, 4; inter-RAT NR. [18]; [3.5]; 80; Note 3, 4; inter-RAT NR. [19]; [3.5]; 160; Note 3, 4; inter-RAT NR. [20]; [1.5]; 20; Note 3, 4; inter-RAT NR. [21]; [1.5]; 40; Note 3, 4; inter-RAT NR. [22]; [1.5]; 80; Note 3, 4; inter-RAT NR. [23]; [1.5]; [160]; Note 3, 4; inter-RAT NR. NOTE 1: When determing UE requirements using Tinter1 for GP2 and GP3, Tinter1 = [60] for GP2 and Tinter1 = [30] for GP3 shall be used. NOTE 2: This gap pattern is supported by UEs which support shortMeasurementGap-r14 or by UEs which are configured to perform inter-RAT NR measurement only. NOTE 3: Editor’s note:NR measurement requirement may not be scaled by Tinter NOTE 4: This gap pattern is applicable when per FR measurement gap is configured and one of ther serving cells in on FR2 and one of the to-be-measured cells is on FR2..

- Revision to MG applicability table in NR RRM spec TS38.133

Table 9.1.2-2: Applicability for Gap Pattern Configurations supported by the E-UTRA-NR dual connectivity UE Measurement gap pattern configuration; Serving cell; Measurement Purpose; Applicable Gap Pattern Id. Per-UE measurement gap; E-UTRA + FR1, or E-UTRA + FR2, or E-UTRA + FR1 + FR2; non-NR RAT Note1,2; 0,1,2,3. FR1 and/or FR2; 0-11. non-NR RATNote1,2 and FR1 and/or FR2; 0,1,2,3. Per FR measurement gap; E-UTRA and, FR1 if configured; non-NR RAT Note1,2; 0,1,2,3. FR2 if configured; No gap. E-UTRA and, FR1 if configured; FR1 only; 0-11. FR2 if configured; No gap. E-UTRA and, FR1 if configured; FR2 only; No gap. FR2 if configured; 12-23. E-UTRA and, FR1 if configured; non-NR RAT Note1,2 and FR1; 0,1,2,3. FR2 if configured; No gap. E-UTRA and, FR1 if configured; FR1 and FR2; 0-11. FR2 if configured; 12-23. E-UTRA and, FR1 if configured; non-NR RAT Note1,2 and FR2; 0,1,2,3. FR2 if configured; 12-23. E-UTRA and, FR1 if configured; non-NR RAT Note1,2 and FR1 and FR2; 0,1,2,3. FR2 if configured; 12-23. Note: if GSM or UTRA TDD or UTRA FDD inter-RAT frequency layer is configured to be monitered, only measurement gap pattern #0 and #1 can be used for per-FR gap in E-UTRA and FR1 if configured, or for per-UE gap. NOTE 1: Non-NR RAT includes E-UTRA, UTRA and/or GSM. NOTE 2: The gap pattern 2 and 3 are supported by UEs which support shortMeasurementGap-r14..

Regarding NR SA mode, RAN4 believes that it’s necessary to specify the capability indication signaling of supporting or not supporting short measurement gap (pattern #2 and #3) for LTE measurement in R15, such as the signaling of  “shortMeasurementGap-r14” in LTE.

2. Actions:

To RAN WG2:

RAN4 respectfully asks RAN2 to confirm if RAN2 has specified the capability indication signaling of supporting or not supporting short measurement gap (pattern #2 and #3) in NR standalone operation for LTE measurement, and if this signaling hasn’t been specfied, RAN4 respectfully ask RAN2 to specify it in R15 specification.

3. Date of Next RAN4 Meetings:

3GPP RAN4 #AH1807 		2- 6 July 2018, 		Montreal, Canada

3GPP RAN4 #88 		20- 24 Aug 2018, 	Gothenburg, Sweden