3GPP TSG-RAN4 Meeting #87	R4-1807983

# Melbourne, Korea, May 21-25, 2018

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 1 | rev | - | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | RRC Re-establishment Requirements in NR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Mediatek, Huawei, HiSilcon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-05-24 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | To specify requirements for RRC connection re-establishment to NR cell in NR SA operation |  |  |  |  |  |  |  |  |
| Summary of change: |  | In RAN4#86 RAN4 approved WF in R4-1803502 to specify requirements for RRC connection re-establishment to target NR cell in NR SA operation. The corresponding RRC connection re-establishment requirements for these cases are defined. The time to detect target NR cell is based on assumption that SSB SCH Ec/Iot ≥ -6 dB. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The requirements for RRC connection re-establishment to target NR cell shall be missing. This will degrade performance of NR SA operation. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.1, 6.2.1.1, 6.2.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  | TS 36.521-3 |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

----------------------START OF CHANGES----------------------------

### 6.2.1	SA: RRC Re-establishment

Editor’s note: intended to capture the re-establishment requirements only if NR cell is a PCell.

#### 6.2.1.1		Introduction

This clause contains requirements on the UE regarding RRC connection re-establishment procedure. RRC connection re-establishment is initiated when a UE in RRC_CONNECTED state loses RRC connection due to any of failure cases, including radio link failure, handover failure, and RRC connection reconfiguration failure. The RRC connection re-establishment procedure is specified in clause 5.3.7 of TS 38.331 [2].

The requirements in this clause are applicable for RRC connection re-establishment to NR cell.

#### 6.2.1.2		Requirements

### Editor’s note: The current requirement assumes that UE still has correct SFN, frame and subframe level synchronization to the PCell during the whole procedure. In case UE has lost the SFN, frame or subframe level synchronization to the PCell, how to handle this case should be further studied.

In RRC connected mode the UE shall be capable of sending RRCConnectionReestablishmentRequest message within Tre-establish_delay seconds from the moment it detects a loss in RRC connection.  The total RRC connection delay (Tre-establish_delay) shall be less than:

Tre-establish_delay=TUL_grant +TUE_re-establish_delay     ms

TUL_grant: It is the time required to acquire and process uplink grant from the target PCell. The uplink grant is required to transmit RRCConnectionReestablishmentRequest message.

The UE re-establishment delay (TUE_re-establish_delay) is specified in clause 6.2.1.2.1.

##### 6.2.1.2.1	UE Re-establishment delay requirement

The UE re-establishment delay (TUE_re-establish_delay) is the time between the moments when any of the conditions requiring RRC re-establishment as defined in clause 5.3.7 in TS 38.331 [2] is detected by the UE and when the UE sends PRACH to the target PCell. The UE re-establishment delay (TUE_re-establish_delay) requirement shall be less than:

TUE_re-establish_delay=50+Tidentify_intra_NR+i=1Nfreq-1Tidentify_inter_NR,i+TSI-NR+TPRACH     ms

The target NR cell shall be considered detectable when:

-	SS-RSRP related side conditions given in Section 10.x are fulfilled for a corresponding NR Band,

-	SCH_RP and SCH Ês/Iot according to Annex TBD for a corresponding NR Band.

Tidentify_intra_NR: It is the time to identify the target intra-frequency NR cell and it depends on whether the target NR cell is known cell or unknown cell and on the frequency range (FR) of the target NR cell. If the UE is not configured with intra-frequency NR carrier for RRC re-establishment then Tidentify_intra_NR=0; otherwise Tidentify_intra_NR shall not exceed the values defined in table 6.2.1.2.1-1.

Tidentify_inter_NR,i: It is the time to identify the target inter-frequency NR cell on inter-frequency carrier i configured for RRC re-establishment and it depends on whether the target NR cell is known cell or unknown cell and on the frequency range (FR) of the target NR cell. Tidentify_inter_NR,i shall not exceed the values defined in table 6.2.1.2.1-2.

TSMTC,i: It is the periodicity of the SMTC occasion configured for the inter-frequency carrier i.

TSI-NR  = It is the time required for receiving all the relevant system information according to the reception procedure and the RRC procedure delay of system information blocks defined in TS 38.331 [2] for the target PCell.

TPRACH = It is the delay caused due to the random access procedure when sending random access to the target NR cell. The delay depends on the PRACH configuration defined in Table 6.3.3.2-2 [6] or Table 6.3.3.2-3 [6] for FR1 and in Table 6.3.3.2-4 [6] for FR2.

Nfreq: It is the total number of NR frequencies to be monitored for RRC re-establishment; Nfreq = 1 if the target PCell is known.

There is no requirement if the target cell does not contain the UE context.

# Table 6.2.1.2.1-1: Time to identify target NR cell for RRC connection re-establishment to NR intra-frequency cell

| Frequency range (FR) of target NR cell | Tidentify_intra_NR [ms] |  |  |
| --- | --- | --- | --- |
|  |  | Known NR cell | Unknown NR cell |  |  |
| FR1 |  | MAX (200 ms, [5] x TSMTC) | MAX (800 ms, [10] x TSMTC) |  |
| FR2 |  | MAX (400 ms, K2 x [5] x TSMTC) | MAX (1000 ms, (K3 x [10]) x TSMTC)) |  |

Editor’s note: K2 and K3 are FFS and is the number of receiver beam sweeps required to measure/detect NR cell in FR2

# Table 6.2.1.2.1-2: Time to identify target NR cell for RRC connection re-establishment to NR inter-frequency cell

| Frequency range (FR) of target NR cell | Tidentify_inter_NR, i [ms] |  |  |
| --- | --- | --- | --- |
|  | Known NR cell | Unknown NR cell |  |
| FR1 | MAX (200 ms, [6] x TSMTC, i) | MAX (800 ms, [13] x TSMTC, i) |
| FR2 | MAX (400 ms, K4 x [6] x TSMTC, i) | MAX (1000 ms, (K5 x [13]) x TSMTC, i)) |

Editor’s note: K4 and K5 are FFS and is the number of receiver beam sweeps required to measure/detect NR cell in FR2

----------------------END OF CHANGES----------------------------