3GPP TSG-RAN WG4 Meeting #87	R4-1808064

Busan, Korea(Republic of), 21th – 25th May 2018

Source: 	KT

Title: 					TP for TR 36.715-02-01: CA_2DL_8A-27A_1UL_BCS0

Agenda Item: 			6.2.2

Document for:	Approval

1	Introduction

For the B8+B27 CA, KT has been reviewed the filter response for a long time and we has expected performance enhancement. However, we could not see any outstanding performance enhancement. In the meantime, KT can not postpone this CA combo completion any more. As a result, we propose below two values as the ∆TIB,c and ∆RIB,c. for this particular combination based on the combo which is related with Band 8 so far.

ΔTIB,c for 2DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_8A-27A | 8 | 0.8 |
|  | 27 | 0.8 |

ΔRIB,c for 2DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔRIB [dB] |
| --- | --- | --- |
| CA_8A-27A | 8 | 0.3 |
|  | 27 | 0.3 |

This contribution is a TP for introduction of CA_2DL_8A-27A_1UL_BCS0 into 36.715.

In addition, we captured all information from R4-1804063 which was already approved in RAN4 #86 bis.

******************************** Start of the TP *******************************************

5.X	CA_8A-27A

5.X.1	Operating band for CA

Table 5.X.1-1: 2DL Inter-band CA operating bands

| E-UTRA CA Band | E-UTRA Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_8A-27A | 8 | 880 MHz | -- | 915 MHz | 925 MHz |  | 960 MHz | FDD |
|  | 27 | 807 MHz | – | 824 MHz | 852 MHz | – | 869 MHz | FDD |

5.X.2	Channel bandwidths per operating band for CA

Table 5.X.2-1: Supported E-UTRA bandwidths per CA configuration for 2DL inter-band CA

| CA operating / Channel bandwidth |  |  |  |  |  |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth Combination Set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA Configuration | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |  |  |
| CA_8A-27A | 8 |  |  | yes | yes |  |  | 20 | 0 |
|  | 27 |  |  | yes | yes |  |  |  |  |

5.X.3	UE Co-existence studies

Table 5.X.3-1 summarizes frequency ranges where harmonics occur due to Band 8 and Band 27 CA with 1 UL.

Table 5.X.3-1: Impact of UL/DL Harmonic Interference

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 2nd Harmonic |  | 3rd Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| 8 | 880 | 915 | 925 | 960 | 1 760 | 1 830 | 2 640 | 2 745 | 1 850 | 1 920 | 1 775 | 2 880 |
| 27 | 807 | 824 | 852 | 869 | 1 614 | 1 648 | 2 421 | 2 472 | 1 704 | 1 738 | 2 556 | 2 607 |

It can be seen from Table 5.X.3-1 that there is no harmonic interference for CA_8A-27A towards own receive bands.

Table 5.X.3-2: Band 8 and Band 27 DL harmonics and IMD products

| BS DL carriers | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| DL frequency (MHz) | 925 | 960 | 852 | 869 |
| 2nd order harmonics frequency range (MHz) | 1 850 | 1 920 | 1 704 | 1 738 |
| 3rd order harmonics frequency range (MHz) | 2 775 | 2 880 | 2 556 | 2 607 |
| 2nd order IMD products | (f2_low – f1_high) | (f2_high – f1_low) | (f2_low + f1_low) | (f2_high + f1_high) |
| IMD frequency limits (MHz) | 56 | 108 | 1 777 | 1 829 |
| 3rd order IMD products | (f2_low – 2*f1_high) | (f2_high – 2*f1_low) | (2*f2_low – f1_high) | (2*f2_high – f1_low) |
| IMD frequency limits (MHz) | 981 | 1 068 | 744 | 813 |
| 3rd order IMD products | (2*f1_low + f2_low) | (2*f1_high + f2_high) | (2*f2_low + f1_low) | (2*f2_high + f1_high) |
| IMD frequency limits (MHz) | 2 702 | 2 789 | 2 629 | 2 698 |
| 3rd order IMD products | (f1_low – f2_high + f2_low) | (f1_high + f2_high – f2_low) | (f2_low – f1_high + f1_low) | (f2_high + f1_high – f1_low) |
|  | 908 | 977 | 817 | 904 |
| 3rd order IMD products (Considering Max BW) | (f1_low – max BW f2) | (f1_high + max BW f2) | (f2_low – max BW f1) | (f2_high + max BW f1) |
| IMD frequency limits (MHz) | 915 | 970 | 842 | 879 |

It can be seen from Table 5.X.3-2 that the 2nd harmonics of BS transmitting in Band 8 may fall into the BS receive band of Bands 2, 25, 33, 35,37 and 39 and none of 3rd harmonics may fall into the any E-UTRA operating BS receive band. In addition 2nd harmonics of BS transmitting in Band 27 may fall into the BS receive band of Bands 3, 4, 10 and 3rd harmonics may fall into BS receive band of Bands 7, 38 and 41.

Meanwhile the 2nd IMD products caused by BS supporting carrier aggregation of Band 8 and Band 27 may fall into the BS receive band of Band 3, 9 and 3rd IMD products may fall into the BS receive band of Band 5, 6, 8, 13, 14, 18, 19, 20, 26, 27, 28, 41 and 44. Note that the calculation in Table 5.X.3-2 (except the last row) assumes the BS is transmitting with the whole 35 MHz DL frequency of Band 8 and the whole 17 MHz DL frequency of Band 27.

If the BS is only transmitting an up to 10 MHz DL in Band 8 and 10 MHz in Band 27 as stated in the WIDs, the 3rd IMD products will not fall into the BS receive band of the Band 6, 8, or 18

It should be noted that Bands 2, 4, 10, 13, 14, 18, 19, 20, 25, 33, 35, 37, 38 and 39 are not intended for use in the same geographical area as Bands 8 and 27. Therefore, the focus here will be on the harmonics and IMD falling into Bands 3, 5, 7, 9, 26, 27, 28, 41 and 44. With the performances of the current BS antenna system, transmit and receive path components, amplifiers, pre-distortion algorithms and filters, it is expected that the IMD interference generated within the Band 3, 5, 7, 8, 9, 26, 27, 28, 41 or 44 receiver would be well below the receiver noise floor eliminating the possibility of receiver desensitization, provided that Bands 8 and 27 BS transmitters do not share the same antenna with Band 3, 5, 7, 8, 9, 26, 27, 28, 41 or 44 BS receiver

5.X.4	∆TIB,c and ∆RIB,c values

Following relaxations are allowed for the UE which supports inter-band carrier aggregation of Band 8 and Band 27.

Table 5.X.4-1: ΔTIB,c for 2DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_8A-27A | 8 | 0.8 |
|  | 27 | 0.8 |

Table 5.X.4-2: ΔRIB,c for 2DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔRIB [dB] |
| --- | --- | --- |
| CA_8A-27A | 8 | 0.3 |
|  | 27 | 0.3 |

******************************** End of the TP *******************************************