3GPP TR 36.715-00-01 V0.3.0 (2018-05)

Technical Report

3rd Generation Partnership Project;

Technical Specification Group Radio Access Networks;

LTE Advanced xDL/1UL Carrier Aggregation with x>5 (Release 15)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Report is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners’ Publications Offices.

Keywords

LTE-Radio

3GPP

Postal address

3GPP support office address

650 Route des Lucioles – Sophia Antipolis

Valbonne – FRANCE

Tel. : +33 4 92 94 42 00 Fax : +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2017, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

1	Scope	5

2	References	5

3	Definitions, symbols and abbreviations	6

3.1	Definitions	6

3.2	Symbols	6

3.3	Abbreviations	6

4	Background	6

5	xDLs/1UL Carrier Aggregation: General part	7

5.1	UE RF specific	7

5.2	RRM specific	7

6	6DLs/1UL and 7DLs/1UL Inter-Band Carrier Aggregation: Specific Band Combination Part	7

6.1	CA_2-46-66_1UL_BCS0	7

6.1.1	Channel bandwidths per operating band for CA	7

6.1.2	Co-existence studies	7

6.1.3	ΔTIB,c and ΔRIB,c values	8

6.1.4	Reference sensitivity	8

6.2	CA_2-46-48_1UL_BCS0	9

6.2.1	Channel bandwidths per operating band for CA	9

6.2.2	Co-existence studies	9

6.2.3	ΔTIB,c and ΔRIB,c values	10

6.2.4	Reference sensitivity	10

6.3	CA_2-46-48-66_1UL_BCS0	12

6.3.1	Channel bandwidths per operating band for CA	12

6.3.2	Co-existence studies	12

6.3.3	ΔTIB,c and ΔRIB,c values	13

6.3.4	Reference sensitivity	13

6.4	CA_46-48_1UL_BCS0	15

6.4.1	Channel bandwidths per operating band for CA	15

6.4.2	Co-existence studies	15

6.4.3	ΔTIB,c and ΔRIB,c values	15

6.4.4	Reference sensitivity	16

6.5	CA_46-48-66_1UL_BCS0	17

6.5.1	Channel bandwidths per operating band for CA	17

6.5.2	Co-existence studies	17

6.5.3	ΔTIB,c and ΔRIB,c values	18

6.5.4	Reference sensitivity	18

Annex A: Change history	20

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

Y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# 1	Scope

The present document is a technical report for LTE Advanced 1ULs and xDLs Carrier Aggregation with x>5 under Rel-15 time frame. The purpose is to gather the relevant background information and studies in order to address 1ULs and xDLs Carrier Aggregation requirements.

Table 1-1: 6DL/1UL inter-band carrier aggregation combinations

| CA combination | REL independent from |
| --- | --- |
| CA_6DL_2A-46A-48D-66A_1UL_BCS0 | REL- 15 |
| CA_6DL_2A-46A-48E_1UL_BCS0 | REL- 15 |
| CA_6DL_2A-46C-48C-66A_1UL_BCS0 | REL- 15 |
| CA_6DL_2A-46C-48D_1UL_BCS0 | REL- 15 |
| CA_6DL_2A-46D-48A-66A_1UL_BCS0 | REL- 15 |
| CA_6DL_2A-46D-48C_1UL_BCS0 | REL- 15 |
| CA_6DL_2A-46E-48A_1UL_BCS0 | REL- 15 |
| CA_6DL_2A-46E-66A_1UL_BCS0 | REL- 15 |
| CA_6DL_46A-48E-66A_1UL_BCS0 | REL- 15 |
| CA_6DL_46C-48D-66A_1UL_BCS0 | REL- 15 |
| CA_6DL_46C-48E_1UL_BCS0 | REL- 15 |
| CA_6DL_46D-48C-66A_1UL_BCS0 | REL- 15 |
| CA_6DL_46E-48A-66A_1UL_BCS0 | REL- 15 |
| CA_6DL_46E-48C_1UL_BCS0 | REL- 15 |

Table 1-2: 7DL/1UL inter-band carrier aggregation combinations

| CA combination | REL independent from |
| --- | --- |
| CA_7DL_2A-46C-48D-66A_1UL_BCS0 | REL- 15 |
| CA_7DL_2A-46C-48E_1UL_BCS0 | REL- 15 |
| CA_7DL_2A-46D-48C-66A_1UL_BCS0 | REL- 15 |
| CA_7DL_2A-46E-48C_1UL_BCS0 | REL- 15 |
| CA_7DL_46C-48E-66A_1UL_BCS0 | REL- 15 |
| CA_7DL_46E-48C-66A_1UL_BCS0 | REL- 15 |

This TR contains a general part and band specific combination part. The actual requirements are added to the corresponding technical specifications.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: “Vocabulary for 3GPP Specifications”.

[2]	3GPP TR 30.007: “Guideline on WI/SI for new Operating Bands”

[3]	3GPP TS 36.101: “Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio transmission and reception”

[4]		RP-172839, “New WID on LTE Advanced inter-band CA Rel-15 for xDL/2UL with x>5”, Qualcomm Incorporated

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

Carrier aggregation: Aggregation of two or more component carriers in order to support wider transmission bandwidths.

Channel bandwidth: The RF bandwidth supporting a single E-UTRA RF carrier with the transmission bandwidth configured in the uplink or downlink of a cell. The channel bandwidth is measured in MHz and is used as a reference for transmitter and receiver RF requirements.

Inter-band carrier aggregation: Carrier aggregation of component carriers in different operating bands.

Mixed Intra-band and inter-band carrier aggregation: Carrier aggregation of component carriers in same operating band and different operating bands.

NOTE:	Carriers aggregated in each band can be contiguous or non-contiguous.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

# 4	Background

The present document is a technical report for LTE Advanced 1UL and xDL Carrier Aggregation under Rel-15 time frame. It covers both the UE and BS side. The document is divided in two different parts:

- 	General part: this part covers BS and UE specific which is band combination independent.

- 	Specific band combination part:  this part covers each band combination and its specific issues independently from each other (i.e. one subclause is defined per band combination)

The specific band combination parts are independent and therefore, the working speed also differs

# 5	xDLs/1UL Carrier Aggregation: General part

## 5.1	UE RF specific

## This part mainly specifies UE RF requirements for 36.101 to include all listed 6DLs/1UL and 7DLs/1UL inter-band combinations.

## 5.2	RRM specific

This part mainly specifies UE RRM requirements for 36.133 to extent the number of downlink CCs from 5 to 7.

# 6	6DLs/1UL and 7DLs/1UL Inter-Band Carrier Aggregation: Specific Band Combination Part

## 6.1	CA_2-46-66_1UL_BCS0

### 6.1.1	Channel bandwidths per operating band for CA

Table 6.1.1-1: Inter-band CA operating bands

| E-UTRA Operating Band | Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
| 46 | 5150 MHz | – | 5925 MHz | 5150 MHz | – | 5925 MHz | TDD |
| 66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |

Table 6.1.1-2: Supported E-UTRA bandwidths per CA configuration for inter-band CA

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz |  | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |  |
| CA_2A-46E-66A | 2 |  |  |  | Yes | Yes | Yes | Yes | 120 | 0 |  |
|  | 46 | See the CA_46E Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |
|  | 66 |  |  |  | Yes | Yes | Yes | Yes |  |  |  |

### 6.1.2	Co-existence studies

The co-existence studies for CA_2A-46D-66A in TR 36.714-05-01 can be reused for CA_2-46-66. Table 6.1.2-1 summarizes frequency ranges where harmonics occur for CA_2-46-66 with 1 UL.

Table 6.1.2-1: Impact of 1 UL Harmonic Interference

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| 2 | 1850 | 1910 | 1930 | 1990 | 3700 | 3820 | 5550 | 5730 |
| 46 | 5150 | 5925 | 5150 | 5925 | 10300 | 11850 | 15450 | 17775 |
| 66 | 1710 | 1786 | 2110 | 2200 | 3420 | 3572 | 5130 | 5358 |

### 6.1.3	ΔTIB,c and ΔRIB,c values

For this combination, the TIB,c and RIB,c values are shown in table 6.1.3-1 and 6.1.3-2 assuming separate antenna architecture without HTF:

# Table 6.1.3-1: ΔTIB,c

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_2-46-66 | 2 | 0.5 |
|  | 66 | 0.5 |

# Table 6.1.3-2: ΔRIB,c

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_2-46-66 | 2 | 0 |
|  | 66 | 0 |

### 6.1.4	Reference sensitivity

For the REFSENS of CA_2A-46E-66A, the requirements are shown in table 6.1.4-1.

# Table 6.1.4-1: Reference sensitivity QPSK PREFSENS (CA with band 46)

| Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| CA_2A-46E-66A | 2 |  |  | -98 | -95 | -93.2 | -92 | FDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
|  | 66 |  |  | -99.5 | -96.5 | -94.7 | -93.5 | FDD |

## 6.2	CA_2-46-48_1UL_BCS0

### 6.2.1	Channel bandwidths per operating band for CA

Table 6.2.1-1: Inter-band CA operating bands

| E-UTRA Operating Band | Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
| 46 | 5150 MHz | – | 5925 MHz | 5150 MHz | – | 5925 MHz | TDD |
| 48 | 3550 MHz | – | 3700 MHz | 3550 MHz | – | 3700 MHz | TDD |

Table 6.2.1-2: Supported E-UTRA bandwidths per CA configuration for inter-band CA

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_2A-46E-48A | - | 2 |  |  | Yes | Yes | Yes | Yes | 120 | 0 |
|  |  | 46 | See the CA_46E Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
|  |  | 48 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_2A-46D-48C | - | 2 |  |  | Yes | Yes | Yes | Yes | 120 | 0 |
|  |  | 46 | See the CA_46D Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
|  |  | 48 | See the CA_48C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_2A-46C-48D | - | 2 |  |  | Yes | Yes | Yes | Yes | 120 | 0 |
|  |  | 46 | See the CA_46C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
|  |  | 48 | See the CA_48D Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_2A-46A-48E | - | 2 |  |  | Yes | Yes | Yes | Yes | 120 | 0 |
|  |  | 46 |  |  |  |  |  | Yes |  |  |
|  |  | 48 | See the CA_48E Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_2A-46C-48E | - | 2 |  |  | Yes | Yes | Yes | Yes | 140 | 0 |
|  |  | 46 | See the CA_46C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
|  |  | 48 | See the CA_48E Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_2A-46E-48C | - | 2 |  |  | Yes | Yes | Yes | Yes | 140 | 0 |
|  |  | 46 | See the CA_46E Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
|  |  | 48 | See the CA_48C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |

### 6.2.2	Co-existence studies

Table 6.2.2-1 summarizes frequency ranges where harmonics occur for CA_2-46-48 with 1 UL. The 2nd harmonic of Band 2 UL falls into the Band 48 UE receiving bands and the 3rd harmonic of Band 2 UL falls into the Band 46 UE receiving bands.

Table 6.2.2-1: Impact of 1 UL Harmonic Interference

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| 2 | 1850 | 1910 | 1930 | 1990 | 3700 | 3820 | 5550 | 5730 |
| 46 | 5150 | 5925 | 5150 | 5925 | 10300 | 11850 | 15450 | 17775 |
| 48 | 3550 | 3700 | 3550 | 3700 | 7100 | 7400 | 10650 | 11100 |

### 6.2.3	ΔTIB,c and ΔRIB,c values

Assuming the separate antenna architecture without HTF for Band 46, the ΔTIB,c and ΔRIB,c values are shown in table 6.2.3-1 and in table 6.2.3-2.

Table 6.2.3-1: IB,c

| CA_2-46-48 | 2 | 0.6 |
| --- | --- | --- |
|  | 48 | 0.8 |

Table 6.2.3-2: R IB,c

| CA_2-46-48 | 2 | 0.3 |
| --- | --- | --- |
|  | 48 | 0.5 |

### 6.2.4	Reference sensitivity

The MSD is shown in Table 6.2.4-1.

For harmonic issue not related with Band 46, the uplink configuration of CA_2-48 in Table 7.3.1A-0b in 36.101 can be used.

For harmonic issue related with Band 46, the reference sensitivity measurement exclusion region in Table 7.3.1A-0eC in 36.101 can be used.

Table 6.2.4-1: Reference sensitivity QPSK PREFSENS (CA with band 46)

| Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| CA_2A-46E-48A10,11 , CA_2A-46D-48C10,11 , CA_2A-46C-48D10,11 , CA_2A-46A-48E10,11, CA_2A-46C-48E10,11 , CA_2A-46E-48C10,11 | 2 |  |  | -97.8 | -94.8 | -93.0 | -91.7 | FDD |
|  | 46 |  |  |  |  |  | -83 | TDD |
|  | 48 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_2A-46E-48A12 , CA_2A-46D-48C12 , CA_2A-46C-48D12 , CA_2A-46A-48E12, CA_2A-46C-48E12 , CA_2A-46E-48C12 | 2 |  |  | -97.8 | -94.8 | -93.0 | -91.7 | FDD |
|  | 46 |  |  |  |  |  | -83 | TDD |
|  | 48 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| NOTE 10: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band which excludes band 46 and a range FHD above and below the edge of this downlink transmission bandwidth. The value FHD depends on the E-UTRA configuration: FHD = 10 MHz for CA_2-46-48, CA_46-48-66, and CA_2-46-48-66. For harmonic issue not related with band 46, the uplink configuration of CA_2-48, CA_48-66 and CA_2-48-66 in Table 7.3.1A-0b can be used. |  |  |  |  |  |  |  |  |
| NOTE 11: The requirements should be verified for UL EARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. |  |  |  |  |  |  |  |  |
| NOTE 12: The requirements are only applicable to channel bandwidths with a carrier frequency at MHz offset from in the victim (higher band) with , whereandare the channel bandwidths configured in the aggressor (lower) and victim (higher) bands in MHz, respectively. |  |  |  |  |  |  |  |  |

## 6.3	CA_2-46-48-66_1UL_BCS0

### 6.3.1	Channel bandwidths per operating band for CA

Table 6.3.1-1: Inter-band CA operating bands

| E-UTRA Operating Band | Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
| 46 | 5150 MHz | – | 5925 MHz | 5150 MHz | – | 5925 MHz | TDD |
| 48 | 3550 MHz | – | 3700 MHz | 3550 MHz | – | 3700 MHz | TDD |
| 66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |

Table 6.3.1-2: Supported E-UTRA bandwidths per CA configuration for inter-band CA

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_2A-46D-48A-66A | - | 2 |  |  | Yes | Yes | Yes | Yes | 120 | 0 |
|  |  | 46 | See the CA_46D Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
|  |  | 48 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 66 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_2A-46C-48C-66A | - | 2 |  |  | Yes | Yes | Yes | Yes | 120 | 0 |
|  |  | 46 | See the CA_46C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
|  |  | 48 | See the CA_48C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
|  |  | 66 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_2A-46A-48D-66A | - | 2 |  |  | Yes | Yes | Yes | Yes | 120 | 0 |
|  |  | 46 |  |  |  |  |  | Yes |  |  |
|  |  | 48 | See the CA_48D Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
|  |  | 66 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_2A-46C-48D-66A | - | 2 |  |  | Yes | Yes | Yes | Yes | 140 | 0 |
|  |  | 46 | See the CA_46C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
|  |  | 48 | See the CA_48D Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
|  |  | 66 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_2A-46D-48C-66A | - | 2 |  |  | Yes | Yes | Yes | Yes | 140 | 0 |
|  |  | 46 | See the CA_46D Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
|  |  | 48 | See the CA_48C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
|  |  | 66 |  |  | Yes | Yes | Yes | Yes |  |  |

### 6.3.2	Co-existence studies

Table 6.3.2-1 summarizes frequency ranges where harmonics occur for CA_2-46-48-66 with 1 UL. The 2nd harmonic of Band 2 and Band 66 UL falls into the Band 48 UE receiving bands. The 3rd harmonic of Band 2 and Band 66 UL falls into the Band 46 UE receiving bands.

Table 6.3.2-1: Impact of 1 UL Harmonic Interference

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| 2 | 1850 | 1910 | 1930 | 1990 | 3700 | 3820 | 5550 | 5730 |
| 46 | 5150 | 5925 | 5150 | 5925 | 10300 | 11850 | 15450 | 17775 |
| 48 | 3550 | 3700 | 3550 | 3700 | 7100 | 7400 | 10650 | 11100 |
| 66 | 1710 | 1786 | 2110 | 2200 | 3420 | 3572 | 5130 | 5358 |

### 6.3.3	ΔTIB,c and ΔRIB,c values

Assuming the separate antenna architecture without HTF for Band 46, the ΔTIB,c and ΔRIB,c values are shown in table 6.3.3-1 and in table 6.3.3-2.

Table 6.3.3-1: IB,c

| CA_2-46-48-66 | 2 | 0.6 |
| --- | --- | --- |
|  | 48 | 0.8 |
|  | 66 | 0.6 |

Table 6.3.3-2: R IB,c

| CA_2-46-48-66 | 2 | 0.3 |
| --- | --- | --- |
|  | 48 | 0.5 |
|  | 66 | 0.3 |

### 6.3.4	Reference sensitivity

The MSD is shown in Table 6.3.4-1.

For harmonic issue not related with Band 46, the uplink configuration of CA_2-48-66 in Table 7.3.1A-0b in 36.101 can be used.

For harmonic issue related with Band 46, the reference sensitivity measurement exclusion region in Table 7.3.1A-0eC in 36.101 can be used.

Table 6.3.4-1: Reference sensitivity QPSK PREFSENS (CA with band 46)

| Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| CA_2A-46D-48A-66A10,11, CA_2A-46C-48C-66A10,11, CA_2A-46A-48D-66A10,11, CA_2A-46C-48D-66A10,11, CA_2A-46D-48C-66A10,11 | 2 |  |  | -97.7 | -94.7 | -92.9 | -91.7 | FDD |
|  | 46 |  |  |  |  |  | -83 | TDD |
|  | 48 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
|  | 66 |  |  | -99.3 | -96.3 | -94.5 | -93.3 | FDD |
| CA_2A-46D-48A-66A 12, CA_2A-46C-48C-66A12, CA_2A-46A-48D-66A12, CA_2A-46C-48D-66A12, CA_2A-46D-48C-66A12 | 2 |  |  | -97.7 | -94.7 | -92.9 | -91.7 | FDD |
|  | 46 |  |  |  |  |  | -83 | TDD |
|  | 48 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
|  | 66 |  |  | -99.3 | -96.3 | -94.5 | -93.3 | FDD |
| NOTE 10: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band which excludes band 46 and a range FHD above and below the edge of this downlink transmission bandwidth. The value FHD depends on the E-UTRA configuration: FHD = 10 MHz for CA_2-46-48, CA_46-48-66, and CA_2-46-48-66. For harmonic issue not related with band 46, the uplink configuration of CA_2-48, CA_48-66 and CA_2-48-66 in Table 7.3.1A-0b can be used. |  |  |  |  |  |  |  |  |
| NOTE 11: The requirements should be verified for UL EARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. |  |  |  |  |  |  |  |  |
| NOTE 12: The requirements are only applicable to channel bandwidths with a carrier frequency at MHz offset from in the victim (higher band) with , whereandare the channel bandwidths configured in the aggressor (lower) and victim (higher) bands in MHz, respectively. |  |  |  |  |  |  |  |  |

## 6.4	CA_46-48_1UL_BCS0

### 6.4.1	Channel bandwidths per operating band for CA

Table 6.4.1-1: Inter-band CA operating bands

| E-UTRA Operating Band | Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| 46 | 5150 MHz | – | 5925 MHz | 5150 MHz | – | 5925 MHz | TDD |
| 48 | 3550 MHz | – | 3700 MHz | 3550 MHz | – | 3700 MHz | TDD |

Table 6.4.1-2: Supported E-UTRA bandwidths per CA configuration for inter-band CA

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_46C-48E | - | 46 | See the CA_48C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  | 120 | 0 |
|  |  | 48 | See the CA_48E Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_46E-48C |  | 46 | See the CA_46E Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  | 120 | 0 |
|  |  | 48 | See the CA_48C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |

### 6.4.2	Co-existence studies

Table 6.4.2-1 summarizes frequency ranges where harmonics occur due to Band 46 and Band 48 CA with 1 UL. Table 6.4.2-1 shows there are no harmonic issues for this combination.

Table 6.4.2-1: Impact of 1 UL Harmonic Interference

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| 46 | 5150 | 5925 | 5150 | 5925 | 10300 | 11850 | 15450 | 17775 |
| 48 | 3550 | 3700 | 3550 | 3700 | 7100 | 7400 | 10650 | 11100 |

### 6.4.3	ΔTIB,c and ΔRIB,c values

Assuming the separate antenna architecture without HTF for Band 46, the ΔTIB,c and ΔRIB,c values are shown in table 6.4.3-1 and in table 6.4.3-2.

Table 6.4.3-1: IB,c

| CA_46-48 | 48 | 0.8 |
| --- | --- | --- |

Table 6.4.3-2: R IB,c

| CA_46-48-66 | 48 | 0.5 |
| --- | --- | --- |

### 6.4.4	Reference sensitivity

The MSD is shown in Table 6.4.4-1.

Table 6.4.4-1: Reference sensitivity QPSK PREFSENS (CA with band 46)

| Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| CA_46C-48E, CA_46E-48C | 46 |  |  |  |  |  | -83 | TDD |
|  | 48 |  |  | -99 | -96 | -94.2 | -93 | TDD |

## 6.5	CA_46-48-66_1UL_BCS0

### 6.5.1	Channel bandwidths per operating band for CA

Table 6.5.1-1: Inter-band CA operating bands

| E-UTRA Operating Band | Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| 46 | 5150 MHz | – | 5925 MHz | 5150 MHz | – | 5925 MHz | TDD |
| 48 | 3550 MHz | – | 3700 MHz | 3550 MHz | – | 3700 MHz | TDD |
| 66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |

Table 6.5.1-2: Supported E-UTRA bandwidths per CA configuration for inter-band CA

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_46A-48E-66A | - | 46 |  |  |  |  |  | Yes | 120 | 0 |
|  |  | 48 | See the CA_48E Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
|  |  | 66 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_46C-48D-66A |  | 46 | See the CA_46C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  | 120 | 0 |
|  |  | 48 | See the CA_48D Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
|  |  | 66 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_46D-48C-66A | - | 46 | See the CA_46D Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  | 120 | 0 |
|  |  | 48 | See the CA_48C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
|  |  | 66 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_46E-48A-66A | - | 46 | See the CA_46E Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  | 120 | 0 |
|  |  | 48 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 66 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_46C-48E-66A | - | 46 | See the CA_46C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  | 140 | 0 |
|  |  | 48 | See the CA_48E Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
|  |  | 66 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_46E-48C-66A | - | 46 | See the CA_46E Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  | 140 | 0 |
|  |  | 48 | See the CA_48C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
|  |  | 66 |  |  | Yes | Yes | Yes | Yes |  |  |

### 6.5.2	Co-existence studies

Table 6.5.2-1 summarizes frequency ranges where harmonics occur for CA_46-48-66 with 1 UL. The 2nd harmonic of Band 66 UL falls into the Band 48 UE receiving bands and the 3rd harmonic of Band 66 UL falls into the Band 46 UE receiving bands.

Table 6.5.2-1: Impact of 1 UL Harmonic Interference

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| 46 | 5150 | 5925 | 5150 | 5925 | 10300 | 11850 | 15450 | 17775 |
| 48 | 3550 | 3700 | 3550 | 3700 | 7100 | 7400 | 10650 | 11100 |
| 66 | 1710 | 1786 | 2110 | 2200 | 3420 | 3572 | 5130 | 5358 |

### 6.5.3	ΔTIB,c and ΔRIB,c values

Assuming the separate antenna architecture without HTF for Band 46, the ΔTIB,c and ΔRIB,c values are shown in table 6.5.3-1 and in table 6.5.3-2.

Table 6.5.3-1: IB,c

| CA_46-48-66 | 48 | 0.8 |
| --- | --- | --- |
|  | 66 | 0.6 |

Table 6.5.3-2: R IB,c

| CA_46-48-66 | 48 | 0.5 |
| --- | --- | --- |
|  | 66 | 0.3 |

### 6.5.4	Reference sensitivity

The MSD is shown in Table 6.5.4-1.

For harmonic issue not related with Band 46, the uplink configuration of CA_48-66 in Table 7.3.1A-0b in 36.101 can be used.

For harmonic issue related with Band 46, the reference sensitivity measurement exclusion region in Table 7.3.1A-0eC in 36.101 can be used.

Table 6.5.4-1: Reference sensitivity QPSK PREFSENS (CA with band 46)

| Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| CA_46A-48E-66A10,11, CA_46C-48D-66A10, 11, CA_46D-48C-66A 10,11, CA_46E-48A-66A10,11, CA_46C-48E-66A10,11, CA_46E-48C-66A10,11 | 46 |  |  |  |  |  | -83 | FDD |
|  | 48 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
|  | 66 |  |  | -99.3 | -96.3 | -94.5 | -93.2 | FDD |
| CA_46A-48E-66A12, CA_46C-48D-66A12, CA_46D-48C-66A 12, CA_46E-48A-66A12, CA_46C-48E-66A12, CA_46E-48C-66A12 | 46 |  |  |  |  |  | -83 | FDD |
|  | 48 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
|  | 66 |  |  | -99.3 | -96.3 | -94.5 | -93.2 | FDD |
| NOTE 10: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band which excludes band 46 and a range FHD above and below the edge of this downlink transmission bandwidth. The value FHD depends on the E-UTRA configuration: FHD = 10 MHz for CA_2-46-48, CA_46-48-66, and CA_2-46-48-66. For harmonic issue not related with band 46, the uplink configuration of CA_2-48, CA_48-66 and CA_2-48-66 in Table 7.3.1A-0b can be used. |  |  |  |  |  |  |  |  |
| NOTE 11: The requirements should be verified for UL EARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. |  |  |  |  |  |  |  |  |
| NOTE 12: The requirements are only applicable to channel bandwidths with a carrier frequency at MHz offset from in the victim (higher band) with , whereandare the channel bandwidths configured in the aggressor (lower) and victim (higher) bands in MHz, respectively. |  |  |  |  |  |  |  |  |

# Annex A: Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | TSG # | TSG Doc. | CR | Rev | Subject/Comment | Old | New |
| 2018-02 | 3GPP RAN4#86 | R4-1802467 |  |  | TR36.715-00-01 Skeleton for xDL/1UL CA with x>5 | N/A | 0.0.1 |
| 2018-04 | 3GPP RAN4#86Bis | R4-1805714 |  |  | Update typos in TR36.715-00-01 | 0.0.1 | 0.1.0 |
| 2018-05 | 3GPP RAN4#87 | R4-1807584 |  |  | Implemented TPs from RAN4#86Bis meeting: R4-1805104, “Updated scope of TR: LTE Advanced inter-band CA Rel-15 for xDL/1UL with x>5”, Qualcomm Incorporated R4-1805196, “TP to TR 36.715-00-01: Introduction of CA_6DL_2A-46E-66A”, Qualcomm Incorporated | 0.1.0 | 0.2.0 |
| 2018-05 | 3GPP RAN4#87 | R4-1808083 |  |  | Implemented TPs from RAN4#87 meeting: R4-1806791, “Editorial TP for TR36.715-00-01”, Qualcomm Incorporated. R4-1806794 “TP to TR 36.715-00-01: Introduction of CA_2-46-48”, Qualcomm Incorporated. R4-1806795, “TP to TR 36.715-00-01: Introduction of CA_2-46-48-66”, Qualcomm Incorporated. R4-1806797 TP to TR 36.715-00-01: Introduction of CA_46-48, Qualcomm Incorporated. R4-1806796, TP to TR 36.715-00-01: Introduction of CA_46-48-66, Qualcomm Incorporated. | 0.2.0 | 0.3.0 |