3GPP TSG-RAN WG4 Meeting #87 			R4-1808109

Busan, Korea, May 21st – 25th, 2018

Agenda Item:	7.2.3.10

Source:	Apple

Title:	Discussion and clarification on the Single Tx and Dual Tx performance comparison

Document for:	Discussion/Decision

# Introduction

A key enabler for early commercialization of 5G NR is the non-stand-alone (NSA) operation, which is expected to be adopted in many of the initial deployment world-wide. The NSA mode, which is based on LTE-NR dual connectivity (EN-DC), requires UEs to support both LTE and NR uplink transmissions.

In EN-DC, based on whether UE is required to transmit LTE UL and NR UL simultaneously, UE can have two different UL operation modes

- Dual Tx: in this mode, UE is required to transmit LTE UL and NR UL simultaneously

- Single Tx: in this mode, at any given time, UE is only required to transmit uplink in at most one RAT, i.e. either LTE or NR. We also refer to it as Single Tx Switched UL.

We have submitted a list of contributions discussing the Single Tx operation in EN-DC deployment. Contributions [1][4] provide the performance comparison between Single Tx and Dual Tx operation in both TDD and FDD bands. Contribution [2] provides our A-MPR measurement for intra-band EN-DC in Band 41. Contribution [3] discusses the impact from Dual Tx operation on the UE RF frontend architecture and complexity. Last but not the least, contribution [5] provides the solution or Single Tx in TDD band.

Meanwhile, contribution [6] provides comments on Single Tx operation which we believe needs further discussions and clarifications. In this contribution, we compare the Single Tx and Dual Tx operation/performance, in the following areas

- Impact on Latency:

To support the Single Tx operation, especially HARQ timing case 1 (SUO case 1), the number of subframes that can carry DL ACK/NAK is reduced. This, consequently, increases the latency for the HARQ operations. However, the average latency increase is ~2ms instead of ~5ms.

- Impact on NR/LTE DL date rate:

From the PHY layer perspective, SUO case 1 allows all the subframes to be available for DL scheduling. Therefore, it has no impact on PHY layer data rate especially the peak data rate.

From the application layer perspective, a few millisecond latency increases do not impact the TCP performance noticeably, especially considering the fact that TCP RTT is typically dominated by sources other than PHY, such as application layer, server side or NW side etc.

- Impact on NR/LTE UL data rate/coverage:

We need to compare under the framework of EN-DC. For EN-DC operation, UE is expected to operate on both LTE and NR. Otherwise, it is more preferable for NW to deactivate EN-DC and re-config UE to LTE only, which is irrespective of the Dual Tx and Single Tx comparison.

Under the framework of EN-DC, at cell edge, the UL data rate or coverage only depends on the total transmit power and the duration of the transmission. Regardless of Single Tx or Dual Tx operation, UE can transmit UL on all the available subframes. However, in terms of the transmit power, Dual Tx suffers from the transmit power back off due to IMD issue in the difficult band combinations defined in RAN4. Therefore, under EN-DC, Single Tx provides better UL data rate/coverage compared to Dual Tx.

- Impact on other channels, e.g. SRS, PRACH:

RAN1 had specific agreement on SRS and PRACH transmission under SUO case 1 to address the configuration issues. In addition, RAN1 also introduced “harq-Offset-r15” in order to distribute the UL loading among Single Tx UEs evenly within the 10ms radio frame. Therefore, the impact of Single Tx operation on SRS and PRACH is none or minimum.

- EN-DC deployment with different infra-vendors:

For EN-DC deployment with different infra-vendors, tight coordination between NR and LTE scheduler is not feasible. Single Tx operation with SUO case 1 allows semi-static UL resource partition between LTE and NR which can facilitate the EN-DC deployment using different infra-vendors.

# Impact on latency

Compared to Dual Tx operation, Single Tx operation requires the NW to semi-statically partition the UL resources between LTE and NR in TDM manner. Therefore, there are less number of UL subframes to be used for LTE or NR for Single Tx operation. In this contribution, we provide detailed analysis comparing the HARQ timing between Dual Tx and Single Tx for both FDD band and TDD band

## Impact on DL HARQ timing in FDD band

For EN-DC deployment in FDD band, when Dual Tx operation is used, LTE has 4ms latency between PDSCH and ACK/NAK. For Single Tx operation with SUO case 1, we assume that LTE TDD config 2 (2 UL subframes per 10ms) is used, The ACK/NACK latency is {4, 5, 6, 7, 8}ms depending on the DL subframe location with an average latency of 6ms. Figure 1 provides the detailed illustration of DL HARQ timing comparison between Single Tx and Dual Tx for FDD EN-DC. Clearly average latency increase is only 2ms, instead of ~5ms quoted in [6]. It is also important to note that if other LTE TDD configuration with more UL subframes is configured for Single Tx operation, e.g. TDD config 0,1,3, or 6, the latency difference can be even smaller.

Figure 1 DL HARQ timing comparison between Single Tx and Dual Tx for FDD EN-DC

## Impact on DL HARQ timing in TDD band

For EN-DC deployment in TDD band, we assume Dual Tx operates with TDD config 2 (2 UL subframes) which is used in the commercial deployment. For Single Tx operation, we assume TDD config 5 (1 UL subframe) is used to semi-statically allocate 1 UL to LTE and 1 UL to NR. Figure 2 provides the DL HARQ timing comparison between Single Tx and Dual Tx for TDD EN-DC. For Dual Tx, the average latency, calculated as mean of {4,6,7,8}, is 6.25ms. For Single Tx, the average latency, calculated as mean of {4,6,7,8,9,11,12,13}, is 8.75ms which is 2.5ms more compared to Dual Tx. It is also important to note that if other LTE TDD configuration with more UL subframes is configured for Single Tx operation, e.g. TDD config 0,1, 2, 3, 4 or 6, the latency difference can be even smaller.

Figure 2 DL HARQ timing comparison between Single Tx and Dual Tx for TDD EN-DC

## Impact on UL HARQ timing in FDD band

LTE UL operates in synchronous mode. For FDD deployment, i.e. Dual Tx operation in FDD EN-DC, there is constant 8ms latency between the first PUSCH transmission and PUSCH retransmission. For Single Tx SUO case 1, the latency of PUSCH retransmissions is agreed to be 10ms. Hence, there is 2ms latency increase from Single Tx operation.

## Impact on UL HARQ timing in TDD band

For EN-DC deployment in TDD band, the UL retransmission timing follows the TDD configuration. For TDD config 1,2,3,4,5, the latency of PUSCH retransmission is always 10ms. Therefore, there is no latency difference between Single Tx and Dual Tx operations

Based on the above discussion, it is clear that for both DL and UL HARQ operations, Single Tx operation has average latency increase ~2ms or even smaller compared to Dual Tx operation.

However, we also have to consider the latency impact from the data rate that can be achieved at the cell edge. For the difficult band combination such as Band 71 and Band 41, large power backoff is needed to meet the coexistence or emission requirement due to IMD issue. We presented results in [4] to show that Dual Tx suffers worse UL throughput than Single Tx due to large power back off. There are also difficult band combinations where Dual Tx can cause severe DL desense issue, which reduces the DL throughput for Dual Tx operation. Hence, Single Tx operation has latency advantage in terms of high UL/DL throughput at cell edge compared to Dual Tx. In summary, we have the following observation

Observation 1: Comparing Single Tx operation with Dual Tx operation, in terms of latency impact

  - For DL and UL HARQ timing, Single Tx has average latency increase of ~2ms or even smaller

  - For DL and UL data rate, Single Tx can reduce the latency by offering better throughput at cell edge, especially for the band combinations that suffer from DL desense problem or requires large transmit power backoff due to IMD issue.

# Impact on NR/LTE DL data rate

This section discusses the impact from Single Tx operation on NR/LTE DL data rate. It is important to note that NR can already operate with restricted UL resources due to its flexible design. For example, NR supports asynchronous DL and asynchronous UL operation, scheduling and HARQ timing for both DL and UL can be flexibly configured. Furthermore, NR also supports 2, 4 or 7 symbol min-slot operation, in addition to flexible PUCCH configurations, including short PUCCH (1-2 symbols) and long PUCCH (4-14 symbols). Last but not the least, NR supports scalable numerology. Therefore, restricted UL subframes has much less impact on NR DL operation. As results, we focus our discussion on the LTE side from both PHY layer and application layer

## Impact on PHY layer throughput

We compare the DL peak rate that can be achieved by Single Tx and Dual Tx operation. It is clear from Figure 1 and 2 that Single Tx operation with SUO case 1 allows all the subframes to be available for DL scheduling. Hence, Single Tx provides the same DL peak data rate as Dual Tx. However, it is also important to note that, for Dual Tx operation in difficult band combinations that suffer from DL desense issue due to IMD, Dual Tx actually has worse DL data rate compared to Single Tx as Dual Tx suffers from lower SINR due to self-interference between Tx and Rx. This is especially true for the UE at cell edge where UE transmit power is high while UE DL performance is more noise limited. Figure 3 illustrates the DL desense problem with an example of intra-band contiguous EN-DC deployment, e.g. Band 71. The DL desense problem can also happen in other deployment, i.e. inter-band and/or non-contiguous.

Figure 3 Intra-band EN-DC in FDD band: DL Desense problem due to IMD

## Impact on application layer throughput

We use TCP throughput as an example. The TCP throughput is limited by the ratio between TCP window size and RTT (round trip time of TCP ACK). Therefore, RTT or latency could impact the TCP throughput. However, it is important to note that RTT or latency is typically NOT dominated by the PHY layer, but by other sources/layers such as NW side, Server side, etc.,

TCP window scaling allows the TCP window to be as large as 1GB. For RTT, if it is dominated by PHY, we can conservatively assume RTT < 20ms which is well within the PHY layer capability. Under this conservative assumption, we can still achieve >400Gbps TCP peak data rate. Clearly, a few ms latency impact on the PHY would not impact the TCP peak rate noticeably. The latency also impacts the TCP slow start. However, typical applications cannot observe noticeable user experience difference due to a few ms PHY layer latency impact. In addition to that, applications can also have various ways to optimize the TCP window such as remembering or utilizing the TCP window size from the previous connection.

Based on the above discussion, we have the following observation

Observation 2: Comparing Single Tx operation with Dual Tx operation, in terms of DL data rate

  - At PHY layer, Single Tx can achieve the same peak rate as Dual Tx. In addition, Single Tx offers better DL data rate compared to Dual Tx at cell edge, for the band combinations that suffer from DL desense problem due to IMD.

  - At application layer, a few ms latency at PHY layer introduced by Single Tx will not affect TCP peak data rate and will not affect user experience noticeably due to TCP slow start.

# Impact on NR/LTE UL data rate/coverage

In this section, we compare the UL performance between Single Tx and Dual Tx. We focus on the coverage of the control channel (PUCCH) and data channel (PUSCH).

First of all, it is important to point out that for EN-DC deployment, the main purpose is to serve UE both on the LTE and NR RAT, especially for DL. We should not focus on the case where no UL resources are allocated for NR, which consequently means there is no DL for NR as well. We view this operation as deactivating NR via PHY scheduling which is not desirable.

Deactivation of NR via PHY scheduling, but still keep NR RRC connection, is wasteful and inefficient from NW perspective. More importantly, it causes unnecessary complexity and power consumption at UE side. UE needs to consume significant amount of power to maintain a connection, since UE needs to take many actions, including RRM, DCI monitoring, UL SRS transmission, CSI feedback, etc. Therefore, if NW decides to operate on one RAT, it is more efficient to deactivate NR via RRC reconfiguration.

There can be a case where NR suffers from worse coverage compared to LTE due to high frequency band, etc. As results, UE can lose NR coverage at LTE coverage edge. The right solution to handle this scenario is to deactivate NR, i.e. EN-DC, via RRC for efficiency and robustness. Keeping UE in EN-DC while only offer opportunistic NR operation is not the right choice. It has many disadvantage such as complicate the scheduler design, negatively impact the NR L1/L2/L3 operation as there are no reliable UL resource to support both NR UL and NR DL operation, etc.

In summary, the requirement to deactivate NR or EN-DC operation is irrespective to the comparison between Dual Tx and Single Tx operation. We should focus on the case when both NR and LTE RAT are transmitting on the UL for Dual Tx operation, compared to the case of Single Tx operation.

## Impact on UL control channel (PUCCH)

PUCCH is required to carry DL ACK/NAK. In order to compare the control channel coverage, we need to consider both the PUCCH payload and the resources available for PUCCH.

For the payload on PUCCH, it is the same between Dual Tx and Single Tx, i.e. both LTE and NR needs to feedback ACK/NAK for all the DL subframes. For example, there are 20 (10 for LTE and 10 for NR) ACK/NAK that are needed to be sent on PUCCH within 10ms.

For the resource, it includes both time and transmit power. In terms of time, it is the same between Dual Tx and Single Tx, meaning UE is allowed to transmit on all the UL subframes. The difference between Single Tx and Dual Tx operation is whether LTE and NR are allowed to transmit simultaneously or in TDM way. However, in terms of transmit power, as we discussed before, Dual Tx may suffer from transit power back off to meet coexistence or emission requirement due to IMD issue.

We use FDD EN-DC as an example to illustrate the problem, If PUCCH is the coverage bottleneck, NW can configure SUO case 1 with TDD config 6 (5 UL subframes). Table 1 provides the UL PUCCH coverage comparison between Single Tx and Dual Tx. It is important to note two things, (1) NR offers more flexible HARQ timing configuration which allows Single Tx to operate more efficiently. (2) For Dual Tx, to support simultaneous LTE and NR UL transmission, we assume power is equally split between two RAT (20dBm for LTE and NR). Other power allocation will result in Dual Tx performs much worse than Single Tx in one RAT.

Table 1 clearly shows that Single Tx has the same or better UL control channel coverage compared to Dual Tx, especially for the band combinations that requires transmit power back off for Dual Tx operation due to IMD issue.

Table 1 UL PUCCH coverage comparison between Single Tx and Dual Tx

## Impact on UL data channel (PUSCH)

We provided the numerical analysis comparing UL throughput of Single Tx and Dual Tx in [4]. We showed that Single Tx offers better UL throughput compared to Dual Tx, especially for the band combinations that require transmit power back off for Dual Tx operation.

The fundamental reason is easy to understand. At cell edge, when SINR is low, the throughput is linearly proportional to the transmit power and the duration of the transmission. More specially, according to Shannon capacity, the achievable data rate is r=W∙log(1+PW∙n0) where W is the bandwidth, P is the total received power and n0 is noise spectral density. At the cell edge when UE is power limited, the SINR on UL is very low, log operation can be approximated by the linear operation, which suggests that capacity grows linearly as the SINR grows, i.e. r≈W∙PW∙n0=Pn0.

Compared to Dual Tx operation, Single Tx can also use all the transmit power and time. The only difference between Dual Tx and Single Tx is in terms of resource allocation. Dual Tx allocates the resource in the power domain, but Single Tx allocates the resources in time domain. As throughput being linearly proportional to the power and time at cell edge, Single Tx offers the same throughput as Dual Tx, and can even provide higher UL throughput if Dual Tx is required to back off transmit power due to IMD issue.

In summary, we have the following observation

Observation 3: Comparing Single Tx operation with Dual Tx operation, for both UL control (PUCCH) and data (PUSCH), Single Tx offers better coverage than Dual Tx, especially for the band combinations that requires transmit power back off for Dual Tx due to IMD.

# Impact on other channels, such as SRS and PRACH

Single Tx requires semi-static partition of UL subframes between NR and LTE, which means there are less number of UL subframes for LTE and NR compared to Dual Tx. However, RAN1 has already made the agreement to solve this issue in [7] as follows

Agreement: - For Case 1, FDD timing configuration (i.e., periodicity and offset) is applied to LTE PRACH and SRS on LTE UL carriers. - The UE is not required to support transmission of LTE PRACH or SRS transmission which does not coincide with the configured HARQ-ACK transmission occasions

As we discussed before, NR allows more flexible configuration compared to LTE, and consequently, suffers less from Single Tx operation if there is any. Therefore, in this section, we focus on the Single Tx impact on SRS and PRACH on the more vulnerable RAT, i.e. LTE.

## Impact on SRS

This section focuses on both the SRS coverage and SRS capacity. The SRS coverage is impacted by the transmit power and duration. In terms of the transmit power, Dual Tx may suffer from transmit power backoff in the difficult band combination, especially for the UE that does not support dynamic power sharing. In terms of duration, SRS transmission in LTE is always single symbol with {2,5,10,20,40,80,160,320}ms periodicity. Therefore, we need to check the support of SRS periodicity for Single Tx operation

Single Tx operation SUO case 1 relies on the LTE TDD configuration to partition the UL resource. It is important to note that, many TDD configurations, i.e. 0, 1, 2 and 5, have UL subframes every 5ms. Therefore, there is no issue for Single Tx to support SRS periodicity of 5ms or more. We only need to check the SRS periodicity of 2ms which may not have much practical use to start with. We consider Single Tx operation with TDD config 0 (6 UL) as shown in Figure 4. Within 10ms, Single Tx operation needs to drop 2 SRS among 5 SRS with effective periodicity of 3.33ms which should be sufficient enough for practical use.

Figure 4 2ms SRS periodicity in Single Tx operation

Next, we consider the SRS capacity. If only a subset of the UL subframes are available for SRS transmission, there will be impact on the SRS capacity. However, there is already RAN1 agreement/design in [7], as follows, to introduce, harq-Offset-r15, so that different Single Tx UEs can operate in different subset of UL subframes. This allows the load for both SRS and PRACH to be evenly distributed among UEs. As a result, there is no or minimum impact on SRS capacity from Single Tx.

Agreements: - In Case 1, LTE TDD UL HARQ timing is supported and the UE is allowed to transmit only in the subframes designated as UL in the reference TDD configuration. Additionally, a UE-specific HARQ subframe offset can be configured. - The offset (HARQ_offset) is in the range [0…9] - The offset doesn’t change any subframe or slot number - If, before applying the offset, the HARQ feedback for DL SF number m was mapped to UL SF number n, then after the offset, the HARQ feedback for DL SF number (m + HARQ_offset) is mapped to UL SF number (n + HARQ_offset) - If, before applying the offset, the PUSCH in SF number m was scheduled (by UL grant or PHICH) in SF number n, then after the offset, the PUSCH in SF number (m + HARQ_offset) is scheduled (by UL grant or PHICH) in SF number (n + HARQ_offset) - If, before applying the offset, the HARQ-ACK (if carried on PHICH) corresponding to PUSCH in SF number m was transmitted in SF number n, then after the offset, the HARQ-ACK (if carried on PHICH) corresponding to PUSCH in SF number (m + HARQ_offset) is transmitted in SF number (n + HARQ_offset) - SRS and PRACH transmission may only occur in subframes indicated as UL in the reference TDD configuration but shifted by the configured non-zero HARQ_offset

## Impact on PRACH

PRACH can be configured in LTE with two different ways, either in SIB2 or via RRC configurations. SIB2 is cell specific which needs to support both Dual Tx and Single Tx UEs. However, RRC configuration is UE specific and can handle Dual Tx and Single Tx UE separately.

It is important to note that even for SIB2, LTE allows 64 different PRACH configuration, i.e. prach-ConfigIndex, which provides enough flexibility in terms of RACH configurations for NW to support both the Dual Tx and Single Tx UEs at the same time. In addition to that, PRACH can also be configured with RRC per UE for the use case of handover, etc.

Based on the above discussions, we have the following observation

Observation 4: Comparing Single Tx operation with Dual Tx operation, Single Tx has no or minimum impact on the SRS and PRACH configuration and performance.

# Impact on EN-DC operation with different infra-vendors

For EN-DC deployment with different infra-vendors, tight coordination between NR and LTE scheduler is not feasible. Single Tx operation with SUO case 1 allows semi-static UL resource partition between LTE and NR which can facilitate the EN-DC deployment using different infra-vendors.

Consider a case when EN-DC is deployed with different infra-vendors, i.e. LTE eNB and NR gNB from different vendors. LTE and NR will run its scheduler without tight coordination. Current power control rule in EN-DC prioritizes LTE over NR. Hence for Dual Tx operation at cell edge, when NR coverage is limited, UE power control will penalize or drop NR UL transmission based on power control rule. Without reliable UL transmission, NR operation could have many problems such as DL ACK/NAK decoding issue, L1 DL transmission failure, L2 RLC reset, etc. Therefore, without tight coordination, it is desirable to semi-statically allocate the UL resource between LTE and NR to ensure reliable communication in both LTE and NR. In summary, we have the following observation

Observation 5: Comparing Single Tx operation with Dual Tx operation, Single Tx makes it easy to semi-statically allocated UL resource between LTE and NR to facilitate the EN-DC operation with different infra-vendors

# Conclusions

This contribution provides further discussion and clarification on the compassion of Single Tx and Dual Tx operation. It is important to note that for the difficult band combinations determined in RAN4, Dual Tx operation faces two major issues due to IMD (1) DL dense problem (2) UL transmit power backoff to meet coexistence or emission requirements. With this reality in mind, we have the following observations

Observation 1: Comparing Single Tx operation with Dual Tx operation, in terms of latency impact

  - For DL and UL HARQ timing, Single Tx has average latency increase of ~2ms or even smaller

  - For DL and UL data rate, Single Tx can reduce the latency by offering better throughput at cell edge, especially for the band combinations that suffer from DL desense problem or requires large transmit power backoff due to IMD issue.

Observation 2: Comparing Single Tx operation with Dual Tx operation, in terms of DL data rate

  - At PHY layer, Single Tx can achieve the same peak rate as Dual Tx. In addition, Single Tx offers better DL data rate compared to Dual Tx at cell edge, for the band combinations that suffer from DL desense problem due to IMD.

  - At application layer, a few ms latency at PHY layer introduced by Single Tx will not affect TCP peak data rate and will not affect user experience noticeably due to TCP slow start.

Observation 3: Comparing Single Tx operation with Dual Tx operation, for both UL control (PUCCH) and data (PUSCH), Single Tx offers better coverage than Dual Tx, especially for the band combinations that requires transmit power back off for Dual Tx due to IMD.

Observation 4: Comparing Single Tx operation with Dual Tx operation, Single Tx has no or minimum impact on the SRS and PRACH configuration and performance.

Observation 5: Comparing Single Tx operation with Dual Tx operation, Single Tx makes it easy to semi-statically allocated UL resource between LTE and NR to facilitate the EN-DC operation with different infra-vendors

Based on the above observations, we have the following proposals

Proposal 1: RAN4 allows Single Tx operation for EN-DC in DC_(n)71B, at least for Type 2 UE that are not capable of dynamic power sharing.

Proposal 2: RAN4 further discusses the other EN-DC band combinations that allow Single Tx operation. An example how the list can be specified is shown in contribution [3]

# References

- R4-1710727, “Performance Evaluation of LTE NR DC Dual UL and Single UL”, Apple Inc., RAN4 #84bis, Dubrovnik, Croatia, 9 -13 October, 2017

- R4-1803062, “A-MPR for DC_41A_n41A”, Apple Inc., RAN4#86, Athens, Greece, Feb. 26-March 2, 2018

- R4-1805411, “The need of FR1 single-switched UL for EN-DC”, Apple Inc., RAN4#86bis, Melbourne, Australia, April 16-20, 2018

- R4-1805323, “Performance comparison of single Tx and dual Tx for intra-band EN-DC”, Apple Inc., RAN4#86bis, Melbourne, Australia, April 16-20, 2018

- R1-1807004, “Single Tx Switched UL Support for EN-DC in LTE TDD band”, Apple Inc., RAN1#93, Busan, Korea, May 21st – 25th, 2018

- R4-1806721, “Support of dual concurrent UL essential for NSA performance: impact of single-switched UL”, Ericsson, RAN4#87, Busan, Korea, May 21st – 25th, 2018

- Chairman Note, 3GPP TSG RAN1 #90bis, Prague, CZ, 9th – 13th, October 2017