3GPP TSG-RAN WG4 Meeting #87                                   	                	R4-1808197

Busan, KR, 21th – 25th May, 2018

Title:	LS on evaluation period in UE capability of maxUplinkDutyCycle

Response to:

Release:	Rel-15

Work Item:	NR_newRAT-Core

Source:	RAN WG4

To:	RAN WG2

Cc:

Contact Person:

Name:	Xiaoran ZHANG

E-mail Address:	zhangxiaoran@chinamobile.com

1. Overall Description:

In RAN4#86bis meeting, RAN4 discussed how to solve the potential SAR (Specific Absorption Rate) issue for FR1 power class 2 UE on Band n41 n77 n78 n79, and a LS on the UE capability of maxUplinkDutyCycle was sent to RAN2 (R4-1805786).

The value of evaluation period was TBD. In RAN4#87 meeting, RAN4 further discuss the evaluation period and uplink transmission time, the agreements are as follows:

- The uplink transmission time is in terms of symbols

- If the percentage of uplink symbols transmitted in a certain evaluation period is larger than maxUplinkDutyCycle as defined in TS 38.331, the requirements for power class 2 are not applicable, and the corresponding requirements for a power class 3 UE shall apply.

  - The exact evaluation period is up to UE implementation, no less than one radio frame.

which means that:

The UE capability of maxUplinkDutyCycle indicates the maximum percentage of uplink symbols can be scheduled within a certain evaluation period so as to ensure compliance with applicable electromagnetic energy absorption requirements provided by regulatory bodies.

2. Actions:

To RAN1/ RAN2 WG:

RAN4 kindly asks RAN2 to take the above agreement into consideration.

3. Date of Next RAN4 Meetings:

TSG-RAN4 Meeting #88	  	August 20 – 24, 2018		Gothenburg, SE

TSG-RAN4 Meeting #88bis	October 08 – 12, 2018		Chengdu, CN