3GPP TSG RAN WG4 Meeting #87	R4-1808268

Busan, Korea, May 21 – 25, 2018

Agenda Item:	7.4.2.3

Source:	Huawei, HiSilicon

Title:	TP to capture the GSCN calculation

Document for:	Approval

# Introduction

In the last meeting, decisions finalizing the synchronization raster design were made [1] [3] [4]. In [2], a procedure to determine the parameters for the GSCN within a band is presented based on the decisions.

There are two proposals presented. One to fix an oversight in [3]. The second is to capture the computation based on [2]. This is a revision of [5].

# Conclusion

The contribution presents text proposals for computing parameters for the GSCN

# References

R4-1805983, “Draft CR for TS 38.104: Sync raster offset in re-farming bands (5.4.3)”, TS38.104, Ericsson, Samsung, Apr. 16-20, 2018.

R4-1807420, “Detailing the GSCN calculation”, Huawei, HiSilicon, May 21-25, 2018.

R4-1805984, “TP to TR 38.817-01: Sync raster open issues (4.3.1)”, Ericsson, Apr. 16-20, 2018.

R4-1805980, “WF on sync raster for NR,” Ericsson, Apr. 16-20, 2018.

R4-1807423, “TP to capture the GSCN calculation”, Huawei, HiSilicon, May 21-25, 2018.

# Text Proposal (1)

This proposal identifies an oversight in [3].

================================ START ========================

#### 4.3.1.4	Synchronization Channel raster

Sync raster is defined such that there is a minimum number of entries for each band. Sync raster entries will be included in the specifications for each band. Sync raster entries will be defined for initial system acquisition, sync blocks can be transmitted in other frequency locations if the position is signalled to the UE

The sync block is not RB aligned with the data RBs in the channel as shown in Figure 4.3.1.4-1. Instead, there is an arbitrary offset between the edge of the sync block RBs and the edge of the data RBs in the channel, this offset can be up to 11 REs. This enables multiple channels that are subcarrier grid aligned but not RB grid aligned to use the same sync block location. Different channels that are offset by up to 11 REs in frequency can re-use the same sync block frequency location.

Figure 4.3.1.4-1: Alignment between SS block and channel RBs.

Subcarrier mapping:

-	Sync raster will indicate the position of RE=#0(subcarrier #0) of RB#10 of the SS block.

SS Raster Definition for the 0 – 3.0 GHz range:

-	SS Raster is defined as

-	Raster entry = N * 1200 kHz + M * 50 kHz, N=1:2499, M ϵ {1,3,5}
(For operating bands with SCS spaced channel raster, the value M=3 is used.)

-	Raster entry numbering in this range is in increasing order of frequency, starting numbering at #2 (entry #3=1250 kHz, entry #3=1350 kHz, entry #4=1450 kHz, entry#5=2450 kHz…)

-	Total number of entries 7497

-	Raster applies to bands with 100 kHz channel raster and 15 kHz channel raster with 5 MHz minimum bandwidth (bands might be defined in the future) in this frequency range

SS Raster Definition for the 3.0 - 24.25 GHz range:

-	SS Raster is defined as:

-	Raster entry = 3000 MHz + N * 1.44 MHz, N= 0:14756

-	Raster entry numbering is in increasing order of frequency, starting at 7499

-	Total number of entries 14757

-	Raster applies to bands with 15 kHz channel raster and 10 MHz minimum bandwidth (n41, n77,n78, n79, etc) in this frequency range

[…]

================================ END ========================

# Text Proposal (2)

================================ START ========================

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	Recommendation ITU-R M.1036-5 (10/2015), “Frequency arrangements for implementation of the terrestrial component of International Mobile Telecommunications (IMT) in the bands identified for IMT in the Radio Regulations (RR)”.

[3]	R4-1808269, “GSCN calculation excel sheet”, Ericsson, RAN4#87, May 21-25, 2018.

…

[x]	<doctype> <#>[ ([up to and including]{yyyy[-mm]|V<a[.b[.c]]>}[onwards])]: "<Title>".

================================ END ========================

================================ START ========================

#### 4.3.1.5	Calculations of sync raster GSCN per operating band

For each operating band, the GSCN that can be used in the band are tabulated in clause 5.4.3.3 of TS 38.104, TS 38.101-1 and 38.0101-2. Selection of GSCN for Table(s) is done the following way:

-	Include GSCN that correspond to SS block that completely fit within the channel bandwidth, accounting for guard bands needed.

-	Guard bands are calculated based on the minimum channel BW, the SCS for the SS block and the corresponding NRB (spectrum utilization), assuming that the RF reference frequency is at the centre of a block

-	For GSCN ranges with step size <N>, the GSCN numbers selected should be multiples of N (this ensures that overlapping bands will have the same GSCN sequences)

R4-1808269 [3] is a spreadsheet that was used to generate the tables in clause 5.4.3.3 of TS 38.104, TS 38.101-1 and 38.0101-2. The following description for the determination of the GSCN rates are aligned with the calculations in the spreadsheet.

To determine the GSCN range, let fmin denote the lowest frequency location of the SS block within a band after accounting for the guard band and the bandwidth between RE#0 of RB#0 and RE#0 of RB#10 of the SS block. Let fmax denote the high frequency location of the SS block within a band after accounting for the guard band and the bandwidth between RE#0 of RB#10 and RE#11 of RB#19 of the SS block. Let Flow represent the lowest frequency of the frequency range in clause 4.3.1.4, Fraster represent the raster spacing in MHz, Fshift represent the raster shift in MHz (applicable for the frequency range 0 to 3000MHz). The first possible raster location within a band (prior to applying the step size) is given by

fmin≤Flow+N∆Fraster+M∆Fshift

while the last possible raster location within a band (prior to applying the step size) is given by

fmax≥Flow+N∆Fraster+M∆Fshift

where Fshift is defined to be 0 outside the frequency range 0 to 3000MHz and where N (and M) are defined in clause 4.3.1.4.

For the first possible raster location, Nmin is the smallest integer satisfying

Nmin≥1∆Frasterfmin-Flow-max⁡M∆Fshift

and the corresponding value of M (if defined and if multiple values are defined)

M≥1∆Fshiftfmin-Flow-Nmin∆Fraster.

For the last possible raster location, Nmax is the largest integer satisfying

Nmax≤fmax-Flow-minM∆Fshift∆Fraster

and the corresponding value of M (if defined and if multiple values are defined)

M≤fmax-Flow-Nmax∆Fraster∆Fshift.

For the frequency ranges in clause 4.3.1.4, table 4.3.1.5-2 indicates the formulas used to compute the values of N and M (if necessary).

Table 4.3.1.5-2. Formulas to compute the minimum and maximum values of GSCN

| Type | Parameter | Range | Formula |
| --- | --- | --- | --- |
| For GSCNmin | N (compute first) | 0-3000 MHz | N=fmin-Flow-maxM∆Fshift∆Fraster,Mϵ1,3,53Note 1 |
|  |  | ≥ 3000 MHz | N=fmin-Flow∆Fraster |
|  | M | 0-3000 MHz | M'=maxfmin-Flow-N∆Fraster∆Fshift,1 M=M'+M'+1mod2 |
|  |  | 0-3000 MHz, note 1 | 3 |
|  |  | ≥ 3000 MHz | N/A |
| For GSCNmax | N (compute first) | 0-3000 MHz | N=fmax-Flow+minM∆Fshift∆Fraster,Mϵ1,3,53Note 1 N=fmax-Flow-minM∆Fshift∆Fraster,Mϵ1,3,53Note 1 |
|  |  | ≥ 3000 MHz | N=fmax-Flow∆Fraster |
|  | M | 0-3000 MHz | M=minfmax-Flow-N∆Fraster∆Fshift,5 M=M'+M'+1mod2 |
|  |  | 0-3000 MHz, note 1 | 3 |
|  |  | ≥ 3000 MHz | N/A |

To compute the GSCN ranges with step size <N>, the N and M (when defined) values computed with the formulas in table 4.3.1.5-2 are used to determine GSCNmin and GSCNmax according to the appropriate raster entry formula from section 4.3.1.4. The first entry in the GSCN range, GSCNmin, is determined by

GSCNmin=NGSCNmin'N

and the last entry in the range, GSCNmax, is determined by

GSCNmax=NGSCNmax'N.

================================ END ========================