3GPP TSG-RAN WG4 RAN4#87	R4-1808270

Busan, Korea, May 21st – 25th, 2018

Title:	LS on Lead Time for Beam Switching

Release:	Rel-15

Work Item:	NR_newRAT-Core

Source:	RAN4

To:	RAN1

Contact Person:

#### Name:	Valentin Gheorghiu

E-mail Address:	vgheorgh [AT] qti.[DOT]qualcomm [DOT] com

# 1	Overall description

RAN4 has discussed the beam switching delay and how this impacts the UE power consumption in FR2. In order for the UE to be able to optimize power consumption (switch on/off antenna panels that are not used), the UE should be given enough time from the moment it receives a command that leads to a change in Rx beam to the moment that the switch has happened (e.g. TCI state change, aperiodic CSI-RS transmission, etc).

RAN4 agreed that the lead time should be 2ms from the time the beam switch command is received (control channel based) or acknowledged (MAC-CE based) until the switch is finalized. RAN4 also agreed that there should be at least 3ms between consecutive commands implying beam switch.

During the time from receiving the command until the switch is executed the UE is still using the active Rx beam and there is no interruption in communication with the gNB.

# 2	Actions

ACTION to RAN 1:

RAN4 respectfully asks RAN1 to take into considerations the above when deciding the beam switching timelines.

# 3	Date of Next RAN4 Meetings:

TSG-RAN WG4 Meeting #88	20th – 24th August 2018		Gothenburg, Sweden

TSG-RAN WG4 Meeting #88 	8th – 12th October 2018        Chengdu, China