3GPP TSG-WG4 RAN#88 	R4-1809846

# Gothenburg, Sweden, 20th – 24th August, 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-2 | CR | 0000 | rev | - | Current version: | 15.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to TS 38.101-2: MPR for CA |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-08-20 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | MPR for CA is defined with PCG assumption. If PCG is not approved, the table shoud be changed without PCG assumption |  |  |  |  |  |  |  |  |
| Summary of change: |  | MPR values are updated without PCG assumption |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | MPR values are not updated based assumption of without PCG |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2A.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  | X |  | Test specifications |  |  | TS/TR 38.521-2 |  |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

## <Start of Changes>

### 6.2A.2		UE maximum output power reduction for CA

For intra-band contiguous carrier aggregation, UE is allowed to reduce the maximum output power due to higher order modulations and transmit bandwidth configurations for aggregated bandwidth less than 400 MHz. The allowed maximum power reduction (MPR) is defined in Table 6.2A.2-1. The requirement is defined for 2 equal, contiguous CCs, with a single contiguous RB allocation that encloses the inter-CC gap, and with the same type of waveform in both CCs.

Table 6.2A.2-1 Maximum power reduction (MPR) for UE

|  |  | Aggregated channel bandwidth |
| --- | --- | --- |
|  |  | < 400MHz |
| DFT-s-OFDM | Pi/2 BPSK | 5.5 |
|  | QPSK | 5.5 |
|  | 16 QAM | 7.0 |
|  | 64 QAM | 10.0 |
| CP-OFDM | QPSK | 5.5 |
|  | 16 QAM | 7.0 |
|  | 64 QAM | 10.0 |

For the UE maximum output power modified by MPR, the power limits specified in subclause 6.2A.4 apply.

## <End of Changes>