3GPP TSG-RAN WG4 Meeting #88	R4- 1809854

Gothenburg, SE, 20th – 24th Aug, 2018

Agenda Item:	7.11.4.4

Source: 	Intel Corporation

Title: 	Discussion on link reconfiguration requirements

Document for:	Discussion

# Introduction

In RAN4 AH1807 CR to introduce link reconfiguration requirements in NR was endorsed. In this contribution we discuss some of the aspects related to beam failure detection and candidate beam detection requirements.

# Discussion

The link reconfiguration procedure consists of beam failure detection and candidate beam detection.

## Beam Failure Detection

Threshold for BFD

For beam failure detection (BFD) the UE monitors a list of resources () and declares a beam failure instance to higher layers if the SNR of all the BFD resources is below a threshold Qout,LR. Qout,LR corresponds to the SNR at BLERout of a hypothetical PDCCH transmission. BLERout corresponds to the BLERout used for RLM. For RLM the BLERin and BLERout values are specified in TS 38.133 Table 8.1.1-1 and the corresponding BLER pair is selected based on the config parameter RLM-IS-OOS-thresholdConfig. The BLERout for config 0 is 10% whereas the BLERout for config 1 is TBD. The BLERout used for BFD is the same as that used for RLM and not always 10%. In case the BLERout used for BFD is always 10% and RLM is configured to use the 2nd BLER pair which has a lower BLERout than 10%, then the UE would detect out-of-sync before it detects and reports beam failure. Hence the BLERout threshold for BFD and RLM should be the same and based on config parameter RLM-IS-OOS-thresholdConfig.

Table 1: Out-of-sync and in-sync block error rates

| Configuration | BLERout | BLERin |
| --- | --- | --- |
| 0 | 10% | 2% |
| 1 | TBD | TBD |

Proposal#1: The BLERout used for Beam failure detection is the same as that used for RLM and depends on the config parameter RLM-IS-OOS-thresholdConfig

Evaluation Period

In RAN4 AH1807 the evaluation period for SSB based and CSI-RS based BFD in FR2 were agreed as following:

Table 8.5.2.2-2: Evaluation period TEvaluate_BFD_out for FR2

| Configuration | TEvaluate_BFD_SSB (ms) |
| --- | --- |
| non-DRX | max([50], ceil(5*P*N)*TSSB) |
| DRX cycle ≤ 320ms | max([50], ceil(7.5*P*N)*max(TDRX,TSSB)) |
| DRX cycle > 320ms | ceil(5*P*N)*TDRX |
| Note: TSSB is the periodicity of SSB in the set . TDRX is the DRX cycle length. |  |

Where

N=1,

if UE is not provided higher layer parameter RadioLinkMonitoringRS and UE is provided by higher layer parameter TCI-state for PDCCH SSB that has QCL-TypeD, or

if the SSB configured for BFD is QCL-Type D with DM-RS for PDCCH and the QCL association is known to UE, or

if the SSB configured for BFD is QCL-Type D and TDMed to CSI-RS resources configured for L1-RSRP reporting and the QCL association is known to UE;

-	N=FFS, otherwise.

Table 8.5.3.2-2: Evaluation period TEvaluate_BFD_CSI-RS for FR2

| Configuration | TEvaluate_BFD_CSI-RS (ms) |
| --- | --- |
| non-DRX | max([50], [MBFD *P*N] * TCSI-RS) |
| DRX cycle ≤ 320ms | max([50], [1.5×MBFD*P*N]*max(TDRX, TCSI-RS)) |
| DRX cycle > 320ms | [MBFD *P*N] * TDRX |
| Note: TCSI-RS is the periodicity of CSI-RS resource in the set . TDRX is the DRX cycle length. |  |

Where:

N=1,

if UE is not provided higher layer parameter RadioLinkMonitoringRS and UE is provided by higher layer parameter TCI-state for PDCCH CSI-RS that has QCL-TypeD, or

if the CSI-RS configured for BFD is QCL-Type D with DM-RS for PDCCH and the QCL association is known to UE, or

if the CSI-RS resource configured for BFD is QCL-Type D and TDMed to CSI-RS resources configured for L1-RSRP reporting or SSBs configured for L1-RSRP reporting, all CSI-RS resources configured for BFD are mutually TDMed, and the QCL association is known to UE;

-	N=FFS, otherwise.

When the BFD RS (CSI-RS/ SSB) are QCL-Type D and TDMed to CSI-RS or SSB resources for L1-RSRP reporting, the N=1 condition shall apply only if a CSI report with the L1-RSRP measurement of the BFD RS is already made.

Proposal #2: The N=1 condition shall be clarified as follows:

For SSB based BFD:

if the SSB configured for BFD is QCL-Type D and TDMed to CSI-RS resources configured for L1-RSRP reporting and the QCL association is known to UE and a CSI report with L1-RSRP for the SSB configured for BFD is made

For CSI-RS based BFD:

if the CSI-RS resource configured for BFD is QCL-Type D and TDMed to CSI-RS resources configured for L1-RSRP reporting or SSBs configured for L1-RSRP reporting, all CSI-RS resources configured for BFD are mutually TDMed, and the QCL association is known to UE, and a CSI report with L1-RSRP for the CSI-RS configured for BFD is made

For CSI-RS based BFD the number of samples for evaluation is based on that for RLM in-sync. Based on that assumption defined in [1]:	MBFD = 10, if the CSI-RS resource configured for BFD is transmitted with Density = 3. In our companion paper [2] we propose to revise the evaluation period for in-sync and out-of-sync. We propose to update the same for BFD.

Proposal #3: For CSI-RS based BFD, define requirements with D=3.

Proposal #4: For CSI-RS based BFD with D=3, evaluation period of 10 samples are extended to 20 samples, or 10 samples are only applied to 96RB case.

## Candidate Beam Detection

In RAN4 AH1807 the requirements for candidate beam detection requirements we discussed and the options are listed below:

    - Option 1: Requirements for L1-RSRP reporting for beam management shall cover candidate beam selection. No separate requirements shall be introduced for candidate beam detection.

    - Option 2: Separate requirements will be introduced for candidate beam detection.

Candidate beam detection is closely related to beam reporting and both are based on L1-RSRP measurements and hence feasible to introduce the same requirements as beam reporting. But the requirements for L1-RSRP measurements are still under discussion and it is also under discussion if L1-RSRP measurement for beam reporting shall be based on single shot measurement or multiple samples. For candidate beam detection, the L1-RSRP shall be averaged in order to have a reliable estimate for candidate beam selection. The candidate beam selection requirements can re-use beam reporting requirements if L1-RSRP for beam reporting is based on multiple samples. However, if beam reporting requirements are defined based on single shot measurement we would need to define separate requirements for candidate beam detection.

Proposal #5: Option (1) - Candidate beam detection requirements shall be based on beam reporting requirements if L1-RSRP measurement for beam reporting are based on multiple samples. Option (2) – If L1-RSRP for beam reporting is based on single shot measurement, candidate beam detection requirements shall be introduced separately

# Conclusion

In this paper we discuss some aspects related to link re-configuration requirements and have the following proposals:

Proposal#1: The BLERout used for Beam failure detection is the same as that used for RLM and depends on the config parameter RLM-IS-OOS-thresholdConfig

Proposal #2: The N=1 condition shall be clarified as follows:

For SSB based BFD:

if the SSB configured for BFD is QCL-Type D and TDMed to CSI-RS resources configured for L1-RSRP reporting and the QCL association is known to UE and a CSI report with L1-RSRP for the SSB configured for BFD is made

For CSI-RS based BFD:

if the CSI-RS resource configured for BFD is QCL-Type D and TDMed to CSI-RS resources configured for L1-RSRP reporting or SSBs configured for L1-RSRP reporting, all CSI-RS resources configured for BFD are mutually TDMed, and the QCL association is known to UE, and a CSI report with L1-RSRP for the CSI-RS configured for BFD is made

Proposal #3: For CSI-RS based BFD, define requirements with D=3.

Proposal #4: For CSI-RS based BFD with D=3, evaluation period of 10 samples are extended to 20 samples, or 10 samples are only applied to 96RB case.

Proposal #5: Option (1) - Candidate beam detection requirements shall be based on beam reporting requirements if L1-RSRP measurement for beam reporting are based on multiple samples. Option (2) – If L1-RSRP for beam reporting is based on single shot measurement, candidate beam detection requirements shall be introduced separately

# References

- R4-1809568, CR on TS38.133 for link recovery procedure requirements, Huawei, HiSilicon

- R4-1809880, Discussion about evaluation time for CSI-RS based NR RLM, Intel Corporation